
NAESB Coordinate Interchange Business Practice Standard, Version 0
Supporting Documentation
Background:

In light of the August 2003 Blackout, NERC is transitioning the existing Board-approved operating policies and planning standards, and the compliance templates approved by NERC Board on April 2, 2004 into an initial baseline Version 0 set of reliability Standards as recommended by the U.S.-Canada Power System Outage Task Force.  

Current efforts are already under way by the NERC Coordinate Interchange Standard (Version 1) Drafting Team to address the reliability issues associated with bilateral Interchange Transactions. The Version 1 Standard is being developed using the Functional Model as a basis for defining the “Functions” necessary for Bulk Electric System reliability rather than the existing NERC Operating Policies for “Control Areas”.

NERC’s Version 0 Standard Drafting Team will be responsible for a Version 0 Standard that will address Policy 3 reliability issues prior to the above-mentioned Version 1 Standard being enforced. The Version 0 Standard will also utilize the Functional Model language but will not incorporate the new Interchange Authority Entity. Instead, it refers to the Sink Balancing Authority (Sink BA) for Entities performing functions which the IA would perform under Version 1.

Introduction:

The NAESB Business Practices Subcommittee (BPS) is in the development stages of producing its complementary Version 0 Standards which will encompass the business practices currently contained within the NERC Operating Policies. 

The NAESB Coordinate Interchange Standard Version 0 presented here identifies market-supported processes necessary to facilitate bilateral Interchange Transactions prior to the implementation of the approved NAESB CIBP Standard (Version 1). It specifies the arrangements that need to be made and the data that needs to be communicated to the Sink BA and to all involved parties of the Interchange Transaction Tag in order for Interchange transactions to take place between the Source and Sink Balancing Authorities. 

This Standard only covers the business arrangements, data, and timing requirements necessary to submit the Transaction Tag to the Sink BA. It recognizes that FERC- approved tariffs may supersede some provisions in this Standard. 

The Standard applies Functional Model definitions to provide consistency with NERC’s reliability Standards. However, like the NERC Version 0 Standards, this Standard will not incorporate the Interchange Authority entity. 

Attached below are the Policy 3 requirements highlighted in blue from which the Version 0 Standard bases it’s requirements upon. 

Policy 3 – Interchange


Version 0 – Draft 01

Key:









Yellow Reliability Related
 Notes

Blue NAESB
Version 5.2
[See also, “Interchange Reference Document”]

Policy Subsections
A.
Interchange Transaction Implementation

B.
Interchange Schedule Implementation
C.
Interchange Schedule Standards

D.
Interchange Transaction Modifications
Introduction

This Policy addresses the following issues:

· Responsibilities of all Purchasing-Selling Entities involved in Interchange Transactions.
· Information requirements for Interchange Transactions.

· Requirements of Balancing Authority (BA) , Reliability authority (RA), and Transmission Service Provider (tSP) to assess and confirm Interchange Transactions.

· Accountability of BA for implementing all Interchange Schedules in a manner that ensures the reliability of the Interconnections.

· Standards for Interchange Schedules between Source and Sink BAs.
· Requirements for Interchange Transaction Cancellation, Termination, and Curtailment. 
A.
Interchange Transaction Implementation

[Policy 2A, “Transmission—Transmission Operations”]

[Appendix 3A1, “Tag Submission and Response Timetables”]

[Appendix 3A2, “Tagging Across Interconnection Boundaries]

[“E-Tag Spec”]

[“Transaction Tagging Process within ERCOT Reference Document”]

Introduction

This section specifies the Purchasing-Selling Entity’s requirements for tagging all Interchange Transactions, the Balancing Authorities’ and Transmission Service Providers’ obligations for accepting the tags, and the BA’s obligation for implementing the Interchange Transactions. The tag data is integral for providing the BA , RA and TSP, and other operating entities the information they need to assess, confirm, approve or deny, implement, and curtail Interchange Transactions as necessary to accommodate the marketplace and ensure the operational security of the Interconnection.

Requirements

1. Interchange Transaction arrangements. The Purchasing-selling Entity shall arrange for all Transmission Services, tagging, and contact personnel for each Interchange Transaction to which it is a party.
Naesb  Requirement 3.0

1.1  Transmission services. The Purchasing-Selling Entity shall arrange the Transmission Services necessary for the receipt, transfer, and delivery of the Transaction. 

NERC requires Transmission Service Provider to review prior to approving tag.
 Naesb  Requirement 2.0, 3.0     
      1.2 Tagging. The Purchasing-Selling Entity serving the load shall be responsible for providing the Interchange Transaction tag. (Note: 1. Any PSE may provide the tag; however, the load-serving PSE is responsible for ensuring that a single tag is provided. 2. If a PSE is not involved in the Transaction, such as delivery from a jointly owned generator, then the Sink BA is responsible for providing the tag. PSEs must provide tags for all Interchange Transactions in accordance with Requirement 2 below)

Covered under NERC V-0 Standard.
Naesb  Requirement 1.2
1.3 Contact personnel. Each Purchasing-selling Entity with title to an Interchange Transaction must have, or arrange to have, personnel directly and immediately available for notification of Interchange Transaction changes. These personnel shall be available from the time that title to the Interchange Transaction is acquired until the Interchange Transaction has been completed. Naesb  Requirement 11.0 

1.4 E-Tag monitoring. Each BA, RA, TSP, and PSE who are responsible for a tagged Transaction shall have facilities to receive unsolicited notification from the Sink BA of changes in the status of a tag with which the user is a participant. 

Naesb  Requirement 11.0
2.0 Interchange transaction tagging. Each Interchange Transaction shall be tagged before implementation as required by each Interconnection as specified in the “E-Tag Spec” or “Transaction Tagging Process within ERCOT Reference Document.” In addition to providing necessary operating information, the Interchange Transaction tag is the official request from the load-serving Purchasing-Selling Entity to the Sink BA to implement the Interchange Transaction. The information that must be provided on the tag is listed in Appendix 3A4.
Naesb  Requirement 1.1
2.1 Application to Transactions. All Interchange Transactions and certain Interchange Schedules shall be tagged. 
Covered under NERC V-0 Standard

Naesb  Requirement 1.0
In addition, intra-BA transfers using Point-to-Point Transmission Service
 shall be tagged. This includes:
· Interchange Transactions (those that are between BAs).
· Transactions that are entirely within a BA.
· Dynamic Interchange Schedules (tagged at the expected average MW profile for each hour). (Note: a change in the hourly energy profile of 25% or more requires a revised tag.)
· Interchange Transactions for bilateral Inadvertent Interchange payback (tagged by the Sink BA).
· Interchange Transactions established to replace unexpected generation loss, such as through prearranged reserve sharing agreements or other arrangements, are exempt from tagging for 60 minutes from the time at which the Interchange Transaction begins (tagged by Sink BA). [See also, Policy 1E2 and 2.1, “Disturbance Control Standard”]

Covered under NERC V-0 Standard
2.2  Parties to whom the complete tag is provided. The tag, including all updates and notifications, shall be provided to the following entities:

· Purchasing-Selling Entities
· Balancing Authorities
· Transmission Service Providers

· Generator Owners  

· Load-Serving Entities
· Reliability Authorities

· Security Analysis Services 
Covered under NERC V-0 Standard
Naesb  Requirement 5.0
2.3  Method of transmitting the tag. The Purchasing-Selling Entity shall submit the Interchange Transaction tag in the format established by each Interconnection. [“E-Tag Spec” or “Transaction Tagging Process within ERCOT Reference Document”] 
Naesb  Requirement 8.0
2.3.1 Tags for Interchange Transactions that cross Interconnection boundaries. Procedures are found in Appendix 3A2, “Tagging Across Interconnection Boundaries.”

Covered under NERC V-0 Standard 
2.4 Interchange Transaction submission time. To provide adequate time for Interchange Schedule implementation, Interchange Transactions shall be submitted to the Sink BA and assessed by the RA, BA, TSP as specified in Appendix 3A1, “Tag Submission and Response Timetable.” 
            Covered under NERC V-0 Standard
Naesb  Requirement 4.0
2.4.1 Exception for security reasons. Exception to the submission time requirements in Section 2.4 is allowed if immediate changes to the Interchange Transactions are required to mitigate an Operating Security Limit violation. The tag may be submitted after the emergency Transaction has been implemented but no later than 60 minutes.
    Covered under NERC V-0 Standard 
2.5 Confirmation of tag receipt. Confirmation of tag receipt shall be provided to the Purchasing-Selling Entity who submitted the tag in accordance with Interconnection tagging practices. [“E-Tag Spec”]
Naesb  Requirement 6.0
              2.6 Tag acceptance. An Interchange Transaction tag shall be accepted if all required information is valid and provided in accordance with the tagging specifications in Requirement 2. 

Not in NAESB or NERC Standard

3.0  Interchange Transaction tag receipt verification. The Sink BA shall verify the receipt of each Interchange Transaction tag  with the Transmission Providers and Balancing Authorities before the Interchange Transaction is implemented.  

Naesb  Requirement 6.0
4.0 Interchange Transaction assessment. All Transmission Service Providers, Load Serving Entities, Purchasing-Selling Entities and Balancing Authorities  involved in the transaction, and other operating entities responsible for operational security shall be responsible for assessing and “approving” or “denying” Interchange Transactions as requested by the PSE based on established reliability criteria and adequacy of Interconnected Operations Services and transmission rights as well as the reasonableness of the Interchange Transaction tag.  Purchasing-Selling Entities and Load Serving Entities may elect to defer their approval responsibility to the Host BA.  This assessment shall include the following:
The BA assesses:

· Transaction start and end time

· Energy profile (ability of generation maneuverability to accommodate)

· Scheduling PATH (proper connectivity of ADJACENT BAs)

The Transmission Provider assesses:

· Valid OASIS reservation number or transmission contract identifier

· Proper transmission priority

· Energy profile accommodation (does energy profile fit OASIS reservation?)

· OASIS reservation accommodation of all Interchange Transactions
· Loss accounting
Covered under NERC V-0 Standard 
Naesb  Requirement 1.1, 6.0
 The Purchasing-Selling Entity and Load-Serving Entity assess:
· Transaction is valid representation of contractually agreed upon energy delivery. 
Naesb  Requirement 2.0, 3.0
·  Tag corrections. During the BA’s and TSP’s assessment time, the Purchasing-Selling Entity who submitted the tag may elect to submit a tag correction. Tag corrections are changes to an existing tag that do not affect the reliability impacts of the Interchange Transaction; therefore, tag corrections do not require the complete re-assessment of the tag by all BAs and TSPs on the Scheduling Path, or the completion and submission of a new tag by the Purchasing-Selling Entity. The Sink BA shall notify the Bas and TSPs, as to the changes  and specifically alert those entities for which a correction has impact. Entities who are impacted by the correction will have an opportunity to reevaluate the tag status.  The timing requirements for corrections are found in Appendix 3A1, “Tag Submission and Response Timetable.” Tag items that may be corrected are found in Appendix 3A4, “Required Tag Data.” A description of those entities who may correct an Interchange Transaction tag is found in Appendix 3D, “Transaction Tag Actions.” [See Appendix 3A1 Subsection C, Interchange Transaction Corrections.]

Naesb  Requirement 9.0
5.0 Interchange Transaction approval or denial. Each BA and TSP involved in the transaction responsible for assessing and “approving” or “denying” the Interchange Transaction shall notify the Sink BA as to the results of the assessment. The Sink BA in turn notifies the Purchasing-Selling Entity who submitted the Interchange Transaction tag, plus all Balancing Authorities and Transmission Providers on the Scheduling Path. Assessment timing requirements are found in Appendix 3A1, “Tag Submission and Response Timetable.” A description of those entities who may approve or deny an Interchange Transaction is found in Appendix 3D, “Transaction Tag Actions.”


Covered under NERC V-0 Standard 
Naesb  Requirement 6.0
5.1 Interchange Transaction denial. If denied, this notification shall include the reason for the denial.

Naesb  Requirement 6.1
5.2 Interchange Transaction approval. The Interchange Transaction is considered approved if the Purchasing-Selling Entity who submitted the Interchange Transaction tag has received confirmation of tag receipt and has not been notified that the transaction is denied.

Not specifically covered in Naesb or nerc standard
6.0  Responsibility for Interchange Transaction implementation. The Sink BA is responsible for initiating the implementation of each Interchange Transaction as tagged in accordance with Policy 3.A. Requirement 2 (and its subparts). The Interchange Transaction is incorporated into the Interchange Schedule(s) of  balancing authorities on the Scheduling Path in accordance with Policy 3B.

Covered under NERC V-0 Standard 
6.1 Tag requirements for Interchange Transaction implementation. The BA shall implement only those Interchange Transactions that:
· Have been tagged in accordance with Requirement 2 above, or,
· Are exempt from tagging in accordance with Requirement 2.1 above.
Covered under NERC V-0 Standard 
7.  Tag requirements after curtailment has ended. After the curtailment of a Transaction has ended, the Interchange Transaction’s energy profile will return to the originally requested level unless otherwise specified by the Purchasing-Selling Entity. [See Interchange Transaction Reallocation During TLR Levels 3a and 5a Reference Document, Version 1 Draft 6.]
Covered under NERC V-0 Standard
8.0  Confidentiality of information. Reliability Authorities, BAs, Transmission Providers, Purchasing-Selling Entities, and entities serving as tag agents or service providers as provided in the “E-Tag Spec” shall not disclose Interchange Transaction information to any Purchasing-Selling Entity except as provided for in Requirement 2.2 above, “Parties to whom the complete tag is provided.”

Not Covered in NAESB or NERC Standard

B. Interchange Schedule Implementation

[Policy 2A, “Transmission—Transmission Operations”]

Introduction

This section explains Control Area requirements for implementing the Interchange Schedules that result from the Interchange Transactions tagged by the Purchasing-Selling Entities in Section A. 

Requirements

1. Balancing Authorities must be adjacent .Interchange Schedules shall only be implemented between Adjacent Balancing Authorities. 

Covered under NERC V-0 Standard 
2. Sharing Interchange Schedules details. The Sending and Receiving bas must provide the details of their Interchange Schedules via the Interregional Security Network as specified in Policy 4.B. 

Covered under NERC V-0 Standard 
3. Providing tags for approved Transactions to the Reliability authority. The sink BA shall provide it’s Reliability Authority the information from the Interchange Transaction tag electronically for each Approved Interchange Transaction.
Covered under NERC V-0 Standard 
4. Interchange Schedule confirmation and implementation. The Receiving Ba is responsible for initiating the confirmation and implementation of the Interchange Schedule with the SENDING Ba .
Covered under NERC V-0 Standard 
4.1. Interchange Schedule agreement. The Sending and Receiving  ba shall agree with each other on the: 

· Interchange Schedule start and end time

· Ramp start time and rate

· Energy profile

This agreement shall be made before either the Sending or Receiving  ba makes any generation changes to implement the Interchange Schedule.
Covered under NERC V-0 Standard .

4.1.1. Interchange Schedule standards. The Sending and receiving ba shall comply with the Interchange Schedule Standards in Policy 3C, “Interchange – Schedule Standards.”
Reference only

4.1.2. Operating reliability criteria. BAs shall operate such that Interchange Schedules or schedule changes do not knowingly cause any other systems to violate established operating reliability criteria.

.
4.1.3. DC tie operator. Sending and Receiving bas shall coordinate with any DC tie operators on the Scheduling Path.

Covered under NERC V-0 Standard 
5.
Maximum scheduled interchange. The maximum Net Interchange Schedule between two bas shall not exceed the lesser of the following:

5.1.
Total capacity of facilities. The total capacity of both the owned and arranged-for transmission facilities in service for any transmission service provider along the path, or
5.2.
Total Transfer Capability. The established network Total Transfer Capability (TTC) between  Bas, which considers other transmission facilities available to them under specific arrangements, and the overall physical constraints of the transmission network. Total Transfer Capability is defined in Available Transfer Capability Definitions and Determination, NERC, June 1996.
C.
Interchange Schedule Standards

Standards
1. Interchange Schedule start and end time. Interchange Schedules shall begin and end at a time agreed to by the source and sink bas, and the Intermediary BAs. 
Covered under NERC V-0 Standard
2. Ramp start times. Balancing Authorities shall ramp the Interchange equally across the start and end times of the schedule.

3.          Ramp duration. Bas shall use the ramp duration established by their Interconnection as follows unless they agree otherwise: 
3.1   Interchange Schedules within the Eastern and ERCOT Interconnections. ten-minute ramp duration.

3.2  Interchange Schedules within the Western Interconnection. 20-minute ramp duration. 
Naesb  Requirement 14.0, 14.1, 14.2
3.3  Interchange Schedules that cross an Interconnection boundary. The BAs that implement Interchange Schedules that cross an Interconnection boundary must use the same start time and ramp durations.

Covered under NERC V-0 Standard
3.4  Exceptions for Compliance with Disturbance Control Standard and Line Load Relief. Ramp durations for Interchange Schedules implemented for compliance with NERC’s Disturbance Control Standard (recovery from a disturbance condition) and Interchange Transaction curtailment in response to line loading relief procedures may be shorter, but must be identical for the Sending and Receiving bas [See also Policy 1B, “Generation Control Performance – Disturbance Control Standard,” Requirement 2 and subsections on contingency reserve.]

Covered under NERC V-0 Standard 
4.0 Interchange Schedule accounting. Block accounting shall be used.
NAESB REQUIREMENT  12.0
D. 
Interchange Transaction Modifications 

Introduction

This section specifies Purchasing-Selling Entity’s, Transmission Provider’s , and Balancing Authorities’ rights and requirements for modifying an Interchange Transaction tag after it has been approved and implemented as described in the preceding sections. 

Requirements

1. Interchange Transaction modification for market-related issues. The Purchasing-Selling Entity that submitted an Interchange Transaction tag may modify an Interchange Transaction tag that is in progress or scheduled to be started.  These modifications may be made due to changes in contracts, economic decisions, or other market-based influences.  In cases where a market operator is serving as the source or sink for a Transaction, then they shall have the right to effect changes to the energy flow as well (based on the results of the market clearing). 
NAESB REQUIREMENT  10.0
1.1. Increases.  The Interchange Transaction tag’s energy and/or committed transmission reservation(s) profile may be increased to reflect a desire to flow more energy or commit more transmission than originally requested. Necessary transmission must be either available from the earlier Transaction or provided with the increase.
1.2. Extensions.  The Interchange Transaction tag’s energy profile may be extended to reflect a desire to flow energy during hours not previously specified.  Necessary transmission capacity must be provided with the extension.  
1.3. Reductions.  The Interchange Transaction tag’s energy and/or committed transmission reservation(s) profile may be reduced to reflect a desire to flow less energy or commit less transmission than originally requested.  Reductions are used to indicate cancellations and terminations, as well as partial decreases.
1.4. Combinations of 1.1, 1.2, and 1.3 may be submitted concurrently. 

1.5. Coordination responsibilities of the Purchasing-Selling Entity.  The modification must be provided by the Purchasing-Selling Entity to the following Interchange Transaction participants:
· Purchasing-selling Entities
· Balancing Authorities or their scheduling agents
· Transmission Service Providers

· Reliability Authorities

· Load-Serving Entities
· Generator Owners

· Security Analysis Services
NAESB REQUIREMENT  10.0
1.6 Interchange Transaction modification and evaluation time. To provide adequate time for Interchange Schedule implementation, Interchange Transaction modifications shall be requested and evaluated as specified in Section D of Appendix 3A1, “Tag Submission and Evaluation Timetable.” 

NAESB REQUIREMENT  10.0
2. Interchange Transaction modification for reliability-related issues. A Reliability Authority, Transmission Provider, source or Sink BA may modify an Interchange Transaction tag that is in progress or scheduled to be started.  These modifications may be made only due to TLR events (or other regional congestion management practices), Loss of Generation, or Loss of Load.

Covered under NERC V-0 Standard
2.1. Assignment of coordination responsibilities during TLR events.  At such times when TLR is required to ensure reliable operation of the electrical system, and the TLR requires holding or curtailing Interchange Transactions, the Sink BA is responsible for coordinating the modifications to the appropriate Interchange Transaction tags.  See Policy 9, Appendix 9C1 “Transmission Loading Relief Procedure ( Eastern Interconnection.”
2.1.1. Reductions. When a Reliability Authority must curtail or hold an Interchange Transaction to respect Transmission Service reservation priorities or to mitigate potential or actual Operating Security Limit violations, the Reliability Authority shall inform the Sink BA listed on the Interchange Transaction tag of the greatest reliable level at which the affected Interchange Transaction may flow.

2.1.2. Reloads. At such time as the TLR event allows for the reloading of the transaction, the Reliability authority shall inform the Sink BA  listed on the Interchange transaction tag of the releasing of the Interchange Transaction’s limit.

2.2. Coordination when implementing other congestion management procedures.  As a part of some local and regional congestion management and transmission line overload procedures, the Transmission Service Provider or Balancing Authority is responsible for implementing curtailment of Interchange Transactions.  The Transmission Provider or affected Balancing Authority may adjust the Interchange Transaction tags as required to implement those local and regional congestion management or transmission overload relief procedures that have been approved by the Region(s) or NERC.

2.2.1. Reductions. When a Transmission Provider or Balancing Authority experiences the need to invoke a congestion management or transmission line overload procedure, it may use the curtailment feature of E-Tag to inform the Source and Sink BAs listed on the Interchange Transaction tag of the greatest reliability limit at which the affected Interchange Transaction may flow.

2.2.2. Reloads. At such time as the need for the congestion management or transmission line overload relief procedure allows for the full or partial reloading of the transaction, the Transmission Provider or Balancing Authority may use the reload feature of E-Tag to inform the source and sink BA listed on the Interchange Transaction tag that the Interchange Transaction’s reliability limit has changed.
2.3. Assignment of coordination responsibilities during a loss of generation.  At such times when a loss of generation necessitates curtailing Interchange Transactions, the Source BA is responsible for coordinating the modifications to the appropriate Interchange Transaction tags.

2.3.1. Reductions. When a generation operator experiences a full or partial loss of generation, it shall notify the Host BA (the Source BA for the Interchange Transaction).  The Host BA contacts the PSE that is responsible for the generation.  The Purchasing-selling Entity providing Generation determines what schedule modifications need to be made and may request those modifications as market-based reductions, increases, or extensions (either via the PSE who submitted the tag , or directly if the entity is the PSE who submitted the tag or a market operator).  If the PSE providing Generation does not resolve the condition, the Host BA may at its discretion curtail Interchange Transactions associated with the generation.  

2.3.2. Reloads. Upon return of the generation, the generator operator shall notify the Host BA (the Source BA for the Interchange Transaction).  The Host BA contacts the PSE  that is responsible for the generation.  The Purchasing-selling entity providing generation determines what schedule modifications need to be made and may request those modifications as market-based reductions, increases, or extensions (either via the PSE who submitted the tag, or directly if the entity is PSE who submitted the tag or a market operator).  The Host BA must release the limits previously imposed on Interchange Transactions associated with the generation (but not override any market-based reductions).

2.4. Assignment of coordination responsibilities during a loss of load.  At such times when a loss of load necessitates curtailing Interchange Transactions, the Sink BA is responsible for coordinating the modifications to the appropriate Interchange Transaction tags.

2.4.1. Reductions. When a Load-Serving Entity experiences a loss of load, it shall notify its Host ba (the Sink Ba for the Interchange Transaction) and determine what schedule modifications need to be made.  The Load-Serving Entity may request those modifications as market-based reductions, increases, or extensions (via either the PSE who submitted the tag, or directly if the entity is the PSE who submitted the tag or a market operator).  If the Load-Serving Entity does not notify the Host ba, the Host ba may at its discretion curtail Interchange Transactions associated with the load. 

2.4.2. Reloads.  Upon return of the load, the Load-Serving Entity shall notify its Host BA (the sink ba for the Interchange Transaction) and determine what schedule modifications need to be made.  The Load-Serving Entity may request those modifications as market-based reductions, increases, or extensions (either via PSE who submitted the tag, or directly if the entity is PSE who submitted the tag or a market operator).  If the Load-Serving Entity does not notify the Host BA, the Host BA must release the limits previously imposed on Interchange Transactions associated with the load (but not override any market-based reductions).

Covered under NERC V-0 Standard
2.5. Coordination responsibilities for reliability-related issues.  The modification must be provided by the requesting  Balancing Authority, Transmission Provider, or Reliabiility Authority to the following Interchange Transaction participants:
· Purchasing Selling Entities
· Source or Sink BA or their Scheduling Agent
· Transmission Service Providers

· Load-Serving Entity

· Security Analysis Services

Covered under NERC V-0 Standard 
2.6 Interchange Transaction modification and evaluation time. To provide adequate time for Interchange Schedule implementation, Interchange Transaction modifications shall be requested and evaluated as specified in Appendix 3A1, “Tag Submission and Evaluation Timetable .

Covered under NERC V-0 Standard 
























NERC expects that Approval Entities have the proper resources to perform these assessments. Lack of these tools is not a reason to deny an Interchange Transaction. Resources include personnel and tools.



























































FERC Orders 888, 889, 638, and a provider’s OATT guide transmission requests.  Tagging policy shall not supersede OASIS requirements.  














� This includes all “grandfathered” and other “non-888” Point-to-Point Transmission Service






