Business Standard 

Emergency Operations

Version 0 – Draft 1

Introduction

NERC maintains Standards pertaining to the operation of the BULK ELECTRIC SYSTEM under emergency conditions. These NERC Standards define the reliability considerations for the issuance of ENERGY EMERGENCY ALERTS. The purpose of this NAESB Business Practice Standard is to cover the commercial aspects of the operation of the BULK ELECTRIC SYSTEM under such emergency conditions once initiated as per the NERC applicable Standards.

Requirements

1. Anticipating capacity or energy emergency.
A Ba

 anticipating an operating capacity or energy emergency shall perform all actions necessary to mitigate the anticipated emergency including bringing on all available generation, postponing equipment maintenance, scheduling interchange purchases in advance, and being prepared to reduce firm load.

2. Returning ACE to Acceptable Levels.
A BA shall utilize the following actions to the fullest extent practicable to return ACE to acceptable levels during an energy emergency:

· Load all available generating capacity

· Utilize all operating reserve

· Interrupt all interruptible load and interruptible exports

· Utilize all emergency assistance from other BAS

3. Failure to Return ACE to Acceptable Levels.
A deficient BA that has failed to return its ACE to acceptable levels after having used all Requirement 2 actions to the fullest extent practicable shall initiate the following actions after a 15 minute period:

· Manually shed firm load without delay to return its ACE to zero.
· Declare an Emergency Energy ALERT in accordance with NERC applicable Standards.
4. Elevating Transmission Service Priority within the Eastern Interconnection.
A Transmission Provider
 shall elevate the transmission service priority of an Interchange Transaction from Priority 6 (Network Integration Transmission Service from Non-designated Resources) to Priority 7 (Network Integration Transmission Service from designated Network Resources) only if the following conditions are satisfied:

· Such a process is permitted in its transmission tariff
· The LSE served by the BA or TRANSMISSION PROVIDER has requested its RA to initiate an ENERGY EMERGENCY ALERT

5. Initiating an ENERGY EMERGENCY ALERT.
5.1
ENERGY EMERGENCY ALERTS shall be initiated only by a RA for the following reasons only:
· upon the RA’S own request 
· upon the request of a Ba 
· upon the request of a LSe
5.2
LSES shall be allowed to request an ENERGY EMERGENCY ALERT for the following reasons only:
· The LSE is, or expects to be, unable to provide its customers' energy requirements, and has been unsuccessful in locating other systems with available resources from which to purchase

· The LSE cannot schedule its resources due to, for example, ATC limitations or Transmission Loading Relief limitations

5.3 RAS , BAS and/or LSES shall not request ENERGY EMERGENCY ALERTS based upon cost.

6. Posting of ENERGY EMERGENCY ALERTS.
The RA shall post on the NERC web site the initiation of the ENERGY EMERGENCY ALERT and the expected total transmission service capacity in MW that will be subject to the Transmission Service Priority Elevation. The following steps shall be followed by the RA for posting:
6.1
Upon an ENERGY EMERGENCY ALERT Level 1 (EEA1), the RA shall post the forecasted Transmission Service change from Priority 6 to Priority 7

6.2
Upon an ENERGY EMERGENCY ALERT Level 2 (EEA2), the RA shall post an announcement of the Transmission Service change from Pri
�Suggest beginning standard with definitions page


�


�Can we list TP in the defintions and so reference?





