Inadvertent Interchange Standard – Version 0, Draft 1

Inadvertent Interchange Standard – Version 0, Draft 1


NAESB Inadvertent Interchange Standard 

Version 0

General Requirements

1. INADVERTENT INTERCHANGE Accounting. Adjacent BALANCING AUTHORITIES shall operate to a common Net Interchange Schedule and Actual Net Interchange value and shall record these hourly quantities, with like values but opposite sign. Each BALANCING AUTHORITY shall compute its INADVERTENT INTERCHANGE based on the following:
1.1. Daily accounting. Each BALANCING AUTHORITY, by the end of the next business day, shall agree with its adjacent BALANCING AUTHORITIES to:
1.1.1. The hourly values of Net Interchange Schedule.

1.1.2. The hourly-integrated MWh values of Net Actual Interchange.
1.2. Monthly accounting. Each BALANCING AUTHORITY shall use the agreed-to Daily and Monthly accounting data to compile its monthly accumulated INADVERTENT INTERCHANGE for the On-Peak and Off-Peak hours of the month. (Refer to Appendix A, On Peak – Off Peak Periods“
1.3. After-the-Fact Corrections. After-the-fact corrections to the agreed-to Daily and Monthly accounting data shall only be made to reflect actual operating conditions (e.g. a meter being used for control was sending bad data). Changes or corrections based on non-reliability considerations shall not be reflected in the BALANCING AUTHORITY (S) INADVERTENT INTERCHANGE. After-the-fact corrections to the scheduled or actual values will not be accepted without agreement of the Adjacent BALANCING AUTHORITY (s).

1.4. Periodic Adjustments shall be made to correct for differences between hourly MWh meter totals and the totals derived from register readings of the tie-line meters. Adjacent BALANCING AUTHORITIES shall agree upon the difference determined above and assign this correction tot he proper On-Peak and Off-Peak period at the same times and in equal quantities in the opposite directions. Any adjustments necessary due to known metering errors, franchised territories, transmission losses or other special circumstances shall be made in the same manner. 

1.4.1. Adjustments to schedules shall only be made if an incorrect schedule was used by one BALANCING AUTHORITY. Schedules shall not be adjusted after-the-fact due to marketing considerations or adjustments during the billing procedure.

2. INADVERTENT INTERCHANGE payback. Each BALANCING AUTHORITY shall be diligent in reducing accumulated inadvertent balances. INADVERTENT INTERCHANGE accumulations shall be paid back by either of the following methods:

2.1. Energy “in-kind” payback. INADVERTENT INTERCHANGE accumulated during “On-Peak” hours shall only be paid back during “On-Peak” hours. INADVERTENT INTERCHANGE accumulated during “Off-Peak” hours shall only be paid back during “Off-Peak” hours. [See Appendix A, “On-Peak and Off-Peak Periods.”]

2.1.1. Bilateral payback. INADVERTENT INTERCHANGE accumulations may be paid back via an INTERCHANGE SCHEDULE with another BALANCING AUTHORITY. 
2.1.1.1. Opposite balances. The SOURCE BALANCING AUTHORITY and SINK BALANCING AUTHORITY must have inadvertent accumulations in the opposite direction.

2.1.1.2. Agreement on schedule. The terms of the inadvertent payback INTERCHANGE SCHEDULE shall be agreed upon by all involved BALANCING AUTHORITIES and TRANSMISSION PROVIDERS in accordance with NERC Interchange procedures.

2.1.2. Unilateral payback. INADVERTENT INTERCHANGE accumulations may be paid back unilaterally controlling to a target of non-zero ACE. Controlling to a nonzero ACE ensures that the unilateral payback is accounted for in the CPS calculations. The unilateral payback control offset is limited to BALANCING AUTHORITY ‘s L10 limit and shall not burden the INTERCONNECTION.

2.2. Other payback methods. Upon agreement by all REGIONS within an INTERCONNECTION, other methods of INADVERTENT INTERCHANGE payback may be utilized.

3. Inadvertent Interchange summary. Each BALANCING AUTHORITY shall submit a monthly summary of Inadvertent Interchange as detailed as specified below.  These summaries shall not include any after-the-fact changes that were not agreed to by the Source BALANCING AUTHORITIES, Sink BALANCING AUTHORITIES and all Intermediary BALANCING AUTHORITIES.
3.1. Summary balances. Inadvertent Interchange summaries shall include at least the previous accumulation, net accumulation for the month, and final net accumulation, for both the “on‑peak” and “off-peak” periods.

3.2. Summary submission. Each BALANCING AUTHORITY shall submit its monthly summary report to its Resources Subcommittee Survey Contact by the 15th calendar day of the following month. The Resources Subcommittee Survey Contact will prepare a composite tabulation and submit that tabulation to the NERC staff by the 22nd calendar day of the month.
3.2.1. Failure to Report. A BALANCING AUTHORITY that neither submits a report nor supplies a reason for not submitting the required data by the 20th calendar day of the following month shall be considered non-compliant.
4. Dispute Resolution. Adjacent BALANCING AUTHORITIES that cannot mutually agree upon their respective net actual interchange or net scheduled interchange quantities by the 15th calendar day of the following month shall, for the purposes of dispute resolution, submit a report to their respective resources subcommittee survey contact. The REPORT SHALL DESCRIBE THE NATURE AND THE CAUSE OF THE DISPUTE AS WELL AS A PROCESS FOR CORRECTING THE DISCREPANCY. The dispute resolution process is described in Appendix B, Dispute Resolution Process.
Appendix A – NAESB Inadvertent Interchange On- and Off-Peak Periods

1.
On-Peak and Off-Peak Hours (Monday Through Sunday)

On and Off-Peak designation. The hourly inadvertent energy created by BALANCING AUTHORITIES is classified as either On-Peak or Off-Peak inadvertent. The peak designation assigned is a function of hour of day, day of week, time zone, prevailing time (standard or daylight savings), and special holiday status.

Daylight saving time. The On-Peak to Off-Peak and Off-Peak to On-Peak boundary hours are unaffected by transitions to or from daylight savings time. If BALANCING AUTHORITIES remains on either standard or daylight savings time throughout the year, their inadvertent accounting practices shall use prevailing time.

On-peak hours. Each INTERCONNECTION has a reference time zone and standardized On-Peak and Off-Peak periods. On-Peak periods are summarized in the table below for each INTERCONNECTION. Sundays and special holidays are designated to be Off-Peak periods for the entire day. Hours for Monday through Saturday that are not shown in the table below are also designated as Off-Peak hours.

2.
On-Peak Hours For Monday Through Saturday In Hour-Ending Format

	Interconnection
	Reference Time Zone
	Hour Ending

	
	
	From
	To

	Eastern
	Central
	0700
	2200

	ERCOT
	Central
	0800
	2200

	Western
	Pacific
	0700
	2200


3.
Additional Off-Peak Holidays for the Eastern and Western Interconnections

There are six identified U.S. holidays each year:

· New Year’s Day

· Memorial Day

· Independence Day

· Labor Day

· Thanksgiving Day

· Christmas Day

If any of these holidays fall on a Sunday, the following Monday will be considered an Off-Peak day. Otherwise, the Off-Peak day will be the holiday itself.
 Appendix B – NAESB Inadvertent Interchange Dispute Resolution Process

Introduction

Adjacent BALANCING AUTHORITIES that cannot mutually agree upon their respective Net Interchange quantities by the fifteenth calendar day of the following month shall submit a report to their respective Resources Subcommittee representative. The report shall describe the nature and the cause of the dispute as well as a process for correcting the discrepancy. Should the submitted processes not work, the process for resolving the dispute is described herein.

1. Regional Subcommittee Representative reporting requirements. The Resources Subcommittee representative shall accept the BALANCING AUTHORITY’S report describing the disputed values. To comply with the reporting requirements of NAESB Inadvertent Interchange Requirement 3. That representative shall contact the Resources Subcommittee representative for the opposing BALANCING AUTHORITY (if the dispute is between BALANCING AUTHORITIES in different Regions). The representative(s) shall determine a set of values, which will be reported to NERC. The report(s) will identify:

1.1. The names of the disputing BALANCING AUTHORITIES.

1.2. The reported monthly Net Interchange Schedule (On-Peak and Off-Peak) between the disputing BALANCING AUTHORITIES.

1.3. The mutually agreed to monthly Net Interchange Schedule (On-Peak and Off-Peak) between the disputing BALANCING AUTHORITIES (used to compute the Regional Inadvertent Interchange).

1.4. The reported monthly NET ACTUAL INTERCHANGE (On-Peak and Off-Peak) between the disputing BALANCING AUTHORITIES.

1.5. The mutually agreed to monthly Net Actual Interchange (On-Peak and Off-Peak) between the disputing BALANCING AUTHORITIES (used to compute the Regional Inadvertent Interchange).

2. NERC Staff reporting requirements. The NERC staff representative to the Resources Subcommittee shall receive the Regional reports and, using the mutually agreed to data, compile a balanced Inadvertent Interchange Summary report. This report will also include a tabulated list of the BALANCING AUTHORITIES that have disputed data, as well as the magnitude of the data in dispute. This report will be distributed to the Operating Committee as well as the Resources Subcommittee by the 1st of the succeeding month.

3. Dispute Resolution. All disputes between BALANCING AUTHORITIES within a Region shall be referred to the regional process for dispute resolution to resolve the dispute on an informal basis within 30 days o f the issuance of the NERC Inadvertent Interchange Summary report.
3.1. All disputes between BALANCING AUTHORITIES in different Regions shall be referred to the respective Regions’ Operating Committee representatives, or other Regional-approved representatives, for resolution on an informal basis within 30 days of the issuance of the NERC Inadvertent Interchange Summary report.
3.2. In the event that the informal procedures do not resolve the dispute within 30 days, the dispute shall be submitted to binding arbitration as described below.

4. Binding Arbitration. A professional arbitration service will provide each of the parties in the dispute an opportunity to be heard. Within 30 days of those presentations, the arbitrator shall issue a decision. The decision and the rationale for the decision shall be provided in writing to the disputing parties.


1
3

