Inadvertent Interchange Settlement Standard

Introduction:

The Inadvertent Interchange Payback Task Force (IIPTF) ascertains that Inadvertent is the result of good and poor control practices, built-in accounting anomalies, operator error, and is a necessary result of Interconnected operation in a synchronous multi-Balancing Authority network.  Inadvertent Interchange is defined as the difference between a Balancing Authority’s net actual interchange and its net scheduled interchange.  Inadvertent Interchange is a natural result of interconnected system operations, created unintentionally, and, therefore unscheduled.  The root causes of Inadvertent Interchange are errors in measurement (frequency and/or net actual interchange), errors in schedules (frequency and/or net scheduled interchange), inadequate control systems or telecommunications, and/or energy conversion units responsive to AGC function not available or not assigned.  The standard appropriately holds accountable those Balancing Authorities exhibiting poor control practices and recognize those exhibiting good control practices.
Include language that the standard preserves the procedures for settlement used today, but  enhance them to permit financial settlement. ‘as it is today’
Business Practices:

1.0  
Inadvertent Payback will incorporate a bandwidth defined by frequency where inadvertent accumulated within the bandwidth is paid back pursuant to Section 5, inadvertent outside the bandwidth would be settled on a financial basis where there is a fixed price with discovery (see Section 8) and a settlement entity would match counterparties.  

2.0 Inadvertent Interchange accumulation data will be provided by NERC. 

· NERC has agreed to provide inadvertent energy and frequency deviation measurements with hourly granularity.  Although payback of inadvertent energy and contribution to frequency deviation shall be performed for each hour, reporting of such hourly information to be used for settlement purposes shall occur at such time intervals as deemed feasible by NERC. The IIPTF will accept the limitation of monthly measurement reporting interval until such time that technology and NERC allow this data to be reported more frequently.

2.1 Need to discuss how cumulative account balances are updated once there is a payback.  Who’s going to keep the balances? INADVERTENT INTERCHANGE Accounting. Adjacent Balancing Authorities shall operate to a common Net Interchange Schedule and Actual Net Interchange value and shall record these hourly quantities, with like values but opposite sign. Each Balancing Authority shall compute its INADVERTENT INTERCHANGE based on the following:
2.1.1 Daily accounting. Each Balancing Authority, by the end of the next business day, shall agree with its adjacent CONTROL AREAS to:
2.1.1.1 The hourly values of Net Interchange Schedule.
2.1.1.2 The hourly integrated MWh values of Net Actual Interchange
2.2 Monthly accounting. Each Balancing Authority shall use the agreed-to Daily and Monthly accounting data to compile its monthly accumulated INADVERTENT INTERCHANGE for the On-Peak and Off-Peak hours of the month. [Refer to “Inadvertent Interchange Accounting Training Document”]
2.3 After-the-Fact Corrections. After-the-fact corrections to the agreed-to Daily and Monthly accounting data shall only be made to reflect actual operating conditions (e.g. a meter being used for control was sending bad data). Changes or corrections based on non-reliability considerations shall not be reflected in the Balancing Authority’s INADVERTENT INTERCHANGE. After-the-fact corrections to scheduled or actual values will not be accepted without agreement of the Adjacent  Balancing Authority (s).
3.0
Inadvertent energy and relative frequency will be measured and reported in hourly granularity.  Going to have to address how will this data be adjusted by that inadvertent settled financially that lies outside the bandwidth. (move this up to 2.  uplift 3.1 to 3.0).  This does not affect Illian point.  
3.1 On Peak/Off Peak definition (there are differences in West.  Henery will find that incase we need to include the language)

· On-Peak and Off-Peak Hours (Monday Through Sunday)

· On- and Off-Peak designation. The hourly inadvertent energy created by a Balancing Authority is classified as either On-Peak or Off-Peak inadvertent. The peak designation assigned is a function of hour of day, day of week, time zone, prevailing time (standard or daylight savings), and special holiday status.

· Daylight saving time. The On-Peak to Off-Peak and Off-Peak to On-Peak boundary hours are unaffected by transitions to or from daylight savings time. If a Balancing Authority remains on either standard or daylight savings time throughout the year, their inadvertent accounting practices shall use prevailing time.

· On-peak hours. Each INTERCONNECTION has a reference time zone and standardized On-Peak and Off-Peak periods. On-Peak periods are summarized in the table below for each INTERCONNECTION. Sundays and special holidays are designated to be Off-Peak periods for the entire day. Hours for Monday through Saturday that are not shown in the table below are also designated as Off-Peak hours.

· On-Peak Hours For Monday Through Saturday In Hour-Ending Format

	· Interconnection
	· Reference Time Zone
	· Hour Ending

	· 
	· 
	· From
	· To

	· Eastern
	· Central
	· 0700
	· 2200

	· ERCOT
	· Central
	· 0800
	· 2200

	· Western
	· Pacific
	· 0700
	· 2200


· Additional Off-Peak Holidays for the Eastern and Western Interconnections

· There are six identified U.S. holidays each year:

· New Year’s Day

· Memorial Day

· Independence Day

· Labor Day

· Thanksgiving Day

· Christmas Day

· If any of these holidays fall on a Sunday, the following Monday will be considered an Off-Peak day. Otherwise, the Off-Peak day will be the holiday itself.

4.0  
Frequency bandwidth is plus or minus 20 mHz around scheduled frequency.

5.0 Settlement inside the frequency bandwidth will be as it is today or financial.

5.1 Policy 1, Section F5 – Inadvertent Interchange Payback.   

· Inadvertent Interchange payback. Each Balancing Authority shall be diligent in reducing accumulated inadvertent balances. Inadvertent Interchange accumulations shall be paid back by either of the following methods:

· Energy “in-kind” payback.  Inadvertent Interchange accumulated during “on-peak” hours shall only be paid back during “on‑peak” hours. Inadvertent Interchange accumulated during “off-peak” hours shall only be paid back during “off-peak” hours. [See Appendix 1F, “On-Peak and Off-Peak Periods.”]

· Bilateral payback.  Inadvertent Interchange accumulations may be paid back via an Interchange Schedule with another Balancing Authority. [Refer to Policy 3, “Interchange” for Interchange Scheduling Requirements.]
· Opposite balances. The Source Balancing Authority and Sink Balancing Authority must have inadvertent accumulations in the opposite direction.

· Agreement on schedule. The terms of the inadvertent payback Interchange schedule shall be agreed upon by all involved Balancing Authorities and transmission Providers in accordance with NERC operating Policy 3, “Interchange.”
· Unilateral payback. Inadvertent Interchange accumulations may be paid back unilaterally controlling to a target of non-zero ACE. Controlling to a non-zero ACE ensures that the unilateral payback is accounted for in the CPS calculations. The unilateral payback control offset is limited to the Balancing Authority ‘s L10 limit and shall not burden the Interconnection.  In the event L10 no longer exists, a replacement will be developed.
· Other payback methods. Upon agreement by all Regions within an Interconnection, other methods of Inadvertent Interchange payback may be utilized.
5.2 Financial Settlement of Inadvertent will be allowed …….

6.0 Settlement outside the frequency bandwidth will be settled financially.  Possibly include chart as supporting information.  How does this affect negative pricing?

6.1 At frequencies greater than or equal to 20 mHz over scheduled frequency, 

6.1.1 The over generating balancing authority will be paid $0 per MWh from the under generating balancing authority.  

6.1.2 The under generating balancing authorities will pay $0 per MWh to the over generating balancing authorities.

6.1.2.1 Settlement according to this section will be subject to price discovery.  

6.2 At frequencies less than or equal to 20 mHz below scheduled frequency,

6.2.1 The over generating balancing authority will be paid the greater of $125 per MWh or a discoverable price detailed in 8.0 from the under generating balancing authority. 

6.2.2 The under generating balancing authorities will pay $125 per MWh to the over generating balancing authorities.

6.2.2.1 Settlement according to this section will be subject to price discovery.  

7.0 Settlement Entity

7.1 The settlement entity will receive information from NERC and compute any settlements that may be required for the given month on a per hour basis.  This entity will also match up payer and payee based upon:

7.1.1 the closest full dollar amounts due.  

7.2 A balanced collection will be utilized to assure that no balances remain at the end of the settlement period. Define balanced collection.

7.3 How should the settlement entity be paid?  Fixed percentage or fixed fee per transaction?
7.4 Credit – possibly an addition to NERC BA certification.
8.0 Price Discovery  

8.1 "Replacement Price" means the price at which Buyer, acting in a commercially reasonable manner, purchases at the Delivery Point a replacement for any Product specified in a Transaction but not delivered by Seller, plus (i) costs reasonably incurred by Buyer in purchasing such substitute Product and (ii) additional transmission charges, if any, reasonably incurred by Buyer to the Delivery Point, or at Buyer's option, the market price at the Delivery Point for such Product not delivered as determined by Buyer in a commercially reasonable manner; provided, however, in no event shall such price include any penalties, ratcheted demand or similar charges, nor shall Buyer be required to utilize or change its utilization of its owned or controlled assets or market positions to minimize Seller's liability.  For the purposes of this definition, Buyer shall be considered to have purchased replacement Product to the extent Buyer shall have entered into one or more arrangements in a commercially reasonable manner whereby Buyer repurchases its obligation to sell and deliver the Product to another party at the Delivery Point.
8.2 "Sales Price" means the price at which Seller, acting in a commercially reasonable manner, resells at the Delivery Point any Product not received by Buyer, deducting from such proceeds any (i) costs reasonably incurred by Seller in reselling such Product and (ii) additional transmission charges, if any, reasonably incurred by Seller in delivering such Product to the third party purchasers, or at Seller's option, the market price at the Delivery Point for such Product not received as determined by Seller in a commercially reasonable manner; provided, however, in no event shall such price include any penalties, ratcheted demand or similar charges, nor shall Seller be required to utilize or change its utilization of its owned or controlled assets, including contractual assets, or market positions to minimize Buyer's liability.  For purposes of this definition, Seller shall be considered to have resold such Product to the extent Seller shall have entered into one or more arrangements in a commercially reasonable manner whereby Seller repurchases its obligation to purchase and receive the Product from another party at the Delivery Point.

9.0
Prior to full implementation there will be a one year test period.  

9.1
The test period will follow all of the normal criteria as stated in the Standard.  This will allow for Real Time, Real Data comparisons and the opportunity to make adjustments prior to full implementation.  

9.2
mandatory review process

