Inadvertent Interchange Settlement Standard

Introduction:

Inadvertent Interchange is defined as the difference between a Balancing Authority’s net actual interchange and its net scheduled interchange.  The Inadvertent Interchange Payback Task Force (IIPTF) ascertains that Inadvertent Interchange is a natural result of interconnected system operations.  The root causes of Inadvertent Interchange are errors in measurement (frequency and/or net actual interchange), errors in schedules (frequency and/or net scheduled interchange), inadequate control systems or telecommunications, and/or energy conversion units responsive to AGC function not available or not assigned.  The standard appropriately holds accountable those Balancing Authorities exhibiting poor control practices and recognizes those exhibiting good control practices.

Business Practices:

1. Inadvertent Payback will incorporate a bandwidth defined by frequency where inadvertent accumulated within the bandwidth is paid back pursuant to Section 5, inadvertent outside the bandwidth would be settled on a financial basis (see Section 6) where there is a fixed price with discovery (see Section 8).  

2. Hourly Inadvertent Interchange data and hourly integrated frequency data will be provided to the Settlement Entity by NERC
. 

2.1.1. The Settlement Entity will notify the Balancing Authorities and NERC simultaneously the hours that a financial settlement trigger has occurred.

2.1.2. Once an hourly accumulation is determined to be financial, those amounts will be removed from the NERC Inadvertent Interchange balances by all parties involved.    

2.2. INADVERTENT INTERCHANGE Accounting. Adjacent Balancing Authorities shall operate to a common Net Interchange Schedule and Actual Net Interchange value and shall record these hourly quantities, with like values but opposite sign. Each Balancing Authority shall compute its INADVERTENT INTERCHANGE based on the following:
2.2.1. Daily accounting. Each Balancing Authority, by the end of the next business day, shall agree with its adjacent CONTROL AREAS to:
· The hourly values of Net Interchange Schedule.
· The hourly integrated MWh values of Net Actual Interchange
2.3. NERC and the Settlement Entity will maintain hourly Inadvertent Interchange Balances for all Balancing Authorities.  

2.3.1. Upon payback, regardless of settlement method, the Balancing Authorities involved in the settlement must notify the Settlement Entity that payback has occurred.  

2.3.2. The Settlement Entity will verify with all parties that the described payback has occurred and once verified, will correspondingly adjust the individual Balancing Authorities’ balances to reflect the settlement.  

2.3.3. The Settlement Entity will then notify NERC of the balance adjustment, as to facilitate the same adjustment to the balances maintained at NERC.  

2.3.4. All Balancing Authorities shall be responsible for keeping records that will provide a clear history of all of its Inadvertent Interchange transactions.

2.4. Monthly accounting. Each Balancing Authority shall use the agreed-to Daily and Monthly accounting data to compile its monthly accumulated INADVERTENT INTERCHANGE for the On-Peak and Off-Peak hours of the month. 
2.5. After-the-Fact Corrections. After-the-fact corrections to the agreed-to Daily and Monthly accounting data shall only be made to reflect actual operating conditions (e.g. a meter being used for control was sending bad data). Changes or corrections based on non-reliability considerations shall not be reflected in the Balancing Authority’s INADVERTENT INTERCHANGE. After-the-fact corrections to scheduled or actual values will not be accepted without agreement of the Adjacent Balancing Authority (s).
On Peak/Off Peak definition 

3. The hourly inadvertent energy created by a Balancing Authority is classified as either On-Peak or Off-Peak inadvertent. The peak designation assigned is a function of hour of day, day of week, time zone, prevailing time (standard or daylight savings), and special holiday status.

3.1. Daylight saving time. The On-Peak to Off-Peak and Off-Peak to On-Peak boundary hours are unaffected by transitions to or from daylight savings time. If a Balancing Authority remains on either standard or daylight savings time throughout the year, their inadvertent accounting practices shall use prevailing time.

3.1.1. On-peak hours. Each INTERCONNECTION has a reference time zone and standardized On-Peak and Off-Peak periods. On-Peak periods are summarized in the table below for each INTERCONNECTION. Sundays and special holidays are designated to be Off-Peak periods for the entire day. Hours for Monday through Saturday that are not shown in the table below are also designated as Off-Peak hours.

On-Peak Hours For Monday Through Saturday In Hour-Ending Format

	· Interconnection
	· Reference Time Zone
	· Hour Ending

	· 
	· 
	· From
	· To

	· Eastern
	· Central
	· 0700
	· 2200

	· ERCOT
	· Central
	· 0800
	· 2200

	· Western
	· Pacific
	· 0700
	· 2200


Additional Off-Peak Holidays for the Eastern and Western Interconnections
There are six identified U.S. holidays each year:

· New Year’s Day

· Memorial Day

· Independence Day

· Labor Day

· Thanksgiving Day

· Christmas Day

3.1.2. If any of these holidays fall on a Sunday, the following Monday will be considered an Off-Peak day. Otherwise, the Off-Peak day will be the holiday itself.

4. Frequency bandwidth is plus or minus 20 mHz around HOURLY INTEGRATED  scheduled  frequency.

Settlement inside the frequency bandwidth 

Inadvertent Interchange accumulations shall be paid back by any of the following methods:
5. Each Balancing Authority shall be diligent in reducing accumulated inadvertent balances. 
5.1. Energy “in-kind” payback.  Inadvertent Interchange accumulated during “on-peak” hours shall only be paid back during “on‑peak” hours. Inadvertent Interchange accumulated during “off-peak” hours shall only be paid back during “off-peak” hours. 

5.2. Bilateral Payback. Inadvertent Interchange accumulations may be paid back via an Interchange Schedule with another Balancing Authority. 

5.2.1. Opposite balances. The Source Balancing Authority and Sink Balancing Authority must have inadvertent accumulations in the opposite direction.

5.2.2. Agreement on schedule. The terms of the inadvertent payback Interchange schedule shall be agreed upon by all involved Balancing Authorities and Transmission Providers. 

5.3.      Unilateral payback. Inadvertent Interchange accumulations may be paid back unilaterally controlling to a target of non-zero ACE. Controlling to a non-zero ACE ensures that the unilateral payback is accounted for in the CPS calculations. The unilateral payback control offset is limited to the Balancing Authority ‘s L10 limit and shall not burden the Interconnection.  In the event L10 no longer exists, a replacement will be developed.

5.4. Financial Settlement. Inadvertent Interchange accumulations may be paid back via a financial transaction with another Balancing Authority. 

5.4.1. Opposite balances. The Balancing Authorities must have inadvertent accumulations in the opposite direction.

5.4.2. Agreement on schedule. The terms of the inadvertent payback shall be agreed upon by the involved Balancing Authorities and approved by the Settlement Entity

6. Settlement outside the frequency bandwidth will be settled financially.  


[image: image1]
At hourly integrated frequencies greater than 20 mHz over scheduled frequency: 
6.1. The over generating balancing authority will be paid less than or equal to $0 per MWh from the under generating balancing authority.  

6.1.1. Costs incurred by the under generating BALANCING AUTHORITY (S) to accept the inadvertent energy will be passed on to the over generating entity.

6.1.2. The under generating BALANCING AUTHORITY’S cost is discoverable as defined in Section 8.0.  

6.1.3. Upon settlement the inadvertent interchange balances will be adjusted 

At hourly integrated frequencies less than or equal to 20 mHz below scheduled frequency:

6.2. The over generating balancing authority will be paid the greater of $100 per MWh.or a discoverable price from the under generating   balancing authority. 

6.2.1. The over-generating balancing authority’s price is discoverable as defined in Section 8.0.  

6.2.2. Upon settlement the inadvertent interchange balances will be adjusted 

Settlement Entity

7. The settlement entity will receive information from NERC and compute any settlements that may be required for the given month on a per hour basis.

7.1. The settlement entity will match up counter parties. 

8. A balanced collection will be utilized to assure that no balances remain at the end of the settlement period. 

8.1. The settlement entity will collect a fee to be paid by the Balancing Authority(s) not responding appropriately.  

Price Discovery  

9. Any party subject to the settlement has the right to ask for proof of price/cost.

9.1.1. All Payment/Compensation Calculations will be made using the higher of the Long Party’s top incremental cost (i.e., highest purchase price for power or highest generating unit cost) or its highest sales price during the applicable hour.

10. Absent dispute, there will be no price discovery.

11. Price verification/documentation will remain confidential among Short and Long Parties., Sample Payment/Compensation Calculations for one hour under the Inadvertent Settlement Procedure are provided in Attachment C.

12. Prior to full implementation there will be a one year test period.  

12.1. The test period will follow all of the normal criteria as stated in the Standard.  This will allow for Real Time, Real Data comparisons and the opportunity to make adjustments prior to full implementation.  We will include this in a posting letter.

13. Mandatory review process

[image: image2.png]D ) S S S Y S N N Y E

ECAR
CONTROL (ACTUAL] UNILAT|
D CENTER | HOUR | INTER INADV | ACEH| ACE| ACES| Freq. Error
£ [AEP 600CDT | -3%6] q o sg 231 0.047628]
31 [APS 600COT | -169] q o9 21 0.047628]
i BREC 500CDT 301 9 9 | 7 ~0.047648]
3 IV 600CDT | 70%] q o] 7023] 02| 0047628}
37 DLCO 600C0T 2] q 3 7 0047628}
% DPL 600COT | __-410] q 9 9 BL 0047628}
38 EKPC 600C0T q Lo ] g -0.047628]
50 600C0T q o 42 0 0047628}
3 600C0T q q 3 37| 0047628}
40 IPL 600C0T q o 29 28] 0047628}
) LGEE 600C0T q q 5 BE 0047628}
3 MECS 600C0T q q 3 3 0047628}
a4 NIPS 600CDT q o g 56 0047628}
5 600C0T q o9 BE 0047628}
8 SIGE 600COT q o2 7] 0047628}





[image: image3.png]Inadvertent Payment Proposal PaymenticCompensation Caloulations Example

Price Tor
Positive
Inadvertnet
plus
10% Adder
minus.
Per Cent | Price for Energy | Energy
Control| Actual [ Negative | Positve | Negative | Posiive | 10% | Payback | Payoack Total Total
Area_[Inadvertent Inadvertent | nadvertent|inadvertent|inadvertent] Adder | Creait | Credit |Compensation| Payme Lo
# 300] 300 2128] $480679] 100}
£ 50 50 7,000 _§700] 75| s7685 5384250 60}
#3 ~30) 30 2.13] s48.008] 20}
4 ~000[ 000 7052] §7603.262] 00|
#5 0 i 5,000 _§500] 15| s5485 54,850 50
6 ) 0 10
7 30 30 6,000 _§600] $5585] 5167550 50
e E 0 700}
0 20} 20| 4,000 40| 15| s485 $87.700] 50
#10 0] 80 567 $128261 50
Eil 720} 720] 7,000 _§700] $522.200] £
#12 700} 700] 54,000 _5400] 5438 500] 700
#13 20} 20] 5,000 $500] $109.700] a0}
#14 0 0 6,000 $600] $65,850] a0}
#15 20 0 0}
Sum EiF] —ia10) 360]__10000) $2,260,600] §2.260 600)

[Note - Inadvertent rumbers are p

rovided by the ECAR Conlrol Areas

after the monihly Inadvertent

s balanced by the Ni

ERC Performance Subcommittee us

ing standard

NERC Inadvertent bal

lancing

procedures





Scheduled Frequency plus 20mHz





Scheduled Frequency minus 20mHz








Financial Settlement


Section 6.2





Financial Settlement


Section 6.1





Settlement will occur with the same options as we do today


Section 5.1














� NERC has agreed to provide inadvertent energy and frequency deviation measurements with hourly granularity.  Although payback of inadvertent energy and contribution to frequency deviation shall be performed for each hour, reporting of such hourly information to be used for settlement purposes shall occur at such time intervals as deemed feasible by NERC. The IIPTF will accept the limitation of a monthly measurement reporting interval until such time that technology and NERC allow this data to be reported more frequently.





Draft –July 26, 2004


