Changes for CBM and TRM
1. Changes to SOC to comply with Order 890
WEQ-001-1.6(b)(1)(vii) Capacity Benefit Margin or CBM means the amount of TTC preserved by the Transmission Provider for load-serving entities, whose loads are located on that Transmission Provider’s system, to enable access by the load-serving entities to generation from interconnected systems to meet generation reliability requirements, or such definition as contained in Commission-approved Reliability Standards.

WEQ-001-1.6(b)(1)(viii) Transmission Reliability Margin or TRM means the amount of TTC necessary to provide reasonable assurance that the interconnected transmission network will be secure, or such definition as contained in Commission-approved Reliability Standards.
WEQ-001-1.6(b)(2)(ii) On request, the Responsible Party must make all data used to calculate ATC, TTC, CBM, and TRM for any constrained posted paths publicly available (including the limiting element(s) and the cause of the limit (e.g., thermal, voltage, stability), as well as load forecast assumptions) in electronic form within one week of the posting. The information is required to be provided only in the electronic format in which it was created, along with any necessary decoding instructions, at a cost limited to the cost of reproducing the material. This information is to be retained for six months after the applicable posting period.
WEQ-001-1.6(b)(3) Posting. The ATC, TTC, CBM, and TRM for all Posted Paths must be posted in megawatts by specific direction and in the manner prescribed in this subsection.
WEQ-001-1.6(b)(3)(i)(A)(1) The posting shall show ATC, TTC, CBM, and TRM for a 30-day period. For this period postings shall be: by the hour, for the current hour and the 168 hours next following; and thereafter, by the day. If the Transmission Provider charges separately for on-peak and off-peak periods in its tariff, ATC, TTC, CBM, and TRM will be posted daily for each period.
WEQ-001-1.6(b)(3)(i)(B) For non-firm ATC and TTC. The posting shall show ATC, TTC, CBM and TRM for a 30-day period by the hour and days prescribed under paragraph (b)(3)(i)(A)(1) of this section and, if so requested, by the month and year as prescribed under paragraph (b)(3)(i)(A) (2) and (3) of this section. The posting of non-firm ATC and TTC shall show CBM as zero.
WEQ-001-1.6(b)(3)(ii)(A) Postings of firm and nonfirm ATC, TTC, CBM, and TRM shall be posted separately by the day, showing for the current day and the next six days following and thereafter, by the month for the 12 months next following. If the Transmission Provider charges separately for on-peak and off-peak periods in its tariff, ATC, TTC, CBM, and TRM will be posted separately for the current day and the next six days following for each period. These postings are to be updated whenever the ATC changes by more than 20 percent of the Path's TTC.
WEQ-001-1.6(b)(3)(iii) Calculation of CBM.

(A) The Transmission Provider must reevaluate its CBM needs at least every year. Docket Nos. RM05-17-000 and RM05-25-000 - 1049 – 

(B) The Transmission Provider must post its practices for reevaluating its CBM needs.
2. Changes to WEQ-002 (S&CP) and WEQ-011 (Implementation guide)
CBM & TRM data has already been defined in 002-4.3.4.4 and in 011-3.3.1
Information for the CBM & TRM methodology has already been defined in 002-4.5.1

CBM&TRM data 

