	NERC TLR Reliability Standards (IRO-006-?)
Example page
	NAESB TLR Business Practices Standards WEQBPS-008 Version 1.0 – Example Page



2. Transmission Loading Relief (TLR) Levels and Approved Tag Submission Deadline for Reallocation

Introduction

This requirement describes the various levels of the TLR Procedure. The levels are not meant to imply a required sequence—the Reliability Coordinator can vary the sequence based on system conditions. .  This requirement also states the Approved Tag Submission Deadline for Reallocation during TLR levels 3a and 5a.

2.1. TLR Level 1 – Notify Reliability Coordinators of potential SOL or IROL Violations.

2.1.1. The Reliability Coordinator shall use the following circumstances to establish the need for TLR Level 1:

· The transmission system is secure.

· The Reliability Coordinator foresees a transmission or generation contingency or other operating problem within its Reliability Area that could cause one or more transmission facilities to approach or exceed their SOL or IROL.

2.2. TLR Level 2 – Hold transfers at present level to prevent SOL or IROL Violations
2.2.1. The Reliability Coordinator shall use the following circumstances to establish the need for entering TLR Level 2:

· The transmission system is secure,

One or more transmission facilities are expected to approach, or are approaching, or are at their SOL or IROL

3 Eastern Interconnection Procedure for Physical Curtailment of Interchange Transactions

3.1 TLR level 1.  When a Reliability Coordinator has initiated a TLR level 1 (Notify all Reliability Coordinators of potential SOL or IROL Violations), the Reliability Coordinator shall take no action against any Interchange Transaction. 

3.2 TLR level 2.  When a Reliability Coordinator has initiated a TLR level 2 (Hold transfers at present level to prevent SOL or IROL Violations), the Reliability Coordinator shall take the following actions: 

3.2.1 The Reliability Coordinator should ensure that TLR level 2 is a transient state so that Interchange Transactions are properly initiated according to their transmission reservation priority. 

3.2.1.1 The Reliability Coordinator should make best efforts possible to ensure that TLR level 2 does not exceed 30 minutes in duration.

3.2.1.2 If TLR level 2 exceeds 30 minutes in duration, the Reliability Coordinator shall document this action on the NERC TLR log.

3.2.2 The Reliability Coordinator shall hold the implementation of any additional Interchange Transactions using Non-Firm Transmission Service that are at or above the Curtailment Threshold.  

3.2.3 The Reliability Coordinator shall allow additional Interchange Transactions that flow across the Constrained Facility or Flowgate to be initiated if their flow reduces the loading on the Constrained Facility or Flowgate or has a Transfer Distribution Factor (TDF) less than the Curtailment Threshold. 

The Reliability Coordinator shall allow all Interchange Transactions using Firm Transmission Service to be initiated.
NAESB Business Practice Reference—Additional implementation requirements contained in NAESB TLR Business Practice, Section 3.2








