Postbacks for Posted ATC and AFC
ATC and AFC equations use different rules for determining postbacks based on whether the calculation is for Firm or Non-Firm. Firm calculations utilize the term PostbacksF, while non-firm calculations use the term PostbacksNF.  The two values are defined with the following equations:

PostbacksF = PostbacksFRD  + PostbacksFRS  + PostbacksFRC  + PostbacksFO
Where 

PostbacksFRD  are Firm Redirects.  A Firm Redirect of a Firm Point-to-Point reservation causes a postback to Firm ATC on the parent reservation’s path for the time interval and amount of the Firm Redirect. Note that a Non-Firm Redirect of a Firm reservation does not impact the Firm ATC on the parent reservation’s path, and should not be included.
PostbacksFRS are Firm Resales. A Firm Resale of a Firm Point-to-Point reservation causes a postback to Firm ATC for the time interval and amount of the Firm Resale, which is immediately used by the Confirmed Resale Request. 

PostbacksFRC are Firm Recalls. A Recall of Firm Capacity that does not result in a status of DISPLACED causes a postback to Firm ATC for the time interval and amount of the Recall.

PostbacksFO are other Firm postbacks, as defined in tariffs or other business practices and as described in the ATCID.

PostbacksNF = PostbacksNFU  + PostbacksNFRC  + PostbacksNFO
Where
PostbacksNFU  are all unscheduled Firm Transmission Service 
after the Firm scheduling deadline as described in the ATCID.
, less Firm Postbacks.  Unscheduled Firm Transmission Service (including service that was never scheduled and service that was scheduled but terminated at the customer’s request) 
must be offered as Non-Firm once the Firm scheduling deadline has passed.  Firm Postbacks are already accounted for in the Non-Firm ATC calculation, and should be deducted to eliminate double counting. 

PostbacksNFRC are Non-Firm Recalls. A Recall of Non-Firm Capacity that does not result in a status of DISPLACED causes a postback to Non-Firm ATC for the time interval and amount of the Recall.

PostbacksNFO are other Non-Firm postbacks, as defined in tariffs or other business practices and as described in the ATCID.

Determination of Internal ATC and AFC 

Entities may use additional terms in the ATC and AFC equations to determine Internal ATC and AFC values as described below for both Firm and Non-Firm.  
If these terms are used, the entity must document their use in the ATCID.
ATCFI = ATCF​ – RequestsFO
Where 

RequestsFO is one or more of the following 

· Requests for Firm Transmission Service in a state of STUDY

· Requests for Firm Transmission Service in a state of ACCEPTED

· Requests for Firm Transmission Service in a state of COUNTEROFFER

· Requests for Firm Transmission Service in a state of REBID

ATCNFI = ATCNF​ – RequestsNFO
Where 

RequestsNFO is one or more of the following 

· Requests for Non-Firm Transmission Service in a state of STUDY

· Requests for Non-Firm Transmission Service in a state of ACCEPTED

· Requests for Non-Firm Transmission Service in a state of COUNTEROFFER

· Requests for Non-Firm Transmission Service in a state of REBID
AFCFI = AFCF​ – RequestsFOi
Where 

RequestsFOi is the impacts on flowgates of one or more of the following 

· Requests for Firm Transmission Service in a state of STUDY

· Requests for Firm Transmission Service in a state of ACCEPTED

· Requests for Firm Transmission Service in a state of COUNTEROFFER

· Requests for Firm Transmission Service in a state of REBID

AFCNFI = AFCNF​ – RequestsNFOi
Where 

RequestsNFOi is the impacts on flowgates of one or more of the following 

· Requests for Non-Firm Transmission Service in a state of STUDY

· Requests for Non-Firm Transmission Service in a state of ACCEPTED

· Requests for Non-Firm Transmission Service in a state of COUNTEROFFER

· Requests for Non-Firm Transmission Service in a state of REBID
�Is this correct?  Reading the S&CP, it appears to be.


�NAESB needs to add requirement that ATCID include this.


�Intended to cover Firm Network too.


�NAESB needs to add requirement that ATCID include this.


�Intent of this is to cover all those “tags.”


�NAESB needs to add requirement that ATCID include this.


�All the postbacks for things like retracted, superseded, etc..., are unnecessary if you say this.  Then, once they get out of these transient states, they automatically get posted back.  Not sure the language is quite right, but you get the idea.  





