Draft Lists of Postbacks


 
The following actions impact both the posted ATC and the internal adjusted ATC (to recognize pending requests). It is recognized that a Provider’s use of this list will be conditional of the approved tariff of the Transmission Provider. Not all may be incorporated by every Transmission Provider.
Concept of counter flows being used as Postbacks needs to be addressed. 
Postback definition: The change in ATC:

Category 1: due to a change in status or capacity (such as a redirect) of a Transmission Service request prior to inclusion in the recalculation of ATC/ETC (e.g. from confirmed),

Category 2: due to a potential change in status or capacity (such as a counteroffer) or inclusion of a Transmission Service request (i.e. pending – e.g. not confirmed), 
Category 3:  if all firm reservations (confirmed) are included in ETC for use in the non-firm ATC calculation, the inclusion of unscheduled firm Transmission Service will be a postback in the determination of non-firm ATC
Category 4:  if scheduled firm transmission service is included in the ETC for use in the non-firm ATC calculation, there is no need to postback unscheduled Transmission Service

Category 5:  if all non-firm reservations (confirmed) are included in ETC for use in the non-firm ATC calculation, the inclusion of unscheduled non-firm Transmission Service will be a postback in the determination of non-firm ATC

Category 6:  if scheduled non-firm transmission service is included in the ETC for use in the non-firm ATC calculation, there is no need to postback unscheduled Transmission Service

Example:
If you have 100 MW of firm reservations, and 60 MW were scheduled (40 MW unscheduled) on that 100 MW, during the scheduling horizon:

ATCF = TTC – 100(ETC Firm) – CBM - TRM

#3 ATCNF = TTC – 100(ETC Firm) – CBM – TRM + 40(PostBack)
#4 ATCNF = TTC – 60(ETC Firm) – CBM – TRM + 0 (PostBack)
Interim Postbacks may occur when, due to the methodology and the TP’s ATC calculation and posting frequency used for determining ATC, there is a lag between action on a reservation and the formal recalculation of ATC reflecting the action on the reservation. 

Actions that remove a reservation from being a firm commitment (e.g. annulment, displacement) are ETC and don’t require a business practice.
Draft List of Postbacks that apply to both Internally Adjusted ATC and Posted ATC

	POSTBACK
	ACTION
	Internal Only?
	Category

FATC
	Category

NFATC
	NAESB BP

	1. Annulled, Displaced, Recalled Firm reservation posts back to F ATC and may also post back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being annulled. 

	Annul
Displace,

Recall
	
	1
	1 or during Scheduling 

3, 4 if scheduled
	Annulled and Displaced are defined states. Displaced is in 013-2.4
Recalled in 013-2.5.

	2. Annulled, Displaced, Recalled Non-Firm reservation posts back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being annulled. 

	Annul, Displace,
Recall
	
	N/A
	1 or during Scheduling horizon 

5, 6 if scheduled
	Annulled and Displaced are defined states. Displaced is in 013-2.4

Recalled in 013-2.5.

	3. 
	
	
	
	

	

	4. 
	
	
	
	

	

	5. Firm Redirect of a Firm PTP causes post back to F ATC on the path of the Parent reservations’ path for the time interval of the Firm Redirect and may also post back to NF ATC depending on the horizon.  
(Note:  Non-Firm Redirect of a Firm reservation does not impact the Firm ATC on the Parent Reservation’s path.) 
	Firm Redirect
	
	1
	1 if prior to the scheduling horizon

	Firm Redirects in 001-9. 

	6. Firm Redirect of a Firm PTP may cause post back to F ATC on the path of the Parent reservations’ path for rollover rights that were set-aside in the Firm ATC calculation.  This is dependent on the term of the Firm Redirect. 

	Firm Redirect Rollover Rights
	
	1
	1
	Rollover (renewal) of Redirects is in 001-9.7

	7. 
	
	
	
	
	

	8. 
	
	
	
	
	

	9. Downward adjustments and terminations of Firm Network schedule (7FN) may post back to NF ATC depending on the horizon. 
	Release of Unscheduled Firm Network to NF
	
	N/A
	3,4 in Scheduling Horizon
	ESS/ITS is currently working on NITS-related BPS.

	10. 
	
	
	
	
	

	11. Downward adjustments and terminations of Firm PTP schedules (7F) may post back to NF ATC depending on the horizon. 
	Release of Unscheduled Firm PtP to NF
	
	N/A
	3,4 in Scheduling Horizon
	Do not believe that release of unscheduled capacity is addressed in any NAESB BPS.

	12. Downward adjustments and terminations of Non-Firm PTP schedules (NERC priority 2NH-5NM), Secondary Network schedules (6NN), Secondary PTP schedules (1NS)   may post back to NF ATC depending on the horizon. 
	Release of Unscheduled Non-Firm to NF
	
	N/A
	N/A
 Except when TPs ATCID specifies, then 5, 6 in Scheduling Horizon 
	Do not believe that release of unscheduled capacity is addressed in any NAESB BPS.

	13. 
	
	
	
	
	

	14. 
	
	
	
	
	

	15. Unscheduled Firm Network reservations posts back to NF ATC. 
	Release of Unscheduled Firm Network to NF
	
	N/A
	3, 4 in Scheduling Horizon
	Do not believe that release of unscheduled capacity is addressed in any NAESB BPS. ESS/ITS is currently working on NITS-related BPS. 

	16. Unscheduled Firm PTP reservations posts back to NF ATC. 
	Release of Unscheduled Firm PtP to NF
	
	N/A
	3,4 in Scheduling Horizon
	Do not believe that release of unscheduled capacity is addressed in any NAESB BPS.

	17. Unscheduled Non-Firm PTP reservations (including Secondary PTP) posts back to NF ATC. 

	Release of Unscheduled Non-Firm PtP Reservation to NF
	
	N/A
	N/A Except when TPs ATCID specifies then 5, 6 in Scheduling Horizon 
	Do not believe that release of unscheduled capacity is addressed in any NAESB BPS.

	18. Unscheduled Secondary Network reservations (Non-Firm Network – as info, this is not a redirect of a Firm Network) posts back to NF ATC. 

	Release of Unscheduled Secondary Network Reservation to NF
	
	N/A
	N/A Except when TPs ATCID specifies then 5, 6 in Scheduling Horizon
	Do not believe that release of unscheduled capacity is addressed in any NAESB BPS. ESS/ITS is currently working on NITS-related BPS.

	19. 
	
	
	N
	Y
	

	20. Termination of a DNR that then results in the termination/ reduction of a firm transmission reservation posts back to F ATC and may also post back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being annulled. 

	Termination of DNR
	
	1
	1 or during Scheduling 

3, 4 if scheduled
	ESS/ITS is currently working on NITS-related BPS.

	21. Terminated Secondary Network reservation posts back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being annulled. 

	Termination of Secondary Network
	
	N/A
	5, 6 in Scheduling Horizon for some TPs
	ESS/ITS is currently working on NITS-related BPS.

	22. Relinquish secondary non-firm PtP capacity
	Relinquish
	
	N/A
	1 if prior to Scheduling horizon; N/A during scheduling horizon, unless TP’s ATCID allows during scheduling then 

5, 6
	001-10.5.

	23. 
	
	
	
	
	


 
 
The following actions impact only the internal adjusted ATC (to recognize pending requests). It is recognized that a Provider’s use of this list will be conditional of the approved tariff of the Transmission Provider. Not all may be incorporated by every Transmission Provider.
Draft List of Postbacks for Internally Adjusted ATC Only

	POSTBACK
	ACTION
	FATC
	NFACT
	NAESB BP

	1. Counteroffer with lower MW of a Firm PTP or Network request posts back to F ATC and may also post back to NF ATC depending on the horizon. 

	Counteroffer
	Y
	Y
	

	2. 
	
	
	Y
	

	3. 
	
	
	
	

	4. 
	
	
	
	

	5. Counteroffer with lower MW of a Non-Firm PTP (including Secondary) or Secondary Network request may post back to NF ATC depending on the horizon. 

	Counteroffer
	2
	2
	

	6. 
	
	
	
	

	7. 
	
	
	
	

	8. 
	
	
	
	

	9. 
	
	
	
	

	10. 
	
	
	
	

	11. Retracted Firm Network request that has been Accepted or counteroffered by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 

	Retract
	2
	2 if not Scheduling Horizon
	

	12. 
	
	
	
	

	13. Retracted Firm PTP request that has been Accepted by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 

	Retract
	Y
	Y
	

	14. Retracted Firm PTP request that has been Counteroffered by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 

	Retract
	Y
	Y
	

	15. Retracted Non-Firm PTP request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 

	Retract
	N
	Y
	

	16. Retracted Non-Firm PTP request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 

	Retract
	N
	Y
	

	17. Retracted Secondary Network request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 

	Retract
	N
	Y
	

	18. Retracted Secondary Network request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 

	Retract
	N
	Y
	

	19. Retracted Secondary PTP request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 

	Retract
	N
	Y
	

	20. Retracted Secondary PTP request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 

	Retract
	N
	Y
	

	21. Superseded Firm Network request that has been Accepted by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 

	Supersede
	Y
	Y
	

	22. Superseded Firm Network request that has been Counteroffered by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 

	Supersede
	Y
	Y
	

	23. Superseded Firm PTP request that has been Accepted by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 

	Supersede
	Y
	Y
	

	24. Superseded Firm PTP request that has been Counteroffered by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 

	Supersede
	Y
	Y
	

	25. Superseded Non-Firm PTP request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 

	Supersede
	N
	Y
	

	26. Superseded Non-Firm PTP request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 

	Supersede
	N
	Y
	

	27. Superseded Secondary Network request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 

	Supersede
	N
	Y
	

	28. Superseded Secondary Network request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 

	Supersede
	N
	Y
	

	29. Superseded Secondary PTP request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 

	Supersede
	N
	Y
	

	30. Superseded Secondary PTP request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 

	Supersede
	N
	Y
	

	31. Withdrawn Firm Network request that has been Accepted by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 

	Withdraw
	Y
	Y
	

	32. Withdrawn Firm Network request that has been Counteroffered by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 

	Withdraw
	Y
	Y
	

	33. Withdrawn Firm PTP request that has been Accepted by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 

	Withdraw
	Y
	Y
	

	34. Withdrawn Firm PTP request that has been Counteroffered by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 

	Withdraw
	Y
	Y
	

	35. Withdrawn Non-Firm PTP request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 

	Withdraw
	N
	Y
	

	36. Withdrawn Non-Firm PTP request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 

	Withdraw
	N
	Y
	

	37. Withdrawn Secondary Network request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 

	Withdraw
	N
	Y
	

	38. Withdrawn Secondary Network request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 

	Withdraw
	N
	Y
	

	39. Withdrawn Secondary PTP request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 

	Withdraw
	N
	Y
	

	40. Withdrawn Secondary PTP request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 

	Withdraw
	N
	Y
	

	41. Unused TRM firm and non-firm

	
	
	
	

	42. Unused CBM firm and non-firm

	
	
	
	


�Should not be included, as these states are not “Confirmed” and would not be in ATC anyway,


�Should not be included, as these states are not “Confirmed” and would not be in ATC anyway,


�Should not be included, as these states are not “Confirmed” and would not be in ATC anyway,


�Should not be included, as these states are not “Confirmed” and would not be in ATC anyway,


�I did not include this because I don’t understand why this would be true.


�Since roll-over rights are further than a year out, do we need them?  If so, do you grant rollover on the redirect, or on the original (only should postback if you grant rollover on the redirect). 


�Don’t think this is correct, is it?  This would mean that NF ATC never went down until scheduled.  If that was the case, then why would I need a non-firm ATC based on any reservations at all?


�Don’t think this is correct, is it?  This would mean that NF ATC never went down until scheduled.  If that was the case, then why would I need a non-firm ATC based on any reservations at all?


�Don’t think this is correct, is it?  This would mean that NF ATC never went down until scheduled.  If that was the case, then why would I need a non-firm ATC based on any reservations at all?


�Should not be included, as only DNRs are used to determine firm ATC.  By un-designating it, it is removed automatically.  


�Not sure about this one, but thought you did not need a reservation for Secondary.  


�I think you mean REBID (customer can’t counteroffer), but REBID can’t change MW.  Right?


�I think you mean REBID (customer can’t counteroffer), but REBID can’t change MW.  Right?


�I think you mean REBID (customer can’t counteroffer), but REBID can’t change MW.  Right?


�I think you mean REBID (customer can’t counteroffer), but REBID can’t change MW.  Right?


�Counteroffer results in a new value for the request.  No need to postback. Note that customer cannot counteroffer; they can REBID


�Counteroffer results in a new value for the request.  No need to postback. Note that customer cannot counteroffer; they can REBID


�I think you mean REBID (customer can’t counteroffer), but REBID can’t change MW.  Right?


�I think you mean REBID (customer can’t counteroffer), but REBID can’t change MW.  Right?


�I think you mean REBID (customer can’t counteroffer), but REBID can’t change MW.  Right?


�I think you mean REBID (customer can’t counteroffer), but REBID can’t change MW.  Right?


�RETRACT is a final state, and the TSP should not post back; they should ignore all retracted.


�RETRACT is a final state, and the TSP should not post back; they should ignore all retracted.


�RETRACT is a final state, and the TSP should not post back; they should ignore all retracted.


�RETRACT is a final state, and the TSP should not post back; they should ignore all retracted.


�RETRACT is a final state, and the TSP should not post back; they should ignore all retracted.


�RETRACT is a final state, and the TSP should not post back; they should ignore all retracted.


�RETRACT is a final state, and the TSP should not post back; they should ignore all retracted.


�RETRACT is a final state, and the TSP should not post back; they should ignore all retracted.


�RETRACT is a final state, and the TSP should not post back; they should ignore all retracted.


�RETRACT is a final state, and the TSP should not post back; they should ignore all retracted.


�SUPERSEDED is a final state, and the TSP should not post back; they should ignore all superseded.


�SUPERSEDED is a final state, and the TSP should not post back; they should ignore all superseded.


�SUPERSEDED is a final state, and the TSP should not post back; they should ignore all superseded.


�SUPERSEDED is a final state, and the TSP should not post back; they should ignore all superseded.


�SUPERSEDED is a final state, and the TSP should not post back; they should ignore all superseded.


�SUPERSEDED is a final state, and the TSP should not post back; they should ignore all superseded.


�SUPERSEDED is a final state, and the TSP should not post back; they should ignore all superseded.


�SUPERSEDED is a final state, and the TSP should not post back; they should ignore all superseded.


�SUPERSEDED is a final state, and the TSP should not post back; they should ignore all superseded.


�SUPERSEDED is a final state, and the TSP should not post back; they should ignore all superseded.


�WITHDRAWN is a final state, and the TSP should not post back; they should ignore all WITHDRAWN.


�WITHDRAWN is a final state, and the TSP should not post back; they should ignore all WITHDRAWN.


�WITHDRAWN is a final state, and the TSP should not post back; they should ignore all WITHDRAWN.


�WITHDRAWN is a final state, and the TSP should not post back; they should ignore all WITHDRAWN.


�WITHDRAWN is a final state, and the TSP should not post back; they should ignore all WITHDRAWN.


�WITHDRAWN is a final state, and the TSP should not post back; they should ignore all WITHDRAWN.


�WITHDRAWN is a final state, and the TSP should not post back; they should ignore all WITHDRAWN.


�WITHDRAWN is a final state, and the TSP should not post back; they should ignore all WITHDRAWN.


�WITHDRAWN is a final state, and the TSP should not post back; they should ignore all WITHDRAWN.


�WITHDRAWN is a final state, and the TSP should not post back; they should ignore all WITHDRAWN.


�Already incorporated into NERC standard


�Already incorporated into NERC standard





WEQ ESS/ITS/BPS  03/11/2008
Page 1 of 10

