

[image: image1.wmf] 


RECOMMENDATION TO NAESB EXECUTIVE COMMITTEE

                                       For Quadrant: 
WEQ

                                       Requesters: 
ESS/ITS/BPS Subcommittees



                                       Request No.: 
2008AP 2.b.ii.2

                                       Request Title:
Postbacks and Counterflows Business

               
Practice Standards 
MIDWEST ISO COMMENTS

1.  RECOMMENDED ACTION:
EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

  X  Accept as requested


      Change to Existing Practice

      Accept as modified below


      Status Quo

      Decline

2.  TYPE OF DEVELOPMENT/MAINTENANCE

Per Request:




Per Recommendation:
   X  Initiation




   X  Initiation 

      Modification




  _   Modification

      Interpretation



      Interpretation

      Withdrawal




      Withdrawal

      Principle 




      Principle 

      Definition 




      Definition 

      Business Practice Standard 


  X  Business Practice Standard 

      Document 




      Document 

      Data Element 



      Data Element

      Code Value 




      Code Value 

      X12 Implementation Guide


     _X12 Implementation Guide

      Business Process Documentation

      Business Process Documentation

3.  RECOMMENDATION

SUMMARY:


The WEQ 2008 Annual Plan Item Item 2.b.ii states, “Business practice standards for accounting for counterflows.  These standards will be included in the ATC business practice standards.”  As a part of this annual plan item, postbacks have also been included per discussions between the NERC and NAESB during the Joint NERC/NAESB AFC/ATC meetings.

The Joint ESS/ITS and BPS has determined that no additional work is required for accounting for counterflows based upon the NERC MOD-001 and the letter from Rae McQuade to Jerry Adamski and Andy Rodriquez dated May 1, 2008.  The NERC MOD standard states:
R3. Each Transmission Service Provider shall prepare and keep current an Available Transfer Capability Implementation Document (ATCID) that includes, at a minimum, the following information: [Violation Risk Factor: Lower] [Time Horizon: Operations Planning] 
R3.1. Information describing how the selected methodology (or methodologies) has been implemented, in such detail that, given the same information used by the Transmission Service Provider, the results of the ATC or AFC calculations can be validated. 

R3.2. A description of the manner in which the Transmission Service Provider will account for counterflows including: 

R3.2.1. How confirmed Transmission reservations, expected Interchange and internal counterflow are addressed in firm and non-firm ATC or AFC calculations. 

R3.2.2. A rationale for the defined accounting. 

Since accounting for counterflows is documented in the ATCID and the Joint ESS/ITS and BPS has submitted the recommendation 2008 AP Item 2.b.vii (WEQ-001 changes for “ATC Information Link” on OASIS and TTC and ATC methodologies and values), which requires posting the ATCID on the ATC Information Link webpage, the subcommittee believes it has addressed the requirement for counterflows through previous the previous recommendation.  Therefore the remainder of this recommendation is limited in scope to postbacks.
Postbacks: In FERC Order 890 under Docket numbers RM05-17 and RM05-25, the Commission made conclusions regarding postbacks and counterflows in paragraph 293.  
The ESS/ITS/BPS has reviewed Orders 890 and 890-A and recommends adoption of the following standards.  The Subcommittee feels this recommendation complies with the guidance provided by the Commission. 

The following recommended Standards address these determinations.
Recommended Standards:
Modifications to WEQ-001 (New Requirements)
Business Practices for Open Access Same-Time Information Systems (OASIS)

NOTE: NAESB Staff will assign appropriate enumeration to this standard to replace “n” with next sequential standard number within WEQ-001
001-0.n
Postback 
A component of the Transmission Provider’s selected ATC or AFC calculation methodology that positively impacts ATC or AFC based on a change in status of a 
reservation or unscheduled reservation.
001-n
REQUIREMENTS FOR POSTBACKS
The Transmission Provider shall have the right to postback on OASIS the capacity due to a change in status of transmission reservations or unscheduled firm, as applicable, according to the following standards. 

001-n.1
Determination of Postback

The Transmission Provider shall postback reservation capacity or unscheduled transmission service that meets any of the following criteria: 
001-n.1.1
If there is a change in status or capacity (e.g. REDIRECT, ANNUAL) of a transmission reservation a postback is needed in the calculation of firm and non-firm ATC.
001-n.1.2
If firm reservations are included in ETC for use in the non-firm ATC calculation a postback is needed for unscheduled firm transmission service allcoation in the calculation of non-firm ATC.
001-n.1.3
If scheduled firm tranmission service is included in ETC for use in the non-firm ATC calculation a postback is not needed in the calculation of non-firm ATC.




001-n.2
Postback methodology 
001-n.2.1
If the Transmission Provider incorporates a business practice for using an adjustment to postback for losses then this method of adjusting must be captured in the TP’s Business Practice or the ATCID and used on a non-discriminatory basis.
001-n.2.2
If theTransmission Provider incorporates a business practice by using the energy profile instead of the transmission allocation that expands the non-firm ATC offering then this method of adjusting must be captured in the TP’s Business Practice or the ATCID and used on a non-discriminatory basis.
001-n.2.3
The application of postback can reflect differences related to the NERC methodology used by the TP. The net effect on the ATC calculated values should be the same. The method of adjusting must be captured in the TP’s Business Practice or the ATCID and used on a non-discriminatory basis.

001-n.2.4
The postback methodology must be documented in the TP’s ATCID. 
001-n.3





	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	

	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	

	

	
	
	
	
	

	
	
	
	
	



001-nn
APPENDIX nn – POSTBACK STANDARDS EXAMPLES


Example 1: Examples of the application of  different Postback categories and its effect on 
ATCNF depending on the way transmission service is applied during the scheduling 
horizon:

If you have 100 MW of firm reservations, and 60 MW were scheduled (40 MW unscheduled) on that 100 MW, during the scheduling horizon:

ATCF = TTC – 100(ETC Firm) – CBM – TRM (prior to the scheduling horizon)

Postback as applied in WEQ 001-n.1.2:   ATCNF = TTC – 100(ETC Firm) – CBM – TRM + 40(PostBack)

Postback as applied in WEQ 001-n.1.3:   ATCNF = TTC – 60(ETC Firm) – CBM – TRM + 0 (PostBack)












Modifications to WEQ-002 (Revisions noted with redlines)

Business Practices for Open Access Same-Time Information Systems (OASIS) Standards & Communication Protocols
There are no revisions needed for WEQ-002
Modifications to WEQ-003 (Revisions noted with redlines)

OASIS Data Dictionary
There are no revisions needed for WEQ-003

Modifications to WEQ-013 (Revisions noted with redlines)

Business Practices for Open Access Same-Time Information Systems (OASIS) Implementation Guide
There are no revisions needed for WEQ-013
4.  SUPPORTING DOCUMENTATION
a.  Description of Request:

In the FERC Order 890 under Docket Nos. RM05-17-000 and RM05-25-000 dated February 16, 2007, the Commission made the following conclusions about the requirement for postbacks and counterflow:
Paragraph 293 - With regard to EPSA’s request for the standardization of additional data input, we believe they are already captured in the Commission’s proposal as adopted in this Final Rule. Xcel asks the Commission to require consistency in the determination of counterflows in the calculation of ATC. Counterflows are included in the list of assumptions that public utilities, working through NERC, are required to make consistent. We believe that counterflows, if treated inconsistently, can adversely affect reliability and competition, depending on how they are accounted for. Accordingly, we reiterate that public utilities, working through NERC and NAESB, are directed to develop an approach for accounting for counterflows, in the relevant ATC standards and business practices. We find unnecessary Xcel’s request that we require a date certain for specific issues in the Western Interconnection to be addressed. Above we require public utilities, working through NERC, to modify the ATC standards within 270 days after the publication of the Final Rule in the Federal Register.

b.  Description of Recommendation:

c.  Business Purpose:

Implementation of FERC Orders 890 and 890-A.
d. Commentary/Rationale of Subcommittee(s)/Task Force(s):

Please review the following Joint BPS and ESS/ITS ATC/AFC Subcommittee meeting minutes:

· April 16-17, 2008

· NEED to add references to NERC/NAESB meetings where postbacks were discussed.

Please review the Motions Document:  http://naesb.org/pdf3/weq_atc_afc031008a2.doc
�Why limit postback to confirmed reservation only?  If there is a change to any reservation that positively affects ATC, then that additonal ATC should be posted back.


�There is nothing to postback, the Non-Firm ETC decreases only. 


�This table seems “all inclusive”, which is too hard to maintain on a regular basis.  Plus, requirement n.3.2 and elsewhere direct the TP to specify any other postbacks they may use in their ATCID.





3
April 21, 2008
Page 1

[image: image2.wmf] 

_1075620371.doc
[image: image1.png]|

Lo







