

[image: image1.wmf] 


RECOMMENDATION TO NAESB EXECUTIVE COMMITTEE

                                       For Quadrant: 
WEQ

                                       Requesters: 
ESS/ITS/BPS Subcommittees



                                       Request No.: 
2008AP 2.b.ii.2

                                       Request Title:
Postbacks and Counterflows Business

               
Practice Standards 

The following are comments from OATI regarding the proposed recommendation for Postbacks and Counterflows.  Note that most comments on the proposal are tracked as MS Word “balloon comments”. 
1.  RECOMMENDED ACTION:
EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

  X  Accept as requested


      Change to Existing Practice

      Accept as modified below


      Status Quo

      Decline

2.  TYPE OF DEVELOPMENT/MAINTENANCE

Per Request:




Per Recommendation:
   X  Initiation




   X  Initiation 

      Modification




  _   Modification

      Interpretation



      Interpretation

      Withdrawal




      Withdrawal

      Principle 




      Principle 

      Definition 




      Definition 

      Business Practice Standard 


  X  Business Practice Standard 

      Document 




      Document 

      Data Element 



      Data Element

      Code Value 




      Code Value 

      X12 Implementation Guide


     _X12 Implementation Guide

      Business Process Documentation

      Business Process Documentation

3.  RECOMMENDATION

SUMMARY:


Postbacks and Counterflows: In FERC Order 890 under Docket numbers RM05-17 and RM05-25, the Commission made conclusions regarding postbacks and counterflows in paragraph 293.  
The ESS/ITS/BPS has reviewed Orders 890 and 890-A and recommends adoption of the following standards.  The Subcommittee feels this recommendation complies with the guidance provided by the Commission. 

 The following recommended Standards address these determinations.
Recommended Standards:
Modifications to WEQ-001 (New Requirements)
Business Practices for Open Access Same-Time Information Systems (OASIS)
Business Practice Standards for PostBacks
NOTE: NAESB Staff will assign appropriate enumeration to this standard to replace “n” with next sequential standard number within WEQ-001
001-0.n
Postback 
A variable component of the Transmission Provider’s selected ATC or AFC calculation methodology that positively impacts ATC or AFC based on a change in status of a transmission service reservation, unscheduled use of reserved capacity, or other conditions as specified in the Transmission Provider’s ATC methodology. 

001-n
REQUIREMENTS FOR DEALING WITH POSTBACKS AND COUNTERFLOWS
The Transmission Provider shall have the right incorporate Postbacks in the Available Transfer Capability (ATC) posted on OASIS due to a change in status of transmission reservations or unscheduled firm transmission
 service, as applicable, according to the following standards. 

001-n.1
Determination of Postback

The Transmission Provider shall postback changes in reserved capacity or unscheduled transmission service that meets any of the following criteria: 
001-n.1.1
If there is a change in status or other modification to the reserved capacity associated with a transmission service reservation, any capacity made available on the Transmission Provider’s facilities shall be incorporated as a postback in the calculation of firm and non-firm ATC.  Such conditions include:
· Annullment of the reservation

· Redirect of transmission service on a firm basis

· Recall of transmission capacity
001-n.1.2
If all firm reserved capacity is included in the Existing Transmission Commitments (ETC) component for use in the non-firm ATC calculation, the Transmission Provider shall incorporate a postback of unscheduled firm reserved capacity in the calculation of non-firm ATC.
001-n.1.2.1
If scheduled firm reserved capacity is included in ETC for use in the non-firm ATC calculation a postback is not needed in the calculation of non-firm ATC.
001-n.2
The incorporation of any additional conditions to be applied as Postbacks to either firm and non-firm ATC calculation methodology must be documented in the Transmission Provider’s ATCID as posted on OASIS.

Recommend that this be the end of the standard.  Methodology is dealt with in n.2, and don’t include the Table unless TP is required to include all the items or the group is comfortable that the table is the  ONLY conditions that can be termed a postback.  
The table seems more of a recommendation on the types of conditions that TP could elect to consider as a postback and include in ATCID.  Would such a recommendation be appropriate to include in a standard?


001-n.2
Postback methodology 
001-n.2.1
If the Transmission Provider incorporates a business practice for using an adjustment to postback for real transmission losses, then this method of adjusting ATC must be documented in the TP’s Business Practice or the ATCID and incorporated into ATC on a non-discriminatory basis.

001-n.2.2
If theTransmission Provider incorporates a business practice that expands the non-firm ATC offering due to unscheduled firm reserved capacity by using the energy profile instead of the transmission allocation specified in a Request for Interchange, then this method of adjusting ATC must be captured in the TP’s Business Practice or the ATCID and incorporated into ATC on a non-discriminatory basis.
001-n.2.3
The application of postback can reflect differences related to the NERC methodology used by the TP. The net effect on the ATC calculated values should be the same. 
The method of adjusting must be captured in the TP’s Business Practice or the ATCID and used on a non-discriminatory basis.

001-n.2.4
The firm and non-firm postback calculation methodology must be documented in the TP’s ATCID. 
001-n.3
Postback conditions for use in ATC calculation
001-n.3.1
Table xx identifies a list of items with conditions for use in the calculation of ATC, unless otherwise accounted for in ETC, as defined by the TP’s ATCID. 




Table xx

Postback Table of Items
	POSTBACK
	ACTION
	WEQ 001-N.1 Determination For FATC
	WEQ 001-N.1 Determination For NFATC
	NAESB BP

	1. ANNULLED, DISPLACED, recalled Firm reservation posts back to F ATC and may also post back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being annulled.
	ANNULLED, DISPLACED, recall
	WEQ 001-n.1.1
	WEQ 001-n.1.1 or during scheduling    WEQ 001-n.1.2, WEQ 001-n.1.3 if scheduled
	ANNULLED and DISPLACED are defined states. Displacement is defined in WEQ 013-2.4 Recalls is defined in WEQ 013-2.5.

	2. ANNULLED, DISPLACED, recalled Non-Firm reservation posts back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being ANNULLED
	ANNULLED, DISPLACED, recall
	N/A
	WEQ 001-n.1.1 or during scheduling    WEQ 001-n.1.4, WEQ 001-n.1.5 if scheduled
	ANNULLED and DISPLACED are defined states. Displacement is defined in WEQ 013-2.4 Recalls is defined in WEQ 013-2.5.

	3. Firm Redirect of a Firm PTP causes post back to FATC on the path of the Parent reservations’ path for the time interval of the Firm Redirect and may also post back to NF ATC depending on the horizon.  (Note:  Non-Firm Redirect of a Firm reservation does not impact the Firm ATC on the Parent Reservation’s path.)
	REDIRECT
	WEQ 001-n.1.1
	WEQ 001-n.1.1 if prior to the scheduling horizon
	REDIRECT is a defined request type. Firm redirects is defined in WEQ 001-9 and Non-firm redirects is defined in WEQ 001-010.

	4. Firm Redirect of a Firm PTP may cause post back to FATC on the path of the Parent reservations’ path for rollover rights that were set-aside in the Firm ATC calculation.  This is dependent on the term of the Firm Redirect.
	REDIRECT
	WEQ 001-n.1.1
	WEQ 001-n.1.1
	Rollover (renewal) of Redirects is in WEQ 001-9.7

	5. Downward adjustments and terminations of Firm Network schedule (7FN) may post back to NF ATC depending on the horizon.
	Release of Unscheduled Firm Network to NF
	N/A
	WEQ 001-n.1.2, WEQ 001-n.1.3 in Scheduling Horizon
	ESS/ITS is currently working on NITS-related BPS.

	6. Downward adjustments and terminations of Firm PTP schedules (7F) may post back to NF ATC depending on the horizon.
	Release of Unscheduled Firm PtP to NF
	N/A
	WEQ 001-n.1.2, WEQ 001-n.1.3 in Scheduling Horizon
	Do not believe that release of unscheduled capacity is addressed in any NAESB BPS.

	7. Downward adjustments and terminations of Non-Firm PTP schedules (NERC priority 2NH-5NM), Secondary Network schedules (6NN), Secondary PTP schedules (1NS)   may post back to NF ATC depending on the horizon.
	Release of Unscheduled Non-Firm to NF
	N/A
	N/A

 Except when TPs ATCID specifies, then WEQ 001-n.1.4, WEQ 001-n.1.5 in Scheduling Horizon
	Do not believe that release of unscheduled capacity is addressed in any NAESB BPS.

	8. Unscheduled Firm Network reservations posts back to NF ATC.
	Release of Unscheduled Firm Network to NF
	N/A
	WEQ 001-n.1.2, WEQ 001-n.1.3 in Scheduling Horizon
	Do not believe that release of unscheduled capacity is addressed in any NAESB BPS. ESS/ITS is currently working on NITS-related BPS.

	9. Unscheduled Firm PTP reservations posts back to NF ATC.
	Release of Unscheduled Firm PtP to NF
	N/A
	WEQ 001-n.1.2, WEQ 001-n.1.3 in Scheduling Horizon
	Do not believe that release of unscheduled capacity is addressed in any NAESB BPS.

	10. Unscheduled Non-Firm PTP reservations (including Secondary PTP) posts back to NF ATC.
	Release of Unscheduled Non-Firm PtP Reservation to NF
	N/A
	N/A Except when TPs ATCID specifies then WEQ 001-n.1.4, WEQ 001-n.1.5 in Scheduling Horizon
	Do not believe that release of unscheduled capacity is addressed in any NAESB BPS.

	11. Unscheduled Secondary Network reservations (Non-Firm Network – as info, this is not a redirect of a Firm Network) posts back to NF ATC.
	Release of Unscheduled Secondary Network Reservation to NF
	N/A
	N/A Except when TPs ATCID specifies then WEQ 001-n.1.4, WEQ 001-n.1.5 in Scheduling Horizon
	Do not believe that release of unscheduled capacity is addressed in any NAESB BPS. ESS/ITS is currently working on NITS-related BPS.

	12. Termination of a DNR that then results in the termination/ reduction of a firm transmission reservation posts back to F ATC and may also post back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being annulled.
	Termination of DNR
	WEQ 001-n.1.1
	WEQ 001-n.1.1 or during Scheduling

WEQ 001-n.1.2, WEQ 001-n.1.3 if scheduled
	ESS/ITS is currently working on NITS-related BPS.

	13. Terminated Secondary Network reservation posts back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being annulled.
	Termination of Secondary Network
	N/A
	WEQ 001-n.1.4, WEQ 001-n.1.5 in Scheduling Horizon for some TPs
	ESS/ITS is currently working on NITS-related BPS.

	14. Relinquish secondary non-firm PtP capacity
	RELINQUISH
	N/A
	WEQ 001-n.1.1 if prior to Scheduling horizon; N/A during scheduling horizon, unless TP’s ATCID allows during scheduling then WEQ 001-n.1.4, WEQ 001-n.1.5
	RELINQUISH is a defined request type. Relinquish is defined in WEQ 001-10.5.


001-n.3.2
A TP can include additional item(s) not called out as a postback item provided the TP includes a description of such additional item in the TP’s ATCID. 

001-nn
APPENDIX nn – POSTBACK STANDARDS EXAMPLES


Example 1: Examples of the application of  different Postback categories and its effect on 
ATCNF depending on the way transmission service is applied during the scheduling 
horizon:

If you have 100 MW of firm reservations, and 60 MW were scheduled (40 MW unscheduled) on that 100 MW, during the scheduling horizon:

ATCF = TTC – 100(ETC Firm) – CBM – TRM (prior to the scheduling horizon)

Postback as applied in WEQ 001-n.1.2:   ATCNF = TTC – 100(ETC Firm) – CBM – TRM + 40(PostBack)

Postback as applied in WEQ 001-n.1.3:   ATCNF = TTC – 60(ETC Firm) – CBM – TRM + 0 (PostBack)

Modifications to WEQ-001 (New Requirements)
Business Practice Standards for Counterflows

NOTE: NAESB Staff will assign appropriate enumeration to this standard to replace “m” with next sequential standard number within WEQ-001
001-0.m
Counterflows 


Definition here
001-m
REQUIREMENTS FOR DEALING WITH Counterflows
The Transmission Provider or applicable RTO/ISO shall have the right to post counterflows on OASIS, as applicable, according to the following standards. 

001-m.1
Determination of counterflows

001-m.2
Counterflow methodology 
001-m.3
Counterflow conditions for use in ATC calculation
001-mm
APPENDIX mm – COUNTERFLOW STANDARDS EXAMPLES

Modifications to WEQ-002 (Revisions noted with redlines)

Business Practices for Open Access Same-Time Information Systems (OASIS) Standards & Communication Protocols
There are no revisions needed for WEQ-002
Modifications to WEQ-003 (Revisions noted with redlines)

OASIS Data Dictionary
There are no revisions needed for WEQ-003

Modifications to WEQ-013 (Revisions noted with redlines)

Business Practices for Open Access Same-Time Information Systems (OASIS) Implementation Guide
There are no revisions needed for WEQ-013
4.  SUPPORTING DOCUMENTATION
a.  Description of Request:

In the FERC Order 890 under Docket Nos. RM05-17-000 and RM05-25-000 dated February 16, 2007, the Commission made the following conclusions about the requirement for postbacks and counterflow:
Paragraph 293 - With regard to EPSA’s request for the standardization of additional data input, we believe they are already captured in the Commission’s proposal as adopted in this Final Rule. Xcel asks the Commission to require consistency in the determination of counterflows in the calculation of ATC. Counterflows are included in the list of assumptions that public utilities, working through NERC, are required to make consistent. We believe that counterflows, if treated inconsistently, can adversely affect reliability and competition, depending on how they are accounted for. Accordingly, we reiterate that public utilities, working through NERC and NAESB, are directed to develop an approach for accounting for counterflows, in the relevant ATC standards and business practices. We find unnecessary Xcel’s request that we require a date certain for specific issues in the Western Interconnection to be addressed. Above we require public utilities, working through NERC, to modify the ATC standards within 270 days after the publication of the Final Rule in the Federal Register.

b.  Description of Recommendation:

c.  Business Purpose:

Implementation of FERC Orders 890 and 890-A.
d. Commentary/Rationale of Subcommittee(s)/Task Force(s):

Please review the following Joint BPS and ESS/ITS ATC/AFC Subcommittee meeting minutes:

· April 16-17, 2008

Please review the Motions Document:  http://naesb.org/pdf3/weq_atc_afc031008a2.doc

�Added this since there seems to be two specific cases of postback, and then anything else TP wants (e.g.,  losses, etc.)


�Do we really need this?  ISO/RTO is the TP.  If they aren’t, are they responsible for ATC?  Think not.   Since RTO/ISO requirements are identical to TP requirements in this case, there is no need to call them out separately.


�Per 890 A: 67. We also agree with TranServ that the Commission’s reference to releasing unused


(non-scheduled) transfer capability as non-firm ATC applies to unscheduled firm


transmission capability, since all unused non-firm capacity is deemed available to any


entity meeting the scheduling requirements. This does not alter the requirement that the


transmission provider offer all available capacity, firm or non-firm, as applicable,


consistent with our long-standing open access principles.


�Since Postback is a defined term as proposed, how can there be “other” types of postbacks that aren’t in the defintion?  Recommend  IF there are things beyond TSR status changes and unscheduled firm, we relax the definition and include this catchall.


�Unclear what direction the B PS is going here.  In one respect, you have a blanket statement requiring that incorporation of any postback be documented in the ATCID.  Yet, here is an explicit callout for losses.  I would think the generic requirement for documenting use of postbacks to be sufficient.


�What does this mean??  Is it really part of a standard or simply a statement


�Do we really want to include this table in the standard.  Are these the only conditions allowed for postback?  Must TP implement them all, or can pick and choose?





I think we limit the “shall” requirements to recognizing changes in TSR status and release of unscheduled firm as requirements.  All other postbacks fall under the catchall that they must be documented in the ATCID


�will add counterflow standards as we go here


�add all meeting minutes


�add hyperlink





3
April 21, 2008
Page 2

[image: image2.wmf] 

_1075620371.doc
[image: image1.png]|

Lo







