Draft Lists of Postbacks


 
The following actions impact both the posted ATC and the internal adjusted ATC (to recognize pending requests). It is recognized that a Provider’s use of this list will be conditional of the approved tariff of the Transmission Provider. Not all may be incorporated by every Transmission Provider.
Concept of counter flows being used as Postbacks needs to be addressed. 
Draft List of Postbacks for Internally Adjusted ATC and Posted ATC

	POSTBACK
	ACTION
	NAESB BP

	1. Annulled Firm reservation posts back to F ATC and may also post back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being annulled. 
	Annul
	

	2. Annulled Non-Firm reservation posts back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being annulled. 
	Annul
	

	3. Displaced Firm reservation posts back to F ATC and may also post back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being displaced. 
	Displace
	

	4. Displaced Non-Firm reservation posts back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being displaced. 
	Displace
	

	5. Firm Redirect of a Firm PTP causes post back to F ATC on the path of the Parent reservations’ path for the time interval of the Firm Redirect and may also post back to NF ATC depending on the horizon.  (Note:  Non-Firm Redirect of a Firm reservation does not impact the Firm ATC on the Parent Reservation’s path.) 
	Firm Redirect
	

	6. Firm Redirect of a Firm PTP may causes post back to F ATC on the path of the Parent reservations’ path for rollover rights that were set-aside in the Firm ATC calculation.  This is dependent on the term of the Firm Redirect (and details on how to handle have not yet been established as a NAESB BP). 
	Firm Redirect Rollover Rights
	

	7. Recalled Firm reservation posts back to F ATC and may also post back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being recalled. 
	Recall
	

	8. Recalled Non-Firm reservation posts back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being recalled. 
	Recall
	

	9. Downward adjustments and terminations of Firm Network Tags (7FN) may post back to NF ATC depending on the horizon. 
	Release of Unscheduled Reservation to NF
	

	10. Downward adjustments and terminations of Firm Network Tags (7FN) may post back to NF ATC depending on the horizon. 
	Release of Unscheduled Reservation to NF
	

	11. Downward adjustments and terminations of Firm PTP Tags (7F) may post back to NF ATC depending on the horizon. 
	Release of Unscheduled Reservation to NF
	

	12. Downward adjustments and terminations of Non-Firm PTP Tags (NERC priority 2NH-5NM) may post back to NF ATC depending on the horizon. 
	Release of Unscheduled Reservation to NF
	

	13. Downward adjustments and terminations of Secondary Network Tags (6NN) may post back to NF ATC depending on the horizon. 
	Release of Unscheduled Reservation to NF
	

	14. Downward adjustments and terminations of Secondary PTP Tags (1NS) may post back to NF ATC depending on the horizon. 
	Release of Unscheduled Reservation to NF
	

	15. Unscheduled Firm Network reservations posts back to NF ATC. 
	Release of Unscheduled Reservation to NF
	

	16. Unscheduled Firm PTP reservations posts back to NF ATC. 
	Release of Unscheduled Reservation to NF
	

	17. Unscheduled Non-Firm PTP reservations posts back to NF ATC. 
	Release of Unscheduled Reservation to NF
	

	18. Unscheduled Secondary Network reservations (Non-Firm Network – as info, this is not a redirect of a Firm Network) posts back to NF ATC. 
	Release of Unscheduled Reservation to NF
	

	19. Unscheduled Secondary PTP reservations (Non-Firm Redirect of a Firm PTP reservation) posts back to NF ATC. (If your rules require Secondary PTP reservations to decrement NF ATC.  As info, some companies may disagree that NF ATC should be decremented for Secondary PTP reservations.) 
	Release of Unscheduled Reservation to NF
	

	20. Terminated Firm Network reservation posts back to F ATC and may also post back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being annulled. 
	Termination of DNR
	

	21. Terminated Secondary Network reservation posts back to NF ATC depending on the horizon and whether the reservation was scheduled prior to being annulled. 
	Termination of Secondary Network
	


 
 
The following actions impact only the internal adjusted ATC (to recognize pending requests). It is recognized that a Provider’s use of this list will be conditional of the approved tariff of the Transmission Provider. Not all may be incorporated by every Transmission Provider.
Draft List of Postbacks for Internally Adjusted ATC Only

	POSTBACK
	ACTION
	NAESB BP

	1. Counteroffer with lower MW of a Firm Network request that has been Accepted by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Counteroffer
	

	2. Counteroffer with lower MW of a Firm Network request that has been Counteroffered by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Counteroffer
	

	3. Counteroffer with lower MW of a Firm PTP request that has been Accepted by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Counteroffer
	

	4. Counteroffer with lower MW of a Firm PTP request that has been Counteroffered by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Counteroffer
	

	5. Counteroffer with lower MW of a Non-Firm PTP request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 
	Counteroffer
	

	6. Counteroffer with lower MW of a Non-Firm PTP request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 
	Counteroffer
	

	7. Counteroffer with lower MW of a Secondary Network request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 
	Counteroffer
	

	8. Counteroffer with lower MW of a Secondary Network request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 
	Counteroffer
	

	9. Counteroffer with lower MW of a Secondary PTP request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 
	Counteroffer
	

	10. Counteroffer with lower MW of a Secondary PTP request that has been Counteroffered by the TP 
	Counteroffer
	

	11. Retracted Firm Network request that has been Accepted by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Retract
	

	12. Retracted Firm Network request that has been Counteroffered by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Retract
	

	13. Retracted Firm PTP request that has been Accepted by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Retract
	

	14. Retracted Firm PTP request that has been Counteroffered by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Retract
	

	15. Retracted Non-Firm PTP request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 
	Retract
	

	16. Retracted Non-Firm PTP request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 
	Retract
	

	17. Retracted Secondary Network request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 
	Retract
	

	18. Retracted Secondary Network request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 
	Retract
	

	19. Retracted Secondary PTP request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 
	Retract
	

	20. Retracted Secondary PTP request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 
	Retract
	

	21. Superseded Firm Network request that has been Accepted by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Supersede
	

	22. Superseded Firm Network request that has been Counteroffered by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Supersede
	

	23. Superseded Firm PTP request that has been Accepted by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Supersede
	

	24. Superseded Firm PTP request that has been Counteroffered by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Supersede
	

	25. Superseded Non-Firm PTP request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 
	Supersede
	

	26. Superseded Non-Firm PTP request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 
	Supersede
	

	27. Superseded Secondary Network request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 
	Supersede
	

	28. Superseded Secondary Network request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 
	Supersede
	

	29. Superseded Secondary PTP request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 
	Supersede
	

	30. Superseded Secondary PTP request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 
	Supersede
	

	31. Withdrawn Firm Network request that has been Accepted by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Withdraw
	

	32. Withdrawn Firm Network request that has been Counteroffered by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Withdraw
	

	33. Withdrawn Firm PTP request that has been Accepted by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Withdraw
	

	34. Withdrawn Firm PTP request that has been Counteroffered by the TP posts back to F ATC and may also post back to NF ATC depending on the horizon. 
	Withdraw
	

	35. Withdrawn Non-Firm PTP request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 
	Withdraw
	

	36. Withdrawn Non-Firm PTP request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 
	Withdraw
	

	37. Withdrawn Secondary Network request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 
	Withdraw
	

	38. Withdrawn Secondary Network request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 
	Withdraw
	

	39. Withdrawn Secondary PTP request that has been Accepted by the TP may post back to NF ATC depending on the horizon. 
	Withdraw
	

	40. Withdrawn Secondary PTP request that has been Counteroffered by the TP may post back to NF ATC depending on the horizon. 
	Withdraw
	

	41. Unused TRM firm and non-firm
	
	

	42. Unused CBM firm and non-firm
	
	


WEQ ESS/ITS/BPS  08/28/07

Page 1 of 10

