The following are Southern’s comments based on OATI’s comments on the Draft CBM Requirements dated 11/09/2007:
The concept of the changes should be made with the current template and model structures being keep at a minimum of disruptions. The following comments try to keep this in place. 
The data flow for the changes below can be conceptual as follows:

1) The LSE makes a reservation request with the TS_CLASS = CBM this will give the reservation the individualism that it needs to be apart from all other reservations so that these can be excluded from  the ATC calculations.
2) The TP confirms the request. With this confirmation the CBM data in the systemdata template can be altered to reflect the aggregate of the requests. The changes in this template can be audited with the systemdataaudit template. This is smoother than trying to alter an existing CBM aggregate reservation.
3) The LSE can then tag against his existing reservation with the tagging validation changed to go against the CBM data total. 

4) This concept is smoother and will cause less disruptions than trying to make exceptions to the reservation system. 

This are the following changes to the standard:
1.1 - The addition on a TS_CLASS of CBM would be less disruptive than an addition of REQUEST_TYPE.
1.3 – I think that leaving the attribute to long term transmission service request is sufficient. 
1.4 – This needs to be dropped from this standard because it is already specified in NERC MOD-004-R3.1.
1,7 – Renewal rights are being conveyed indirectly in that the LSE can change and update the CBM requirement on an monthly basis for the next ten years. There are provisions in the Order to allow the LSE to hold rights for CBM forever.

1.10 If the concept of allowing the LSE to schedule against the CBM request then this section is not valid.

2.2 Within the confines of the existing templates and models the total set aside for CBM can be set on the systemdata template.

Posted for informal comment:

Below is a conceptual draft of the OASIS mechanism to allow for auditing of CBM usage that was required in paragraph 262 of Order 890. Please provide informal comments on the concepts outlined in this draft to the BPS/ESS/ITS by 11/16/2007. Based upon comments received during this informal comment period, the BPS/ESS/ITS will develop the actual business practice standards to be posted for formal comment and Executive Committee vote. The posting for formal comment is intended to occur prior to the end of 2007.

Draft CBM Requirements
001-x  Capacity Benefit Margin (CBM) 
A Load S-erving Entity (LSE) within a Balancing Authority Area associated with a Transmission Service Provider shall have the right to request the Transmission Provider set aside transfer capability as CBM.
001-x.1: CBM Request 
001-x.1.1 The LSE shall submit a Transmission Service Request on OASIS designating the amount of CBM requested on a given transmission path using the unique TS_CLASS of CBM.


001-x.1.2 The Transmission Provider shall exclude all CBM reservations from its ATC calculations. 
001-x.1.3 The CBM request shall be treated the same as any other long-term firm transmission service request. 




001-x.1.5 The CBM request shall specify the transmission service attributes assigned by the Transmission Provider for the designation of long-term firm transmission service.
001-x.1.6 The CBM request capacity must be made in whole MW’s.


001-x.1.8 The LSE shall not have the right to resell rights acquired through a CBM request.
001-x.1.9 The LSE shall not have the right to redirect rights acquired through a CBM request. 

001-x.1.10 The LSE may request a revision or modification of its CBM reservation by submitting a new CBM request.  Upon confirmation, the new CBM reservation shall replace/displace the LSEs preceding CBM reservation.



001-x.2: Transmission Provider Allocation of CBM

001-x.2.1 The Transmission Provider shall evaluate all CBM requests submitted for a given transmission path, and allocate an amount of capacity to be set-aside as CBM in the Transmission Provider’s ATC calculations.

001-x.2.2 The Transmission Provider shall post on OASIS the total amount of CBM set-aside for all requesting LSEs on each transmission path over time as a SYSTEM_ATTRIBUTE = CBM on the systemddata template.

001-x.2.3 The OASIS ASSIGNMENT_REF assigned to the CBM reservation shall be referenced in all LSE requests to schedule use of CBM for the import of energy to meet a generation deficiency.

001-x.3:  Usage of CBM
001-x.3.1 Any LSE, including those LSEs that have not made a formal request for allocation of CBM, shall have the right to request use of CBM to meet a generation deficiency declared as an Emergency Energy Alert (EEA) level 2 or higher.
001-x.3.2 The LSE shall request to schedule energy deliveries using CBM in accordance with the WEQ-004 Coordinate Interchange standards, and shall reference the CBM Transmission Service Reservation in all such requests.

001-x.3.3 The Transmission Provider shall validate that the sum of all CBM imports scheduled on a given path will not exceed the CBM allocated for that path. 

001-x.3.3 As long as there is sufficient unscheduled CBM capacity available to support the requested schedule, the LSE shall not be restricted to use of CBM based on the capacity granted in their CBM reservation.

001-x.4:  Reallocation or Curtailment of CBM

001-x.4.1 If the sum of all requests to schedule use of CBM exceeds the capacity allocated in the CBM systemdata, the Transmission Provider shall first limit all requests or schedules for use of CBM to the LSE’s granted CBM reservation capacity.  Any CBM capacity that remains available beyond this limit shall be allocated to all requests and schedules for use of CBM on a pro rata basis.
001-x.4.2 In the event the Transmission Provider must curtail firm transmission service, the Transmission Provider shall curtail the total scheduled use of CBM on par with all other schedules for firm transmission service.  Individual LSE schedules for use of CBM shall be reallocated as specified in 001-x.4.1.
001-x.5: Audit of CBM Usage

001-x.5.1 The Transmission Provider shall post all scheduled use of CBM and any curtailments of those schedules such that they may be viewed and audited using the OASIS scheduledetail template.








�Alternative to CBM_REQUEST as a unique request type could be the specification of a unique service via service attributes.  Note that some providers use a designation of “long-term” rather than yearly, so may want to consider not being too prescriptive on this attribute.





