PJM has identified the following Reasons for 10% changes in ATC or ATC being zero : 

The ATC on path "XXX-YYY" has changed from 650.76 to 1657.00 - The limiting flowgate is "Arnold-Hazelton 345kV l/o Sherco3". The ATC changed due to OASIS reservation impacts, forecasted loads, and outages. Reservations can be queried on OASIS. Load and outage changes can be viewed at the links as posted to ATC Information on the OASIS homepage. Generation Outages are considered market sensitive and are confidential.
ATC on path "XXX-YYY" has changed from 650.76 to 1657.00-The limiting flowgate is "Arnold-Hazelton 345kV l/o Sherco3". The ATC changed due to no additional allocation ASTFC available from JOA Entities. ASFTC- Available Share Flowgate Transfer Capability.
ATC can be zero for the following reasons: 

1. OASIS reservation impacts 
2. Forecasted loads 
3. Transmission outages 
4. Generation Outages 
5. No additional allocation ASTFC available from JOA Entities 

Discussion: 

Determining which of the reasons 1-4 caused the ATC change or ATC being zero is hard job. Likely ALL four had some effect. A possible way to narrow it down is to run a sensitivity and post the reason with the largest combined effect. If you post changes to ATC on OASIS hourly then can the changes be for one time during the day, say after the day peak, post ATC narratives that will help customers determine actions for next day or post ATC narrative in morning to help customers decide on actions for current day.  The reason 5 can readily be determined nas the cause but is due in part to reasons 1-4.
Hope this sheds some light on the difficult issue. 

