Standard IRO-006-4 — Reliability Coordination — Transmission Loading Relief

A. Introduction

1. Title:
Reliability Coordination — Transmission Loading Relief (TLR)

2. Number:
IRO-006-5
3. Purpose:
To provide Interconnection-wide transmission loading relief procedures that can be used to prevent or manage potential or actual SOL and IROL violations to maintain reliability of the bulk electric system.  


4. Applicability:
4.1. Reliability Coordinators.

4.2. 
4.3. 
5. Proposed Effective Date:
Effective first day of first quarter after BOT adoption.
B. Requirements

R1. A Reliability Coordinator experiencing a potential or actual SOL or IROL violation within its Reliability Coordinator Area shall, with its authority and at its discretion, implement one or more procedures to provide transmission loading relief.  These procedures can be a “local” (regional, interregional, or sub-regional) transmission loading relief procedure or one of the following Interconnection-wide procedures: [Violation Risk Factor: Medium] [Time Horizon: Real-time Operations] 

· The Interconnection-wide Transmission Loading Relief (TLR) procedure for use in the Eastern Interconnection provided in IRO-006-5-EI.   
· The Interconnection-wide transmission loading relief procedure for use in the Western Interconnection isWECC-IRO-STD-006-0 provided at: ftp://www.nerc.com/pub/sys/all_updl/standards/rrs/IRO-STD-006-0_17Jan07.pdf.
· The Interconnection-wide transmission loading relief procedure for use in ERCOT is provided as Section 7 of the ERCOT Protocols, posted at: 
http://www.ercot.com/mktrules/protocols/current.html
R2. 
R3. 
R4. Reliability Coordinators receiving requests from a Reliability Coordinator in another Interconnection to curtail or reload Interchange Transactions that cross an Interconnection boundary shall comply with those requests.  [Violation Risk Factor: Medium] [Time Horizon: Real-time Operations]
R5. 
C. Measures

	Requirement
	Measure

	R1
	M1
	Each Reliability Coordinator shall be capable of providing evidence (such as logs) that when an actual or potential SOL or IROL violation occurred, it selected and implemented one or more regional, interregional, sub-regional, or Interconnection-wide transmission loading relief procedures.

	R2
	M2
	Each responding Reliability Coordinator shall be capable of providing evidence (such as logs) that when requested to curtail an Interchange Transaction that crossed an Interconnection boundary, the responding Reliability Coordinator complied with the request from the initiating Reliability Coordinator


M1. 

M2. 
M3. 
M4. 
D. Compliance

6. Compliance Monitoring Process

6.1. Compliance Monitoring Responsibility

Regional Entity.

6.2. Compliance Monitoring Period and Reset Time Frame

Compliance Monitoring Period: One calendar year.

Reset Period: One calendar month.

6.3. Data Retention

The Reliability Coordinator shall maintain evidence for eighteen months for M1, and M2.
	Measure
	Entity
	Data Retention Period

	M1
	Reliability Coordinator
	Eighteen months

	M2
	Reliability Coordinator
	Eighteen months




6.4. Additional Compliance Information

Each Reliability Coordinator, Transmission Operator, and Balancing Authority shall demonstrate compliance through self-certification submitted to its Compliance Monitor annually and reporting by exception. The Compliance Monitor may also use scheduled on-site reviews every three years, and investigations upon complaint, to assess performance. 

Each Reliability Coordinator, Transmission Operator, and Balancing Authority shall have the following available for its Compliance Monitor to inspect during a scheduled, on-site review or within 5 days of a request as part of an investigation upon complaint: 

6.4.1 Operations logs, voice recordings or transcripts of voice recordings or other documentation providing the evidence of its compliance to all the requirements for all Interconnection-wide TLR procedures that it has implemented during the review period. 

6.4.2 TLR reports.

7. Violation Severity Levels

7.1. 
7.1.1 
7.1.2 
7.1.3  
7.2. 
7.2.1 
7.3. 
7.3.1 
7.4. 
7.4.1 
7.4.2 
7.4.3 
7.4.4 
7.4.5 
7.4.6 
	R #
	Lower VSL
	Moderate
	High VSL
	Severe VSL

	R1.
	N/A
	N/A
	N/A
	When experiencing an actual or potential SOL or IROL violation, the Transmission Operator did not select and implement one or more regional, interregional, sub-regional, or Interconnection-wide transmission loading relief procedures.

	R2.
	N/A
	N/A
	N/A
	When requested to curtail an Interchange Transaction that crosses an Interconnection boundary as part of an Interconnection-wide procedure, the responding Reliability Coordinator did not comply with the request from the initiating Reliability Coordinator


E. Regional Differences

None.

8. 
9. 









· 
· 
· 
· 




· 
· 
· 
· 
· 
· 



	
	
	

	
	
	

	
	

	

	
	
	

	
	
	





	
	
	

	
	
	

	
	
	

	
	
	



F. Associated Documents
Version History

	Version
	Date
	Action
	Change Tracking

	0
	April 1, 2005
	Effective Date
	New

	0
	August 8, 2005
	Removed “Proposed” from Effective Date
	Errata

	1
	August 8, 2005
	Revised Attachment 1
	Revision

	3
	February 26, 2007
	Revised Purpose and Attachment 1 related to NERC NAESB split of the TLR procedure
	Revision

	4
	
	Completed NERC/NAESB split
	Revision

	5
	
	Removed Attachment 1 and made into a new standard, eliminated unnecessary requirements.  
	Revision


This requirement simply states; the RC has the authority to act, the RC should know at what limits he/she needs to act, the RC has pre-identified regional, interregional and sub-regional TLR procedures.





Comment: see FERC Order 693 paragraph 964 regarding recommendation for using tools other than TLR to mitigate an actual IROL.





Note: the URL has changed. 





Comment: R5 will be reviewed during Phase 3 of the TLR drafting team work.  See white paper for explanation of the three phases of changes to this standard.





This section on Regional Differences is highlighted for transfer to NAESB following completion of the MISO/PJM/SPP field test as described in the white paper.

















Draft:

Page 1 of 4
Proposed Effective Date: Effective Upon BOT adoption.

