List of Phase III Changes
1.) Request for exemption from ramp requirements for TLR-cuts (CoC IRO-006-1, Q3 - OVEC).  Compliance with RC order is mandatory, so this is already covered.
2.) Request that curtailed transactions cannot be reloaded by anyone except the issuer of the curtailment.  (CoC IRO-006-1, Q5 - JISWG) This has been fixed in the IDC.  However, we will forward the following comment to the Interchange folks (Subc? DT?) “The TLRDT believes that there should be a requirement that transactions curtailed for reliability reasons by an RC should have some restrictions on who can reload them.” 
3.) Identify cases where denial of a curtailment may be allowed (CoC IRO-006-1, Q5 - JISWG).  Compliance with RC order is mandatory, so this is already covered.  If results are questionable, RCs must coordinate actions.
4.) Identify cases (if any) where a curtailment may be allowed for non-reliability reasons (CoC IRO-006-1, Q5 - JISWG).  TLR cannot be used for non-reliability.  Curtailments that are not part of TLR should be covered in other standards or business practices. 
5.) Appropriate uses (if any) of “proxy” flowgates (CoC IRO-006-1, Q5 – AEP, OVEC).  There is another task group looking into this area, and we will make changes as necessary based on the results of their work.
6.) Ensure there are no “double jeopardy” situations with IRO-005 (CoC IRO-006-4,Q1-AEP).  No double jeopardy found.
7.) Review role of TOp in IRO-006 (CoC IRO-006-4, Q2)  Jim and Sue agree that the new standard may work.  
8.) Request to move the first sentence in 2.5.3 to NAESB (The issuance of a TLR Level 4 shall result in the curtailment, in the current hour and the next hour, of all Interchange Transactions using Non-firm Point-to-Point Transmission Service that are at or above the Curtailment Threshold that impact the Constrained Facilities.) (CoC IRO-006-4, Q3 – Duke)  If NAESB agrees, they will take this on in the next revision of their Business Practice.  With the planned major rewrite of the standard, these sections will end up being moved into the Joint Operator Manual.  Need to review after the final version of the Joint Operator Manual is done.  KATHY TO BRING BACK TO NAESB, AS THE NEXT VERSION OF TLR DOES NOT CONTAIN THIS LANGUAGE, AND NAESB STANDARD USES “BUILD UP” APPROACH.
9.) Request to keep 5.1.5 within the IRO-006 standard (All Balancing Authorities in the Eastern Interconnection shall be obligated to achieve the amount of Constrained Facility relief assigned to them by the Per Generator Method.) (CoC IRO-006-4, Q3 – Duke) With the planned major rewrite of the standard, these sections will end up being moved into the Joint Operator Manual.  Need to review after the final version of the Joint Operator Manual is done. IRO-001 REQUIRES THE BA TO DO WHAT THEY ARE DIRECTED TO BY THE RC, AND 4.1 ASSIGNS RELEIF OBLIGATION.
10.) Request to move sections 6.2 – 6.2.6 (from mapping document) to either NAESB or the IDC Ref Doc (CoC IRO-006-4, Q3 – Duke) With the planned major rewrite of the standard, these sections will end up being moved into the Joint Operator Manual.  Need to review after the final version of the Joint Operator Manual is done. THESE ARE IN THE IDC REF PORTION OF THE REF DOC.
11.) Request to move sections 7.4.1. – 7.4.3 (from mapping document) to NAESB (CoC IRO-006-4, Q3 – Duke) With the planned major rewrite of the standard, these sections will end up being moved into the Joint Operator Manual.  Need to review after the final version of the Joint Operator Manual is done. THESE ARE IN THE IDC REF PORTION OF THE REF DOC.
12.) Request to move sections 7.7 – 7.9 (from mapping document) to  NAESB or the IDC Ref Doc (CoC IRO-006-4, Q3 – Duke) With the planned major rewrite of the standard, these sections will end up being moved into the Joint Operator Manual.  Need to review after the final version of the Joint Operator Manual is done. THESE ARE IN THE IDC REF PORTION OF THE REF DOC.
13.) Attachment 1 - Section 2. Identify whether reference to “On-Path or Off-path” belongs in standard, attachment, reference document, or NAESB (CoC IRO-006-4, Q3 – Duke)  - Will be cleaned up in new version of Joint operator Manual.  Rewrite will identify where it belongs.  IS OWNED BY NAESB, BUT IS IN THE REFERENCE.
14.) Redraft of R5 and M5 (CoC IRO-006-4, Q6-Duke; q11 Entergy)  Has been deleted form the standard.  NAESB still needs to clean this up.
15.) Restructure of R3 (CoC IRO-006-4, QA6 Duke) R3 needs to be split into two requirements, one that focuses on implementing a local procedure simultaneously with the Interconnection-wide procedure and another that states specifically, “Each Reliability Coordinator shall follow the curtailments as directed by the Interconnection-wide procedure.” Has been restructured.  Part 1 is eliminated and Part 2 is explicit.  Needs team review, so revisit. REVIEWED AND ADDRESSED.
16.) Rewrite of R4 (CoC IRO-006-4, Q7-AEP). We will need to add specific coordination language for each interconnection. DEFER; REVISIT AFTER TOM FINDS OUT WHAT IS HAPPENING ON THE GROUND.
17.) Evaluate the need for TLR Level 6 (CoC IRO-006-4, Q7-AEP) Still under discussion by the team. Frank Koza bringing to RCWG for discussion. Frank confirmed minority concern at RCWG, we will ask industry in comment form.
18.) Restructure of R4, which deals with coordination across Interconnections (CoC IRO-006-4, Q7-ERCOT).  See 16.
19.) Rewrite 4.1 – 4.5 (CoC IRO-006-4, Q11-AEP)  Included now in Reference document.
20.) Update Appendix A diagram with correct terminology (CoC IRO-006-4, Q11-AEP) Will be in Joint manual, and will be updated. DONE
21.) IRO-006-4: The roles of the RC (as initiator or responder) are unclear and should be clarified. (CoC IRO-006-4, Q11-IRC) CLEARED UP.
22.) IRO-006-4, Attachment 1: Should be reviewed to determine whether there is any portion that should become part of a standard.  Attachment 1 largely is procedural in nature, but part(s) of it possibly should be rewritten in the form of a standard. (CoC IRO-006-4, Q11-IRC) DONE, but we may need to adjust the reference manual.
23.) IRO-006-4, Attachment 1: Some of the assumptions made by IDC are fairly crude and can result in the inappropriate selection of interchange transactions to be curtailed. (CoC IRO-006-4 Q11-IRC)  Specific concerns would need to be taken to IDCWG or RCWG or ORS
24.) IRO-006-4, Attachment 1: Should either specify requirements for IDC, or require after-the-fact analysis of IDC results upon request to identify and quantify deficiencies, or both. (CoC IRO-006-4, Q11-IRC) Specific concerns would need to be taken to IDCWG or RCWG or ORS
25.) Move timing guidelines in Attachment 1 to IDC Reference Document DONE.
26.) Move Attachment 1 Appendix E, F to IDC Reference Document DONE.
