|  | = Consistency between NAESB/NERC Definitions |  |
| --- | --- | --- |
|  | = Inconsistency between NAESB/NERC Definitions |  |
| xxxxx | = Specific differences |  |

| COMPARISON OF NAESB/NERC DEFINITIONS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Number | Description | Acronym | NAESB Definition | NERC Definition |
|  | “Cascading Outage” to NERC. On February 12, 2008, the NERC Board of Trustees withdrew its November 1, 2006 approval of that definition, without prejudice to the ongoing work of the FAC standards drafting team and the revised standards that are developed through the standards development process. Therefore, the definition is no longer in effect. |  |  | The uncontrolled successive loss of Bulk Electric System Facilities triggered by an incident (or condition) at any location resulting in the interruption of electric service that cannot be restrained from spreading beyond a predetermined area. |
| 005-0.1 | Ace Control Error | ACE | The instantaneous difference between net actual and scheduled interchange, taking into account the effects of frequency bias including a correction for meter error. |  |
|  | Adequacy |  |  | The ability of the electric system to supply the aggregate electrical demand and energy requirements of the end-use customers at all times, taking into account scheduled and reasonably expected unscheduled outages of system elements. |
|  | Adjacent Balancing Authority |  |  | A Balancing Authority Area that is interconnected another Balancing Authority Area either directly or via a multi-party agreement or transmission tariff. |
|  | Adverse Reliability Impact |  |  | The impact of an event that results in frequency-related instability; unplanned tripping of load or generation; or uncontrolled separation or cascading outages that affects a widespread area of the Interconnection. |
| 001-0.1 | Affiliate |  | (1) For any exempt wholesale generator, as defined under section 32(a) of the Public Utility Holding Company Act of 1935, as amended, the same as provided in section 214 of the Federal Power Act; and (2) For any other entity, the term affiliate has the same meaning as given in 18 CFR 161.2(a). |  |
|  | Agreement |  |  | A contract or arrangement, either written or verbal and sometimes enforceable by law. |
|  | Altitude Correction Factor |  |  | A multiplier applied to specify distances, with adjusts the distances to account for the change in relative air density (RAD) due to altitude from the RAD used to determine the specified distance. Altitude correction factors apply to both minimum worker approach distances and to minimum vegetation clearance distances. |
|  | Ancillary Service |  |  | Those services that are necessary to support the transmission of capacity and energy from resources to loads while maintaining reliable operation of the Transmission Service Provider’s transmission system in accordance with good utility practice. (From FERC order 888-A.) |
|  | Anti-Aliasing Filter |  |  | An analog filter installed at a metering point to remove the high frequency components of the signal over the AGC sample period. |
| 012-0.1 | Applicant |  | An authorized individual of a registered end entity that may submit applications for issuance of certificates to an Authorized Certification Authority for the end entity. |  |
| 001-0.15 | Appropriate Regulating Authority |  | The entity which has regulating authority over a given Transmission Provider. |  |
| 004-0.1 | Approval Entity |  | An entity that has approval rights for an Arranged Interchange; this includes the Transmission Service Providers (TSP), scheduling Balancing Authorities (BA), Generator-Providing Entity (GPE), and the Load-Serving Entity (LSE) that are included in the Arranged Interchange, as well as any Purchasing Selling Entity (PSE) whose transmission rights are cited (TPSE) in the Arranged Interchange. |  |
| 008-0.1 | Approval Entity |  | An entity that has approval rights for an Interchange Transaction Tag. This includes Transmission Service Providers (TSPs), Balancing Authorities (BAs), Purchasing-Selling Entities (PSEs), and Load Serving Entities (LSEs) involved in the Interchange Transaction. |  |
| 008-0.2 | Area Control Error | ACE | The instantaneous difference between a Balancing Authority’s net actual and scheduled interchange, taking into account the effects of Frequency Bias and correction for meter error. | The instantaneous difference between a Balancing Authority’s net actual and scheduled interchange, taking into account the effects of Frequency Bias and correction for meter error. |
| 007-0.1 | Area Control Error | ACE | The instantaneous difference between net actual and scheduled interchange, taking into account the effects of frequency bias, including a correction for meter error. |  |
| 004-0.2 | Arranged Interchange |  | The state where the Interchange Authority has received the Interchange information (initial or revised). | The state where the Interchange Authority has received the Interchange information (initial or revised). |
| 001-0.16 | Assignee |  | An Eligible Customer that receives point-to-point transmission service rights from a Reseller either through a Resale or a Transfer. |  |
| 012-0.2 | Authorized Certification Authority | Authorized CA | A Certification Authority that complies with the NAESB WEQ PKI Standards, has met all terms and conditions and executed all requirements as set forth by the NAESB Certification Program for the NAESB WEQ PKI Standards, and is registered in the Registry. |  |
| 008-0.3 | Automatic Generation Control | AGC | Equipment that automatically adjusts generation in a Balancing Authority Area from a central location to maintain the Balancing Authority’s interchange schedule plus Frequency Bias. AGC may also accommodate automatic inadvertent payback and time error correction. | Equipment that automatically adjusts generation in a Balancing Authority Area from a central location to maintain the Balancing Authority’s interchange schedule plus Frequency Bias. AGC may also accommodate automatic inadvertent payback and time error correction. |
|  | Available Transfer Capability | ATC |  | A measure of the transfer capability remaining in the physical transmission network for further commercial activity over and above already committed uses. It is defined as Total Transfer Capability less existing transmission commitments (including retail customer service), less a Capacity Benefit Margin, less a Transmission Reliability Margin. |
| 004-0.3 005-0.2 | Balancing Authority | BA | The responsible entity that integrates resource plans ahead of time, maintains load-interchange-generation balance within a Balancing Authority Area, and supports Interconnection frequency in real time. | The responsible entity that integrates resource plans ahead of time, maintains load-interchange-generation balance within a Balancing Authority Area, and supports Interconnection frequency in real time. |
| 006-0.1 007-0.2 008-0.4 | Balancing Authority | BA | The entity responsible for integrating resource plans ahead of time, maintaining load-interchange-generation balance within a Balancing Authority Area, and supporting Interconnection frequency in real time. |  |
| 011-0.3 | Balancing Authority | BA | Is the term used by the Wholesale Electric Quadrant to describe the entity responsible for integrating electric resource plans ahead of time, for maintaining electric load-interchange-generation balance within its metered boundaries, and for supporting electric interconnection frequency in real time. In certain circumstances, a BA may be a Regional Transmission Organization or Independent System Operator. This definition applies to NAESB WEQ Standard Nos. WEQ-011-1.5 and WEQ-011-1.6 and NAESB WGQ Standard No. 0.3.15. |  |
| 004-0.4 | Balancing Authority Area | BAA | The collection of generation, transmission, and loads within the metered boundaries of the Balancing Authority. The Balancing Authority maintains load-resource balance within this area. | The collection of generation, transmission, and loads within the metered boundaries of the Balancing Authority. The Balancing Authority maintains load-resource balance within this area. |
| 005-0.3 006-0.2 007-0.3 008-0.5 | Balancing Authority Area | BAA | An electrical system bounded by interconnection (tie-line) metering and telemetry, where the Balancing Authority controls (either directly or by contract) generation to maintain its Interchange Schedule with other Balancing Authority Areas and contributes to frequency regulation of the Interconnection. |  |
|  | Base Load |  |  | The minimum amount of electric power delivered or required over a given period at a constant rate. |
|  | Blackstart Capability Plan |  |  | A documented procedure for a generating unit or station to go from a shutdown condition to an operating condition delivering electric power without assistance from the electric system. This procedure is only a portion of an overall system restoration plan. |
| 008-0.6 | Bulk Electric System |  | The electrical generation resources, transmission lines, interconnections with neighboring systems, and associated equipment, generally operated at voltages of 100 kV or higher. Radial transmission facilities serving only load with one transmission source are generally not included in this definition. | As defined by the Regional Reliability Organization, the electrical generation resources, transmission lines, interconnections with neighboring systems, and associated equipment, generally operated at voltages of 100 kV or higher. Radial transmission facilities serving only load with one transmission source are generally not included in this definition. |
|  | Burden |  |  | Operation of the Bulk Electric System that violates or is expected to violate a System Operating Limit or Interconnection Reliability Operating Limit in the Interconnection, or that violates any other NERC, Regional Reliability Organization, or local operating reliability standards or criteria. |
| 013-0.2 | Business Practice |  | Refers collectively to any business practices adopted by the Primary Provider as defined in their Open Access Transmission Tariff (OATT), NAESB ratified Business Practice Standards, or provider specific practices or requirements. |  |
| 001-0.2 | Capacity Available to Redirect |  | The granted capacity of the Parent Reservation at the time of customer confirmation (CAPACITY_GRANTED) less all confirmed reassignments (e.g., resales), confirmed redirects on a firm basis, confirmed redirects on a non-firm basis, displacements, and approved schedules. |  |
|  | Capacity Benefit Margin | CBM |  | CBM The amount of firm transmission transfer capability preserved by the transmission provider for Load-Serving Entities (LSEs), whose loads are located on that Transmission Service Provider’s system, to enable access by the LSEs to generation from interconnected systems to meet generation reliability requirements. Preservation of CBM for an LSE allows that entity to reduce its installed generating capacity below that which may otherwise have been necessary without interconnections to meet its generation reliability requirements. The transmission transfer capability preserved as CBM is intended to be used by the LSE only in times of emergency generation deficiencies. |
|  | Capacity Emergency |  |  | A capacity emergency exists when a Balancing Authority Area’s operating capacity, plus firm purchases from other systems, to the extent available or limited by transfer capability, is inadequate to meet its demand plus its regulating requirements. |
|  | Cascading |  |  | The uncontrolled successive loss of system elements triggered by an incident at any location. Cascading results in widespread electric service interruption that cannot be restrained from sequentially spreading beyond an area predetermined by studies. |
| 012-0.3 | Certificate |  | A digital document that typically includes the public key, information about the identity of the party holding the corresponding private key, the operational periods of the certificate, and the Issuing CA’s own digital signature. |  |
| 012-0.4 | Certificate Policy | CP | A named set of rules that indicates the applicability of a certificate to a particular community and/or class of application with common security requirements. For example, a particular certificate policy might indicate applicability of a type of certificate to the authentication of electronic data interchange transactions for the trading of goods within a given price range. |  |
| 012-0.7 | Certificate Revocation List | CRL | A list of Certificate serial numbers which have been revoked, are no longer valid, and should not be relied upon by any system user. |  |
| 012-0.5 | Certification Path |  | An ordered sequence of certificates which, together with the public key of the initial object in the path, can be processed to obtain that of the final object in the path. |  |
| 012-0.6 | Certification Practice Statement | CPS | A statement of the practices which a certification authority employs in issuing certificates. |  |
|  | Clock Hour |  |  | The 60-minute period ending at :00. All surveys, measurements, and reports are based on Clock Hour periods unless specifically noted. |
|  | Cogeneration |  |  | Production of electricity from steam, heat, or other forms of energy produced as a by-product of another process. |
| 001-0.3 | Commission |  | The Federal Energy Regulatory Commission, or appropriate regulating authority. |  |
|  | Compliance Monitor |  |  | The entity that monitors, reviews, and ensures compliance of responsible entities with reliability standards. |
| 004-0.5 | Confirmed Interchange |  | The state where the Interchange Authority has verified the Arranged Interchange. | The state where the Interchange Authority has verified the Arranged Interchange. |
|  | Congestion Management Report |  |  | A report that the Interchange Distribution Calculator issues when a Reliability Coordinator initiates the Transmission Loading Relief procedure. This report identifies the transactions and native and network load curtailments that must be initiated to achieve the loading relief requested by the initiating Reliability Coordinator. |
| 008-0.7 | Constrained Facility |  | A transmission facility (line, transformer, breaker, etc.) that is approaching, is at, or is beyond its SOL or IROL. | A transmission facility (line, transformer, breaker, etc.) that is approaching, is at, or is beyond its System Operating Limit or Interconnection Reliability Operating Limit. |
| 008-0.8 | Constrained Flowgate |  | A Flowgate that is approaching, is at, or is beyond System Operating Limits (SOL) or Interconnection Reliability Operating Limits (IROL). |  |
| 008-0.9 | Constraint |  | A limitation placed on Interchange Transactions that flow over a Constrained Facility or Flowgate. |  |
| 008-0.10 | Constraint Path |  | A predetermined Transmission Service electrical path between contiguous Transmission Service Providers established for scheduling and commercial settlement purposes that represents the continuous flow of electrical energy between the parties to a transaction. |  |
|  | Contingency |  |  | The unexpected failure or outage of a system component, such as a generator, transmission line, circuit breaker, switch or other electrical element. |
|  | Contingency Reserve |  |  | The provision of capacity deployed by the Balancing Authority to meet the Disturbance Control Standard (DCS) and other NERC and Regional Reliability Organization contingency requirements. |
|  | Contract Path |  |  | An agreed upon electrical path for the continuous flow of electrical power between the parties of an Interchange Transaction. |
| 007-0.4 | Control Performance Standard (NERC) CPS (NAESB) | CPS | Control Performance Standard as defined by NERC | CPS The reliability standard that sets the limits of a Balancing Authority’s Area Control Error over a specified time period. |
|  | Corrective Action Plan |  |  | A list of actions and an associated timetable for implementation to remedy a specific problem. |
|  | Cranking Path |  |  | A portion of the electric system that can be isolated and then energized to deliver electric power from a generation source to enable the startup of one or more other generating units. |
|  | Critical Assets |  |  | Facilities, systems, and equipment which, if destroyed, degraded, or otherwise rendered unavailable, would affect the reliability or operability of the Bulk Electric System. |
|  | Critical Cyber Assets |  |  | Cyber Assets essential to the reliable operation of Critical Assets. |
| 004-0.6 | Curtailment |  | A reduction in the scheduled capacity or energy delivery of an Interchange transaction. | A reduction in the scheduled capacity or energy delivery of an Interchange Transaction. |
| 008-0.11 | Curtailment Threshold |  | The minimum Transfer Distribution Factor which, if exceeded, will subject an Interchange Transaction to curtailment to relieve a transmission facility Constraint. | The minimum Transfer Distribution Factor which, if exceeded, will subject an Interchange Transaction to curtailment to relieve a transmission facility constraint. |
|  | Cyber Assets |  |  | Programmable electronic devices and communication networks including hardware, software, and data. |
|  | Cyber Security Incident |  |  | Any malicious act or suspicious event that: • Compromises, or was an attempt to compromise, the Electronic Security Perimeter or Physical Security Perimeter of a Critical Cyber Asset, or, • Disrupts, or was an attempt to disrupt, the operation of a Critical Cyber Asset. |
|  | Delayed Fault Clearing |  |  | Fault clearing consistent with correct operation of a breaker failure protection system and its associated breakers, or of a backup protection system with an intentional time delay. |
|  | Demand |  |  | - The rate at which electric energy is delivered to or by a system or part of a system, generally expressed in kilowatts or megawatts, at a given instant or averaged over any designated interval of time. - The rate at which energy is being used by the customer. |
|  | Demand-Side Management | DSM |  | The term for all activities or programs undertaken by Load-Serving Entity or its customers to influence the amount or timing of electricity they use. |
| 001-0.4 | Denial of Service |  | This is the intentional or unintentional degradation of OASIS performance that impacts all customer interactions with OASIS by consuming cyber resources. |  |
|  | Direct Control Load Management | DCLM |  | Demand-Side Management that is under the direct control of the system operator. DCLM may control the electric supply to individual appliances or equipment on customer premises. DCLM as defined here does not include Interruptible Demand. |
|  | Dispersed Load by Substations |  |  | Substation load information configured to represent a system for power flow or system dynamics modeling purposes, or both. |
|  | Distribution Factor | DF |  | The portion of an Interchange Transaction, typically expressed in per unit that flows across a transmission facility (Flowgate). |
|  | Distribution Provider |  |  | Provides and operates the “wires” between the transmission system and the end-use customer. For those end-use customers who are served at transmission voltages, the Transmission Owner also serves as the Distribution Provider. Thus, the Distribution Provider is not defined by a specific voltage, but rather as performing the Distribution function at any voltage. |
|  | Disturbance |  |  | - An unplanned event that produces an abnormal system condition. - Any perturbation to the electric system. - The unexpected change in ACE that is caused by the sudden failure of generation or interruption of load. |
|  | Disturbance Control Standard | DCS |  | The reliability standard that sets the time limit following a Disturbance within which a Balancing Authority must return its Area Control Error to within a specified range. |
|  | Disturbance Monitoring Equipment | DME |  | Devices capable of monitoring and recording system data pertaining to a Disturbance. Such devices include the following categories of records: - Sequence of event recorders which record equipment response to the event - Fault recorder, which record actual waveform data replicating the system primary voltages and currents. This may include protective relays. - Dynamic Disturbance Recorders (DDRs), which record incidents that portray power system behavior during dynamic events such as low-frequency (0.1 Hz – 3 Hz) oscillations and abnormal frequency or voltage excursions |
| 008-0.12 | Dynamic Interchange Schedule or Dynamic Schedule (NERC) Dynamic Schedule (NAESB) |  | A telemetered reading or value that is updated in real time and used as a schedule in the AGC/ACE equation and the integrated value of which is treated as a schedule for interchange accounting purposes. Commonly used for scheduling jointly owned generation to or from another Balancing Authority Area. | A telemetered reading or value that is updated in real time and used as a schedule in the AGC/ACE equation and the integrated value of which is treated as a schedule for interchange accounting purposes. Commonly used for scheduling jointly owned generation to or from another Balancing Authority Area. |
| 005-0.4 | Dynamic Schedule |  | A telemetered reading or value that is updated in real time and used as a schedule in the ACE equation and the integrated value of which is treated as a schedule for interchange accounting purposes. Commonly used for “scheduling” jointly owned generation to or from another Balancing Authority Area. |  |
|  | Dynamic Transfer |  |  | The provision of the real-time monitoring, telemetering, computer software, hardware, communications, engineering, energy accounting (including inadvertent interchange), and administration required to electronically move all or a portion of the real energy services associated with a generator or load out of one Balancing Authority Area into another. |
|  | Economic Dispatch |  |  | The allocation of demand to individual generating units on line to effect the most economical production of electricity. |
|  | Electrical Energy |  |  | The generation or use of electric power by a device over a period of time, expressed in kilowatthours (kWh), megawatthours (MWh), or gigawatthours (GWh). |
|  | Electronic Security Perimeter |  |  | The logical border surrounding a network to which Critical Cyber Assets are connected and for which access is controlled. |
|  | Element |  |  | Any electrical device with terminals that may be connected to other electrical devices such as a generator, transformer, circuit breaker, bus section, or transmission line. An element may be comprised of one or more components. |
| 001-0.17 | Eligible Customer |  | As defined in the FERC Pro Forma Open Access Transmission Tariff. |  |
|  | Emergency or BES Emergency |  |  | Any abnormal system condition that requires automatic or immediate manual action to prevent or limit the failure of transmission facilities or generation supply that could adversely affect the reliability of the Bulk Electric System. |
|  | Emergency Rating |  |  | The rating as defined by the equipment owner that specifies the level of electrical loading or output, usually expressed in megawatts (MW) or Mvar or other appropriate units, that a system, facility, or element can support, produce, or withstand for a finite period. The rating assumes acceptable loss of equipment life or other physical or safety limitations for the equipment involved. |
| 012-0.8 | End Entity |  | A registered business entity or other recognized organization to which Certificates are issued by one or more Authorized Certification Authorities. |  |
|  | Energy Emergency |  |  | A condition when a Load-Serving Entity ahs exhausted all other options and can no longer provide its customers’ expected energy requirements. |
|  | Equipment Rating |  |  | The maximum and minimum voltage, current, frequency, real and reactive power flows on individual equipment under steady state, short-circuit and transient conditions, as permitted or assigned by the equipment owner. |
|  | Facility |  |  | A set of electrical equipment that operates as a single Bulk Electric System Element (e.g., a line, a generator, a shunt compensator, transformer, etc.) |
|  | Facility Rating |  |  | The maximum or minimum voltage, current, frequency, or real or reactive power flow through a facility that does not violate the applicable equipment rating of any equipment comprising the facility. |
|  | Fault |  |  | An event occurring on an electric system such as a short circuit, a broken wire, or an intermittent connection. |
| 001-0.14 | FERC |  | Federal Energy Regulatory Commission |  |
| 001-0.18 | Financially Obligated Transmission Customer | FOTC | The customer financially obligated to the Transmission Provider for transmission service (i.e., service procured either through direct purchase from the Transmission Provider, Reseller, or through a Transfer of transmission rights). |  |
|  | Fire Risk |  |  | The likelihood that a fire will ignite or spread in a particular geographic area. |
|  | Firm Demand |  |  | That portion of the Demand that a power supplier is obligated to provide except when system reliability is threatened or during emergency conditions. |
| 008-0.13 | Firm Transmission Service |  | The highest quality service offered to customers under a filed rate schedule that anticipates no planned interruption. Firm Transmission Service includes Firm Point-to-point Transmission Service and Firm Network Integration Transmission Service. | The highest quality (priority) service offered to customers under a filed rate schedule that anticipates no planned interruption. |
|  | Flashover |  |  | An electrical discharge through air around or over the surface of insulation, between objects of different potential, caused by placing a voltage across the air space that results in the ionization of the air space. |
| 008-0.14 | Flowgate |  | A designated point of the transmission system through which the Interchange Distribution Calculator calculates the power flow from Interchange Transactions. | A designated point on the transmission system through which the Interchange Distribution Calculator calculates the power flow from Interchange Transactions. |
|  | Forced Outage |  |  | - The removal from service availability of a generating unit, transmission line, or other facility for emergency reasons. - The condition in which the equipment is unavailable due to unanticipated failure. |
| 008-0.15 | Frequency Bias |  | A value, usually expressed in megawatts per 0.1 hertz (MW/0.1 Hz), associated with a Balancing Authority Area that approximates the Balancing Authority Area’s response to Interconnection and frequency error. | A value, usually expressed in megawatts per 0.1 Hertz (MW/0.1 Hz), associated with a Balancing Authority Area that approximates the Balancing Authority Area’s response to Interconnection frequency error. |
| 006-0.3 | Frequency Bias Setting |  | A value, in MW/0.1 Hz, set into a Balancing Authority’s AGC equipment to represent a Balancing Authority’s response to a frequency deviation. | A value, usually expressed in MW/0.1 Hz, set into a Balancing Authority ACE algorithm that allows the Balancing Authority to contribute its frequency response to the Interconnection. |
|  | Frequency Deviation |  |  | A change in Interconnection frequency. |
|  | Frequency Error |  |  | The difference between the actual and scheduled frequency. (FA – FS) |
|  | Frequency Regulation |  |  | The ability of a Balancing Authority to help the Interconnection maintain Scheduled Frequency. This assistance can include both turbine governor response and Automatic Generation Control. |
|  | Frequency Response |  |  | (Equipment) The ability of a system or elements of the system to react or respond to a change in system frequency. (System) The sum of the change in demand, plus the change in generation, divided by the change in frequency, expressed in megawatts per 0.1 Hertz (MW/0.2 Hz). |
|  | Generator Operator |  |  | The entity that operates generating unit(s) and performs the functions of supplying energy and Interconnected Operations Services. |
|  | Generator Owner |  |  | Entity that owns and maintains generating units. |
| 008-0.16 | Generator Shift Factor (NERC) Generation Shift Factor (NAESB) | GSF | A factor to be applied to a generator’s expected change in output to determine the amount of flow contribution that change in output will impose on an identified transmission facility or monitored Flowgate. | A factor to be applied to a generator’s expected change in output to determine the amount of flow contribution that change in output will impose on an identified transmission facility or Flowgate. |
| 004-0.7 | Generator-Providing Entity | GPE | The Purchasing Selling Entity who is responsible for providing the source generation from owned, affiliated, or contractually bound generation. |  |
| 008-0.17 | Generator-to-Load Distribution Factor | GLDF | The algebraic sum of a GSF and an LSF to determine to total impact of an Interchange Transaction on an identified transmission facility or monitored Flowgate. | The algebraic sum of a Generator Shift Factor and a Load Shift Factor to determine the total impact of an Interchange Transaction on an identified transmission facility or Flowgate. |
|  | Host Balancing Authority |  |  | - A Balancing Authority that confirms and implements Interchange Transactions for a Purchasing Selling Entity that operates generation or serves customers directly within the Balancing Authority’s metered boundaries. - The Balancing Authority within whose metered boundaries a jointly owned unit is physically located. |
|  | Hourly Value |  |  | Data measured on a Clock Hour basis. |
| 001-0.5 | Identical Service Requests |  | “identical service requests” are those OASIS transmission service requests that have exactly the same values for the following OASIS template Data Elements: - CUSTOMER_CODE - CUSTOMER_DUNS - SERVICE_INCREMENT - TS_CLASS - START_TIME - STOP_TIME - POR* - POD* - PATH* *Service requests where any combination of PATH, POR and/or POD represent exactly the same commercial transmission elements shall be considered as “having the exact same value.” |  |
| 004-0.8 | Implemented Interchange |  | The state where the Balancing Authority enters the Confirmed Interchange into its Area Control Error equation. | The state where the Balancing Authority enters the Confirmed Interchange into its Area control Error equation. |
| 007-0.5 | Inadvertent Interchange |  | The difference between a Balancing Authority’s net actual interchange and net scheduled interchange. | The difference between the Balancing Authority’s Net Actual Interchange and Net Scheduled Interchange. (IA – I S) |
|  | Independent Power Producer | IPP |  | Any entity that owns or operates an electricity generating facility that is not included in an electric utility’s rate base. This term includes, but is not limited to, cogenerators and small power producers and all other nonutility electricity producers, such as exempt wholesale generators, who sell electricity. |
|  | Institute of Electrical and Electronics Engineers, Inc. | IEEE |  |  |
| 004-0.9 | Interchange |  | Energy transfers that cross Balancing Authority boundaries. | Energy transfers that cross Balancing Authority boundaries. |
| 004-0.10 | Interchange Authority | IA | The responsible entity that authorizes implementation of valid and balanced Interchange schedules between Balancing Authority Areas, and ensures communication of Interchange information for reliability assessment purposes. | The responsible entity that authorizes implementation of valid and balanced Interchange Schedules between Balancing Authority Areas, and ensures communication of Interchange information for reliability assessment purposes. |
| 004-0.11 | Interchange Block Accounting |  | Energy accounting that assumes a beginning and ending ramp time of zero minutes. For accounting purposes, this moves the energy associated with the starting and ending ramps into the adjacent starting and ending clock time of the Interchange. |  |
| 008-0.18 | Interchange Distribution Calculator | IDC | The mechanism used by Reliability Coordinators in the Eastern Interconnection to calculate the distribution of Interchange Transactions over specific transmission interfaces, which are known as “Flowgates.” It includes a database of all Interchange Transactions and a matrix of the Distribution Factors for the Eastern Interconnection. | The mechanism used by Reliability Coordinators in the Eastern Interconnection to calculate the distribution of Interchange Transactions over specific Flowgates. It includes a database of all Interchange Transactions and a matrix of the Distribution Factors for the Eastern Interconnection. |
| 005-0.5 006-0.4 007-0.6 | Interchange Schedule |  | The planned energy exchange between two adjacent Balancing Authorities. | An agreed-upon Interchange Transaction size (megawatts), start and end time, beginning and ending ramp times and rate, and type required for delivery and receipt of power and energy between the Source and Sink Balancing Authorities involved in the transaction. |
| 008-0.19 | Interchange Transaction |  | A transaction that crosses one or more Balancing Authorities’ boundaries. The planned energy exchange between two adjacent Balancing Authorities. | An agreement to transfer energy from a seller to a buyer that crosses one or more Balancing Authority Area boundaries. |
| 008-0.20 | Interchange Transaction Tag or Tag |  | An Interchange Transaction being submitted for implementation according to NERC “Electronic Tagging Functional Specification”, version 1.7.095. | The details of an Interchange Transaction required for its physical implementation. |
| 004-0.12 | Interconnected Operations Service |  | A service (exclusive of basic energy and transmission services) that is required to support the reliable operation of the interconnected bulk electric systems. | A service (exclusive of basic energy and transmission services) that is required to support the reliable operation of interconnected Bulk Electric Systems. |
| 005-0.6 006-0.5 007-0.7 008-0.21 | Interconnection |  | Any one of the three major electric system networks in North America: Eastern, Western, and ERCOT. | When capitalized, any one of the three major electric system networks in North America: Eastern, Western, and ERCOT. |
| 008-0.22 | Interconnection Reliability Operating Limit | IROL | The value (such as MW, MVar, Amperes, Frequency or Volts) derived from, or a subset of, the System Operating Limit, which if exceeded, could expose a widespread area of the Bulk Electric System to instability, uncontrolled separation(s) or cascading outages. | A System Operating Limit that, if violated, cold lead to instability, uncontrolled separation, or Cascading Outages that adversely impact the reliability of the Bulk Electric System. |
|  | Interconnection Reliability Operating Limit Tv | IROL Tv |  | The maximum time that an Interconnection Reliability Operating Limit can be violated before the risk to the interconnection or other Reliability Coordinator Area(s) becomes greater than acceptable. Each Interconnection Reliability Operating Limit’s Tv shall be less than or equal to 30 minutes. |
| 006-0.6 | Interconnection Time Monitor |  | An entity that monitors Time Error and initiates and terminates Time Error Corrections. |  |
|  | Intermediate Balancing Authority |  |  | A Balancing Authority Area that has connecting facilities in the Scheduling Path between the Sending Balancing Authority Area and Receiving Balancing Authority Area and operating agreements that establish the conditions for the use of such facilities. |
|  | Interruptible Load or Interruptible Demand |  |  | Demand that the end-use customer makes available to its Load-Serving Entity via contract or agreement for curtailment. |
| 012-0.9 | Issuing Certification Authority | Issuing CA | In the context of a particular certificate, the issuing CA is the CA that has digitally signed and issued the certificate to a Subscriber. |  |
|  | Joint Control |  |  | Automatic Generation Control of jointly owned units by two or more Balancing Authorities. |
| 005-0.7 | Jointly Owned Units | JOU[s] | This term refers to a unit in which two or more entities share ownership. |  |
| 007-0.8 | L10 |  | A control error limitation specified in NERC standards. |  |
| 006-0.7 | Leap Second |  | A Leap Second is a second of time added to Coordinated Universal Time to make it agree with astronomical time to within 0.9 seconds. Historically, Leap Seconds are implemented as needed on June 30th or December 31st. (National Institute of Standards and Technology) |  |
|  | Limiting Element |  |  | The element that is 1) Either operating at its appropriate rating, or 2) would be following the limiting contingency. Thus, the Limiting Element establishes a system limit. |
|  | Load |  |  | An end-use device or customer that receives power from the electric system. |
| 008-0.23 | Load Shift Factor | LSF | A factor to be applied to a load’s expected change in demand to determine the amount of flow contribution that change in demand will impose on an identified transmission facility or monitored Flowgate. | A factor to be applied to a load’s expected change in demand to determine the amount of flow contribution that change in demand will impose on an identified transmission facility or monitored Flowgate. |
| 004-0.13 | Load-Serving Entity | LSE | The responsible entity that secures energy and transmission service (and related Interconnected Operations Services) to serve the electrical demand and energy requirements of its end-use customers. | Secures energy and transmission service (and related Interconnected Operations Services) to serve the electrical demand and energy requirements of its end-use customers. |
| 004-0.14 | Market Assembly |  | The function responsible for coordinating the submittal of a completed and accurate Arranged Interchange from the Requesting Purchasing-Selling Entity to the Interchange Authority within an organized Market. |  |
| 004-0.15 | Market Period |  | The period of time beginning with the Requesting Purchasing-Selling Entity (PSE), or its designee, making required purchase, sale, and transmission service arrangements to support the Arranged Interchange through the period of time when the Interchange Authority receives the Arranged Interchange. |  |
| 013-0.5 | May, should, or optional |  | Define optional requirements that are recommended for implementation in OASIS but are not specifically required under these Standards. |  |
|  | Misoperation |  |  | - Any failure of a Protection System element to operate within the specified time when a fault or abnormal condition occurs within a zone of protection. - Any operation for a fault not within a zone of protection (other than operation as backup protection for a fault in an adjacent zone that is not cleared within a specified time for the protection for that zone). - Any unintentional Protection System operation when no fault or other abnormal condition has occurred unrelated to on-site maintenance and testing activity. |
| 013-0.4 | Must, shall, or required |  | Define specific requirements for OASIS processing that are not optional and must be implemented as described. |  |
| 008-0.24 | Native Load | NL | The demand imposed on an electric utility or an entity by the requirements of all customers located within a franchised service territory that the electric utility or entity has statutory or contractual obligation to serve. | The end-use customers that the Load-Serving Entity is obligated to serve. |
| 008-0.25 | NERC |  | North American Electric Reliability Corporation |  |
| 005-0.8 | Net Actual Interchange | NIa | The algebraic sum of all metered interchange over all interconnections between two physically adjacent Balancing Authority Areas. | The algebraic sum of all metered interchange over all interconnections between two physically Adjacent Balancing authority Areas. |
|  | Net Energy for Load |  |  | Net Balancing Authority Area generation, plus energy received from other Balancing Authority Areas, less energy delivered to Balancing Authority Areas through interchange. It includes Balancing Authority Area losses but excludes energy required for storage at energy storage facilities. |
| 005-0.9 | Net Interchange Schedule | NIs | The algebraic sum of all Interchange Schedules with each adjacent Balancing Authority Area. | The algebraic sum of all Interchange Schedules with each Adjacent Balancing Authority. |
|  | Net Scheduled Interchange |  |  | The algebraic sum of all Interchange Schedules across a given path or between Balancing Authorities for a given period or instant in time. |
| 008-0.26 | Network Integration Transmission Service (NERC) Network Integration (NI) Transmission Service (NAESB) |  | As specified in the Transmission Service Provider’s tariff, service that allows an electric Transmission Customer to integrate, plan, economically dispatch and regulate its network resources in a manner comparable to that in which the transmission owner serves Native Load customers. | Service that allows an electric transmission customer to integrate, plan, economically dispatch and regulate its network reserves in a manner comparable to that in which the Transmission Owner serves Native Load customers. |
| 008-0.27 | Non-Firm Transmission Service |  | As specified in the Transmission Service Provider’s tariff, transmission service that is reserved and scheduled on an as-available basis and is subject to curtailment or interruption, and has less priority than Firm Transmission. | Transmission service that is reserved on an as-available basis and is subject to curtailment or interruption. |
|  | Non-Spinning Reserve |  |  | - That generating reserve not connected to the system but capable of serving demand within a specified time. - Interruptible load that can be removed from the system in a specified time. |
|  | Normal Clearing |  |  | A protection system operates as designed and the fault is cleared in the time normally expected with proper functioning of the installed protection systems. |
|  | Normal Rating |  |  | The rating as defined by the equipment owner that specifies the level of electrical loading, usually expressed in megawatts (MW) or other appropriate units that a system, facility, or element can support or withstand through the daily demand cycles without loss of equipment life. |
|  | Nuclear Plant Generator Operator |  |  | Any Generator Operator or Generator Owner that is a Nuclear Plant Licensee responsible for operation of a nuclear facility licensed to produce commercial power. |
|  | Nuclear Plant Interface Requirements (NPIRs) |  |  | The requirements based on NPLRs and Bulk Electric System requirements that have been mutually agreed to by the Nuclear Plant Generator Operator and the applicable Transmission Entities. |
|  | Nuclear Plant Licensing Requirements (NPLRs) |  |  | Requirements included in the design basis of the nuclear plant and statutorily mandated for the operation of the plant, including nuclear power plant licensing requirements for: - Off-site power supply to enable safe shutdown of the plant during an electric system or plant event; and - Avoiding preventable challenges to nuclear safety as a result of an electric system disturbance, transient, or condition. |
|  | Nuclear Plant Off-site Power Supply (Off-site Power) |  |  | The electric power supply provided from the electric system to the nuclear power plant distribution system as required per the nuclear power plant license. |
|  | Off-Peak |  |  | Those hours or other periods defined by NAESB business practices, contract, agreements, or guides as periods of lower electrical demand. |
|  | On-Peak |  |  | Those hours or other periods defined by NAESB business practices, contract, agreements, or guides as periods of higher electrical demand. |
| 002-0.1 | Open Access Same Time Information Service | OASIS | Comprises the computer systems and associated communications facilities that public utilities are required to provide for the purpose of making available to all transmission users comparable interactions with TS Information. | An electronic posting system that the Transmission Service Provider maintains for transmission access data and that allows all transmission customers to view the data simultaneously. |
| 013-0.1 | OASIS |  | Refers to the Primary Provider’s implementation of the OASIS Transmission Customer interface as defined in Standards WEQ-002, and any back-end supporting systems or user procedures that collectively perform the transaction processing functions associated with handling of requests on OASIS. |  |
| 002-0.2 | Open Access Same-Time Information System Node | OASIS Node | Is a subsystem of the OASIS. It is one computer system in the (OASIS) that provides access to TS Information to a Transmission Customer. |  |
|  | Open Access Transmission Tariff | OATT |  | Electronic transmission tariff accepted by the U.S. Federal Energy Regulatory Commission requiring the Transmission Service Provider to furnish to all shippers with non-discriminating service comparable to that provided by Transmission Owners to themselves. |
|  | Operating Plan |  |  | A document that identifies a group o activities that may be used to achieve some goal. An Operating Plan may contain Operating Procedures and Operating Processes. A company-specific system restoration plan that includes an Operating Procedure for black-starting units, Operating Processes for communicating restoration progress with other entities, etc., is an example of an Operating Plan. |
|  | Operating Procedure |  |  | A document that identifies specific steps or tasks that should be taken by one or more specific operating positions to achieve specific operating goals). The steps in an Operating Procedure should be followed in the order in which they are presented, and should be performed by the position(s) identified. A document that lists the specific steps for a system operator to take in removing a specific transmission line from service is an example of an Operating Procedure. |
|  | Operating Process |  |  | A document that identifies general steps for achieving a generic operating goal. An Operating Process includes steps with options that may be selected depending upon Real-time conditions. A guideline for controlling high voltage is an example of an Operating Process. |
|  | Operating Reserve |  |  | That capability above firm system demand required to provide for regulation, load forecasting error, equipment forced and scheduled outages and local area protection. It consists of spinning and non-spinning reserve. |
|  | Operating Reserve – Spinning |  |  | The portion of Operating Reserve consisting of: - Generation synchronized to the system and fully available to serve load within the Disturbance Recovery Period following the contingency event; or - Load fully removable from the system within the Disturbance Recovery Period following the contingency event. |
|  | Operating Reserve – Supplemental |  |  | The portion of Operating Reserve consisting of: - Generation (synchronized or capable of being synchronized to the system) that is fully available to serve load within the Disturbance Recovery Period following the contingency event; or - Load fully removable from the system within the Disturbance Recovery Period following the contingency event. |
|  | Operating Voltage |  |  | The voltage level by which an electrical system is designated and to which certain operating characteristics of the system are related; also, the effective (root-mean-square) potential difference between any two conductors or between a conductor and the ground. The actual voltage of the circuit may vary somewhat above or below this value. |
|  | Overlap Regulation Service |  |  | A method of providing regulation service in which the Balancing Authority providing the regulation service incorporates another Balancing Authority’s actual interchange, frequency response, and schedules into providing Balancing Authority’s AGC/ACE equation. |
| 001-0.6 | Parent Reservation |  | An existing, confirmed reservation being modified by a Redirect, Transfer, Resale, etc. |  |
|  | Peak Demand |  |  | - The highest hourly integrated Net Energy For Load within a Balancing Authority Area occurring within a given period (e.g., day, month, season, or year). - The highest instantaneous demand within the Balancing Authority Area. |
| 008-0.28 | Per Generator Method |  | A methodology used by the IDC to calculate the portion of parallel flows on any Constrained Facility or Flowgate due to Network Integrated (NI) transmission service customers and service to Native Load (NL) customers for each Balancing Authority. |  |
|  | Performance-Reset Period |  |  | The time period that the entity being assessed must operate without any violations to reset the level of non compliance to zero. |
|  | Physical Security Perimeter |  |  | The physical, completely enclosed (“six-wall”) border surrounding computer rooms, telecommunications rooms, operations centers, and other locations in which Critical Cyber Assets are housed and for which access is controlled. |
|  | Planning Authority |  |  | The responsible entity that coordinates and integrates transmission facility and service plans, resource plans, and protection systems. |
|  | Point of Delivery | POD |  | A location that the Transmission Service Provider specifies on its transmission system where an Interchange Transaction leaves or a Load-Serving Entity receives its energy. |
|  | Point of Receipt | POR |  | A location that the Transmission Service Provider specifies on its transmission system where an Interchange Transaction enters or a Generator delivers its output. |
| 008-0.29 | Point to Point Transmission Service (NERC) Point-to-point (PTP) Transmission Service (NAESB) | PTP | As specified in the Transmission Service Providers tariff, Transmission Service reserved and/or scheduled between specified points of receipt and delivery. | The reservation and transmission of capacity and energy on either a firm or non-firm basis from the Point(s) of Receipt to the Point(s) of Delivery. |
| 012-0.10 | Policy Qualifier |  | Policy-dependent information that accompanies a certificate policy identifier in an X.509 certificate. |  |
| 011-0.1 | Power Plant Operator | PPO | Is the term used to describe the entity(ies) that has responsibility for gas requirements for a natural gas-fired electric generating facility(ies) and is responsible for coordinating natural gas deliveries with the appropriate Transportation Service Provider(s) (TSP) to meet those requirements. The PPO performs a number of coordinated activities, including, but not limited to, power plant operations, unit dispatch, natural gas procurement and/or gas transportation arrangements. Because each PPO is structured differently, specific responsibilities within each PPO should be determined by the PPO and the point of contact for the PPO should be communicated to the TSP(s). This definition applies to NAESB WEQ Standard Nos. WEQ-011-0.2, WEQ-011-1.1, WEQ-011-1.2, WEQ-011-1.3, WEQ-011-1.4, WEQ-011-1.5, and WEQ-011-1.6 and NAESB WGQ Standard Nos. 0.2.2, 0.3.11, 0.3.12, 0.3.13, 0.3.14, and 0.3.15. |  |
| 011-0.2 | Power Plant Operator’s Facility |  | Is the term used to describe the natural gas-fired electric generating unit(s) under the direct control of the Power Plant Operator. This definition applies to NAESB WEQ Standard Nos. WEQ-011-1.2 and WEQ-011-1.3 and NAESB WGQ Standard Nos. 0.3.12 and 0.3.13. |  |
|  | Pro Forma Tariff |  |  | Usually refers to the standard OATT and/or associated transmission rights mandated by the U.S. Federal Energy Regulatory Commission Order No. 888. |
|  | Protection System |  |  | Protective relays, associated communication systems, voltage and current sensing devices, station batteries and DC control circuitry. |
| 005-0.10 | Pseudo-Tie |  | A telemetered reading or value that is updated in real time and used as a tie line flow in the ACE equation but for which no physical tie or energy metering actually exists. The integrated value is used as a metered MWh value for interchange accounting purposes. | A telemetered reading or value that is updated in real time and used as a “virtual” tie line flow in the AGC/ACE equation but for which no physical tie or energy metering actually exists. The integrated value is used as a metered MWh value for interchange accounting purposes. |
| 004-0.16 | Purchasing-Selling Entity | PSE | The entity that purchases or sells, and takes title to, energy, capacity, and Interconnected Operations Services. Purchasing-Selling Entities may be affiliated or unaffiliated merchants and may or may not own generating facilities. | The entity that purchases or sells, and takes title to, energy, capacity, and Interconnected Operations Services. Purchasing-Selling Entities may be affiliated or unaffiliated merchants and may or may not own generating facilities. |
| 008-0.30 | Purchasing-Selling Entity | PSE | The entity that purchases or sells and takes title to energy capacity and interconnected operations services. PSE’s may be affiliated or unaffiliated merchants and may and may not own generating facilities. |  |
| 001-0.7 | Queue Flooding |  | Excessive submission of identical service requests. |  |
| 001-0.8 | Queue Hoarding |  | This is the act, intentionally or unintentionally, of not confirming or withdrawing an accepted service request such that it impacts the ability of other willing buyers to secure service in a timely fashion. |  |
|  | Ramp Rate or Ramp |  |  | (Schedule) The rate, expressed in megawatts per minute, at which the interchange schedule is attained during the ramp period. (Generator) The rate, expressed in megawatts per minute, that a generator changes its output. |
|  | Rated Electrical Operating Conditions |  |  | The specified or reasonably anticipated conditions under which the electrical system or an individual electrical circuit is intend/designed to operate. |
|  | Rating |  |  | The operational limits of a transmission system element under a set of specified conditions. |
|  | Reactive Power |  |  | The portion of electricity that establishes and sustains the electric and magnetic fields of alternating-current equipment. Reactive power must be supplied to most types of magnetic equipment, such as motors and transformers. It also must supply the reactive losses on transmission facilities. Reactive power is provided by generators, synchronous condensers, or electrostatic equipment such as capacitors and directly influences electric system voltage. It is usually expressed in kilovars(kvar) or megavars(Mvar). |
|  | Real Power |  |  | The portion of electricity that supplies energy to the load. |
| 008-0.32 | Reallocation |  | The process used to totally or partially curtail Transactions during TLR levels 3a, 3b or 5a events to allow Transactions using equal or higher priority to be implemented. | The total or partial curtailment of Transactions during TLR Level 3a or 5a to allow Transactions using higher priority to be implemented. |
|  | Real-time |  |  | Present time as opposed to future time. (From Interconnection Reliability Operating Limits standards.) |
|  | Receiving Balancing Authority |  |  | The Balancing Authority importing the Interchange. |
|  | Regional Reliability Organization |  |  | - An entity that ensures that a defined area of the Bulk Electric System is reliable, adequate and secure. - A member of the North American Electric Reliability Council. The Regional Reliability Organization can serve as the Compliance Monitor. |
|  | Regional Reliability Plan |  |  | The plan that specifies the Reliability Coordinators and Balancing Authorities within the Regional Reliability Organization, and explains how reliability coordination will be accomplished. |
| 007-0.9 | Regions |  | One of the North American Electric Reliability Corporation regional councils or affiliate. |  |
| 012-0.11 | Registration Authority | RA | An entity that is responsible for identification and authentication of certificate subjects, but that does not sign or issue certificates (i.e., an RA is delegated certain tasks on behalf of a CA). |  |
|  | Regulating Reserve |  |  | An amount of reserve responsive to Automatic Generation Control, which is sufficient to provide normal regulating margin. |
|  | Regulation Service |  |  | The process whereby one Balancing authority contracts to provide corrective response to all or a portion of the ACE of another Balancing Authority. The Balancing Authority providing the response assumes the obligation of meeting all applicable control criteria as specified by NERC for itself and the Balancing Authority for which it is providing the Regulation Service. |
| 008-0.33 | Reliability Area |  | The collection of generation, transmission, and loads within the boundaries of a Reliability Coordinator. Its boundary coincides with one or more Balancing Authority Areas. |  |
| 004-0.17 | Reliability Coordinator | RC | The entity with the highest level of authority that has responsibility for the reliable operation of the bulk electric system, has the wide area view of the bulk electric system, and has the operating tools, processes and procedures, including the authority to prevent or mitigate emergency operating situations in both next-day analysis and real-time operations. The Reliability Coordinator has the purview that is broad enough to enable the calculation of interconnection reliability operating limits, which may be based on the operating parameters of transmission systems beyond any transmission operator’s vision. | The entity that is the highest level of authority who is responsible for the reliable operation of the Bulk Electric System, has the Wide Area view of the Bulk Electric System, and has the operating tools, processes and procedures, including the authority to prevent or mitigate emergency operating situations in both next-day analysis and real-time operations. The Reliability Coordinator has the purview that is broad enough to enable the calculation of Interconnection Reliability Operating Limits, which may be based on the operating parameters of transmission systems beyond any Transmission Operator’s vision. |
| 008-0.34 | Reliability Coordinator | RC | An entity that provides the security assessment and emergency operations coordination for a group of Balancing Authorities, Transmission Service Providers, and Transmission Operators. |  |
|  | Reliability Coordinator Area |  |  | The collection of generation, transmission, and loads within the boundaries of the Reliability Coordinator. Its boundary coincides with one or more Balancing Authority Areas. |
| 008-0.31 | Reliability Coordinator Information System | RCIS | The system that Reliability Coordinators use to post messages and share operating information in real time. | The system that Reliability Coordinators use to post messages and share operating information in real time. |
| 004-0.18 | Reliability Period |  | The period of time beginning with the Interchange Authority (IA) requesting approvals from the reliability Approval Entities through the completion of the physical flow of the energy associated with the originally submitted Arranged Interchange. |  |
| 012-0.12 | Relying Party |  | A recipient of a certificate who acts in reliance on that certificate and/or digital signatures verified using that certificate. In this document, the terms "certificate user" and "relying party" are used interchangeably. |  |
|  | Remedial Action Scheme | RAS |  | See “Special Protection System” |
|  | Reportable Disturbance |  |  | Any event that causes an ACE change greater than or equal to 80% of a Balancing Authority’s or reserve sharing group’s most severe contingency. The definition of a reportable disturbance is specified by each Regional Reliability Organization. This definition may not be retroactively adjusted in response to observed performance. |
| 012-0.13 | Repository |  | A database of active digital signatures for a CA system. CA’s post certificates and CRLs to repositories that allows relying parties to confirm the status of the digital signatures. |  |
| 004-0.19 | Request for Interchange | RFI | A collection of required data, as defined in Appendix C of this standard, necessary for the purpose of submitting to the Interchange Authority as an Arranged Interchange. | A collection of data as defined in the NAESB RFI Datasheet, to be submitted to the Interchange Authority for the purpose of implementing bilateral Interchange between a Source and Sink Balancing Authority. |
| 004-0.20 | Requesting Purchasing-Selling Entity | PSE | The entity that prepares and submits the Request For Interchange (RFI) to the Interchange Authority (IA) and holds the transmission reservation being presented for use. |  |
| 001-0.19 | Resale |  | The request to convey scheduling rights associated with a reservation for Point-To-Point Transmission Service from a Reseller to an Assignee. |  |
| 001-0.10 | Reseller |  | The customer that holds Point-To-Point Transmission Service rights and offers those rights for sale on the (secondary) transmission market. |  |
|  | Reserve Sharing Group |  |  | A group whose members consist of two or more Balancing Authorities that collectively maintain, allocate, and supply operating reserves required for each Balancing Authority’s use in recovering from contingencies within the group. Scheduling energy from an Adjacent Balancing Authority to aid recovery need not constitute reserve sharing provided the transaction is ramped in over a period the supplying part could reasonably be expected to load generation in (e.g., ten minutes). If the transaction is ramped in quicker (e.., between zero and ten minutes) then, for the purposes of Disturbance Control Performance, the Areas become a Reserve Sharing Group. |
|  | Resource Planner |  |  | The entity that develops a long-term (generally one year and beyond) plan for the resource adequacy of specific loads (customer demand and energy requirements) within a Planning Authority Area. |
|  | Response Rate |  |  | The Ramp Rate that a generating unit can achieve under normal operating conditions expressed in megawatts per minutes (MW/Min). |
| 001-0.9 | Responsible Party |  | The Transmission Provider or an agent to whom the Transmission Provider has delegated the responsibility of meeting any of the requirements of this part. |  |
|  | Right-of-Way | ROW |  | A corridor of land on which electric lines may be located. The Transmission Owner may own the land in fee, own an easement, or have certain franchise, prescription, or license rights to construct and maintain lines. |
|  | Scenario |  |  | Possible event. |
|  | Schedule |  |  | (Verb) To set up a plan or arrangement for an Interchange Transaction. (Noun) An Interchange Schedule. |
|  | Scheduled Frequency |  |  | 60.0 Hertz, except during a time correction. |
|  | Scheduling Entity |  |  | An entity responsible for approving and implementing Interchange Schedules. |
|  | Scheduling Path |  |  | The Transmission Service arrangements reserved by the Purchasing-Selling Entity for a Transaction. |
| 002-0.3 | Secondary Transmission Provider (ST, Reseller, or Secondary Provider) |  | Is any Customer who offers to sell transmission capacity it has purchased. (Consistent with the definition of WEQ-001-0.10). |  |
|  | Sending Balancing Authority |  |  | The Balancing Authority exporting the Interchange. |
| 012-0.14 | Set of provisions |  | A collection of practice and/or policy statements, spanning a range of standard topics, for use in expressing a certificate policy definition or CPS employing the approach described in this framework. |  |
| 008-0.35 | Sink Balancing Authority |  | The Balancing Authority in which the load (Sink) is located for an Interchange Transaction. (This will also be a receiving Balancing Authority for the resulting Interchange Schedule). | The Balancing Authority in which the load (sink) is located for an Interchange Transaction. (This will also be a Receiving Balancing Authority for the resulting Interchange Schedule.) |
| 004-0.21 | Sink Balancing Authority |  | The Balancing Authority in which the load (sink) is located for an Interchange Transaction. |  |
| 004-0.22 | Source Balancing Authority |  | The Balancing Authority in which the generation (source) is located for an Interchange Transaction. | The Balancing Authority in which the generation (source) is located for an Interchange Transaction. (This will also be a Sending Balancing Authority for the resulting Interchange Schedule.) |
|  | Special Protection System (Remedial Action Scheme) |  |  | An automatic protection system designed to detect abnormal or predetermined system conditions, and take corrective actions other than and/or in additional to the isolation of faulted components to maintain system reliability. Such action may include changes in demand, generation (MW and Mvar), or system configuration to maintain system stability, acceptable voltage, or power flows. An SPS does not include (a) underfrequency or undervoltage load shedding or (b) fault conditions that must be isolated or (c) out-of-step relaying (not designed as an integral part of an SPS). Also called Remedial Action Scheme. |
|  | Spinning Reserve |  |  | Unloaded generation that is synchronized and ready to serve additional demand. |
|  | Stability |  |  | The ability of an electric system to maintain a state of equilibrium during normal and abnormal conditions or disturbances. |
|  | Stability Limit |  |  | The maximum power flow possible through some particular point in the system while maintaining stability in the entire system or the part of the system to which the stability limit refers. |
| 012-0.15 | Subscriber |  | An authorized individual, application, server, or ‘role’ associated with a registered end entity that has been issued a certificate by an Authorized Certification Authority. |  |
|  | Supervisory Control and Data Acquisition | SCADA |  | A system of remote control and telemetry used to monitor and control the transmission system. |
| 005-0.11 | Supplemental Regulation Service |  | A method of providing regulation service in which the Balancing Authority providing the regulation service receives a signal representing all or a portion of the other Balancing Authority’s ACE. | A method of providing regulation service in which the Balancing Authority providing the regulation service receives a signal representing all or a portion of the other Balancing Authority’s ACE. |
|  | Surge |  |  | A transient variation of current, voltage, or power flow in an electric circuit or across an electric system. |
|  | Sustained Outage |  |  | The deenergized condition of a transmission line resulting from a fault or disturbance following an unsuccessful automatic reclosing sequence and/or unsuccessful manual reclosing procedure. |
|  | System |  |  | A combination of generation, transmission, and distribution components. |
| 008-0.36 | System Operating Limit | SOL | The value (such as MW, MVar, Amperes, Frequency or Volts) that satisfies the most limiting of the prescribed operating criteria for a specified system configuration to ensure operation within acceptable reliability criteria. System Operating Limits are based upon certain operating criteria. | The value (such as MW, MVar, Amperes, Frequency or Volts) that satisfies the most limiting of the prescribed operating criteria for a specified system configuration to ensure operation within acceptable reliability criteria. System Operating limits are based upon certain operating criteria. These include, but are not limited to: - Facility Rating (Applicable pre- and post- Contingency equipment or facility ratings) - Transient Stability Ratings (Applicable pre- and post-Contingency Stability Limits) - Voltage Stability Ratings (Applicable pre- and post-Contingency Voltage Limits) |
|  | System Operator |  |  | An individual at a control center (Balancing Authority, Transmission Operator, Generator Operator, Reliability Coordinator) whose responsibility it is to monitor and control that electric system in real time. |
|  | Telemetering |  |  | The process by which measurable electrical quantities from substations and generating stations are instantaneously transmitted to the control center, and by which operating commands from the control center are transmitted to the substations and generating stations. |
| 013-0.3 | Template |  | Refers generically, or by reference to a specifically named template, to the templates defined for the Transmission Customer interface to OASIS in Standards WEQ-002, including the displays and forms associated with the web browser based user interface implementing the functions of an OASIS defined template. |  |
|  | Thermal Rating |  |  | The maximum amount of electrical current that a transmission line or electrical facility can conduct over a specified time period before it sustains permanent damage by overheating or before it sags to the point that it violates public safety requirements. |
| 008-0.37 | Tie Facility(ies) |  | The transmission facility(ies) interconnecting Balancing Authority Areas. |  |
|  | Tie Line |  |  | A circuit connecting two Balancing Authority Areas. |
|  | Tie Line Bias |  |  | A mode of Automatic Generation Control that allows the Balancing Authority to 1) maintain its Interchange Schedule and 2) respond to Interconnection frequency error. |
| 006-0.8 | Time Error |  | Accumulated time difference between time based on Interconnection frequency and the National Bureau of Standards time. | The difference between the Interconnection time measured at the Balancing Authority(ies) and the time specified by the National Institute of Standards and Technology. Time error is caused by the accumulation of Frequency Error over a given period. |
| 006-0.9 | Time Error Correction |  | An offset to the Interconnection’s scheduled frequency to correct for accumulated Time Error. | An offset to the Interconnection’s scheduled frequency to return the Interconnection’s Time Error to a predetermined value. |
|  | TLR Log |  |  | Report required to be filed after every TLR Level 2 or higher in a specified format. The NERC IDC prepares the report for review by the issuing Reliability Coordinator. After approval by the issuing Reliability Coordinator, the report is electronically field in a public area of the NERC Web site. |
|  | Total Transfer Capability | TTC |  | The amount of electric power that can be moved or transferred reliably from one area to another area of the interconnected transmission systems by way of all transmission lines (or paths) between those areas under specified system conditions. |
|  | Transaction |  |  | See Interchange Transaction. |
| 001-0.20 | Transfer |  | Request to convey all rights and obligations associated with a reservation for Point-To-Point Transmission Service from a Reseller to an Assignee. |  |
|  | Transfer Capability |  |  | The measure of the ability of interconnected electric systems to move or transfer power in a reliable manner from one area to another over all transmission lines (or paths) between those areas under specified system conditions. The units of transfer capability are in terms of electric power, generally expressed din megawatts (MW). The transfer capability from “Area A’ to “Area B” is not generally equal to the transfer capability form “Area B’ to “Area A.” |
| 008-0.38 | Transfer Distribution Factor | TDF | The portion of an Interchange Transaction, expressed in percent that flows across a transmission facility (Flowgate). | See Distribution Factor. [Distribution Factor - The portion of an Interchange Transaction, typically expressed in per unit that flows across a transmission facility (Flowgate).] |
|  | Transmission |  |  | An interconnected group of lines and associated equipment for the movement or transfer of electric energy between points of supply and points at which it is transformed for delivery to customers or is delivered to other electric systems. |
|  | Transmission Constraint |  |  | A limitation on one or more transmission elements that may be reached during normal or contingency system operations. |
| 001-0.12 008-0.39 | Transmission Customer |  | Any eligible customer (or its designated agent) that can or does execute a transmission service agreement or can or does receive transmission service. | - Any eligible customer (or its designated agent) that can or does execute a transmission service agreement or can or does receive transmission service. - Any of the following responsible entities: Generator Owner, Load-Serving Entity, or Purchasing-Selling Entity. |
| 002-0.4 | Transmission Customer or Customer | TC | Is any eligible Customer (or its designated agent) that can or does execute a transmission service agreement or can or does receive transmission service. (Consistent with the definition in WEQ-001-0.12). |  |
|  | Transmission Line |  |  | A system of structures, wires, insulators and associated hardware that carry electric energy from one point to another in an electric power system. Lines are operated at relatively high voltages varying from 69 kV up to 765 kV, and are capable of transmitting large quantities of electricity over long distances. |
| 008-0.40 | Transmission Loading Relief | TLR | A procedure used in the Eastern Interconnection to relieve potential or actual loading on a Constrained Facility or Flowgate. |  |
| 008-0.41 | Transmission Operator |  | The entity that operates or directs the operations of transmission facilities. | The entity responsible for the reliability of its “local” transmission system, and that operates or directs the operations of the transmission facilities. |
|  | Transmission Owner |  |  | The entity that owns and maintains transmission facilities. |
|  | Transmission Planner |  |  | The entity that develops a long-term (generally one year and beyond) plan for the reliability (adequacy) of the interconnected bulk electric transmission systems within its portion of the Planning Authority Area. |
| 001-0.11 | Transmission Provider |  | Any public utility that owns, operates, or controls facilities used for the transmission of electric energy in interstate commerce. |  |
| 002-0.5 | Transmission Provider or Primary Provider | TP | Is the public utility (or its designated agent) that owns, operates or controls facilities used for the transmission of electric energy in interstate commerce. (Consistent with the definition in WEQ-001-0.11). |  |
|  | Transmission Reliability Margin | TRM |  | The amount of transmission transfer capability necessary to provide reasonable assurance that the interconnected transmission network will be secure. TRM accounts for the inherent uncertainty in system conditions and the need for operating flexibility to ensure reliable system operation as system conditions change. |
| 008-0.42 | Transmission Service |  | Services needed to move energy from a receipt point to a delivery point provided to Transmission Customers by Transmission Service Providers. | Services provided to the Transmission Customer by the Transmission Service Provider to move energy form a Point of Receipt to a Point of Delivery. |
| 007-0.10 | Transmission Service Provider | TSP | The entity that administers the transmission tariff and provides transmission services to qualified market participants under applicable transmission service agreements. | The entity that administers the transmission tariff and provides Transmission Service to Transmission Customers under applicable transmission service agreements. |
| 008-0.43 | Transmission Service Provider or Transmission Provider | TSP or TP | The entity that administers the transmission tariff and provides transmission services to qualified Transmission Customers under applicable transmission service agreements. |  |
| 002-0.6 | Transmission Services Information | TS Information | Is transmission and ancillary services information that must be made available by public utilities on a non-discriminatory basis to meet the regulatory requirements of transmission open access. |  |
| 002-0.7 | Transmission Services Information Provider | TSIP | Is a Transmission Provider or an agent to whom the Transmission Provider has delegated the responsibility of meeting any of the requirements associated with OASIS. (Consistent with the definition of Responsible Party in WEQ-001-0.9). |  |
| 002-0.8 | Value-Added Transmission Services Information Provider | VTSIP | Is an entity who uses TS Information in the same manner as a Customer and provides value-added information services to its Customers. |  |
|  | Vegetation |  |  | All plant material, growing or not, living or dead. |
|  | Vegetation Inspection |  |  | The systematic examination of a transmission corridor to document vegetation conditions. |
| 006-0.10 | WECCNet |  | A messaging system used by the Western Electric Coordinating Council (WECC) for use by participating utility’s dispatchers and network administrators. |  |
| 001-0.13 | Wholesale Merchant Function |  | The sale for resale of electric energy in interstate commerce. |  |
|  | Wide Area |  |  | The entire Reliability Coordinator Area as well as the critical flow and status information from adjacent Reliability Coordinator Areas as determined by detailed system studies to allow the calculation of Interconnected Reliability Operating Limits. |

NAESB WEQ Version 001 Definition of Terms, Published October 31, 2007.

Glossary of Terms Used in Reliability Standards, Adopted by NERC Board of Trustees: February 12, 2008.

Phasor Measurement Units and any other equipment that meets the functional requirements of DMEs may qualify as DMEs.