Standard IRO-006-EAST-1 — TLR Procedure for the Eastern Interconnection

Standard Development Roadmap
This section is maintained by the drafting team during the development of the standard and will be removed when the standard becomes effective.

Development Steps Completed:

1. SC authorized the SAR and assembled a drafting team on December 5, 2006.

2. The revisions to IRO-006 to transfer business practice content to NAESB were approved as IRO-006-4 by the Board of Trustees on October 23, 2007.

3. The SDT has developed a first draft for industry consideration and posted it for comments from October 30, 2008 to December 1, 2008.

4. The SDT has developed this second draft for industry consideration.

Description of Current Draft:

This is the second draft of the proposed standard posted for stakeholder comments.  
Future Development Plan:

	Anticipated Actions
	Anticipated Date

	1. Respond to Comments (Draft 1).
	February 26, 2009

	2. Posting for Comment (Draft 2).
	February 26, 2009

	3. Respond to Comments (Draft 2).
	June 26, 2009

	4. Posting for 30-day Pre-Ballot Review.
	June 26, 2009

	5. Initial Ballot.
	July 27, 2009

	6. Respond to comments.
	September 10, 2009

	7. Recirculation ballot.
	September 10, 2009

	8. Board adoption.
	October 2009


Definitions of Terms Used in Standard
This section includes all newly defined or revised terms used in the proposed standard.  Terms already defined in the Reliability Standards Glossary of Terms are not repeated here.  New or revised definitions listed below become approved when the proposed standard is approved.  When the standard becomes effective, these defined terms will be removed from the individual standard and added to the Glossary. 
Reallocation: The total or partial curtailment of Transactions during TLR Level 3a or 5a to allow Transactions using higher priority to be implemented. (To be retired.)
Market Flow: the total amount of energy flowing across a specified facility or set of facilities due to a market dispatch of internal generation to serve internal load. 
A. Introduction

1. Title: Transmission Loading Relief Procedure for the Eastern Interconnection
2. Number: IRO-006-EAST-1
3. Purpose:
To provide an Interconnection-wide transmission loading relief procedure (TLR) for the Eastern Interconnection that can be used to prevent and/or mitigate potential or actual System Operating Limit (SOL) and Interconnection Reliability Operating Limit (IROL) exceedances to maintain reliability of the Bulk Electric System (BES).
4. Applicability:

4.1. Initiating Reliability Coordinators in the Eastern Interconnection. 

4.2. Responding Reliability Coordinators

5. Proposed Effective Date:
First day of the first calendar quarter following the date this standard is approved by applicable regulatory authorities, or in those jurisdictions where regulatory approval is not required, the standard becomes effective on the first day of the first calendar quarter after the date this standard is approved by the NERC Board of Trustees.
B. Requirements

R1. When acting or directing others to act to mitigate the magnitude and duration of the instance of exceeding an IROL within that IROL’s TV, each Reliability Coordinator shall initiate, prior to or concurrently with the initiation of the Eastern Interconnection TLR procedure (or continuing management of this procedure if already initiated), one or more of the following actions: [Violation Risk Factor: High] [ Time Horizon: Real-time Operations] 
· Inter-area redispatch
· Intra-area redispatch of generation
· 

Reconfiguration of the transmission system
· Voluntary load reductions (e.g., Demand-side Management) 
· Involuntary load reductions
R2. When initiating the Eastern Interconnection TLR procedure to prevent or mitigate a SOL or IROL exceedance, and at least every clock hour after initiation up to and including the hour when the TLR level has been identified as TLR Level 0, the Reliability Coordinator shall identify: [Violation Risk Factor: Medium] [ Time Horizon: Real-time Operations]
R2.1. The TLR level (TLR levels are listed in Appendix A) and
R2.2. A list of flow reduction actions to be implemented based on the TLR level chosen.
R3. Upon the identification of the TLR level and a list of flow reduction actions to be implemented based on the TLR level chosen, the Reliability Coordinator initiating this TLR procedure shall: [Violation Risk Factor: Medium] [ Time Horizon: Real-time Operations]
R3.1. Notify all Reliability Coordinators in the Eastern Interconnection of the identified TLR level
R3.2. Communicate the list of flow reduction actions to be implemented to:

R3.2.1. All Reliability Coordinators in the Eastern Interconnection, and 
R3.2.2. Those Reliability Coordinators in other Interconnections responsible for curtailing Interchange Transactions crossing Interconnection boundaries identified in the list of flow reduction actions.   
R3.3. Request that the following entities implement the flow reduction actions identified in R2.2: 

R3.3.1. Each Reliability Coordinator associated with a Sink Balancing Authority for which Interchange Transactions are to be curtailed 
R3.3.2. Each Reliability Coordinator associated with a Balancing Authority in the Eastern Interconnection for which Network Integration Transmission Service or Native Load is to be curtailed 
R3.3.3. Each Reliability Coordinator associated with a Balancing Authority in the Eastern Interconnection for which its Market Flow is to be curtailed 
R3.3.4. 
R4. Each Reliability Coordinator that receives a request as described in R3.3. shall comply with the request by taking one or more of the following three sets of actions: [Violation Risk Factor: High] [ Time Horizon: Real-time Operations]
1) Implement the communicated flow reduction actions requested by the issuing Reliability Coordinator as follows:

· Direct its Balancing Authorities to implement the Interchange Transaction schedule change requests. 

· Direct its Balancing Authorities to provide the Network Integrated Transmission Service and Native Load schedule changes for which the Balancing Authorities are responsible. 

· Direct its Balancing Authorities to provide the Market Flow schedule changes for which the Balancing Authorities are responsible.
2) Implement a procedure pre-approved by the ERO for use by the responding Reliability Coordinator in lieu of implementing some or all of the requested flow reduction actions in the first set under Requirement 4, provided that its implementation is expected to achieve the same or greater effect than the flow reduction actions requested in the first set under Requirement R4.


3) Implement alternate flow reduction actions to those in the first two sets of flow reduction actions under Requirement R4 provided that:
· Analysis shows that some or all of the flow reduction actions in the first two sets of flow reduction actions under Requirement R4 will result in a reliability concern or will be ineffective, and
· The alternate flow reduction actions have been agreed to by the initiating Reliability Coordinator, and

· Analysis shows that the alternate flow reduction actions will not adversely affect reliability.  
R5. Each Reliability Coordinator that responds to a TLR event shall acknowledge to the initiating Reliability Coordinator the flow reduction actions it will take pursuant to Requirement R4 as soon as possible but not more than ten minutes of receiving the request.  [Violation Risk Factor: Medium] [ Time Horizon: Real-time Operations]
C. Measures 
M1. 
Each Reliability Coordinator shall provide evidence (such as logs, voice recordings, or other information) that when acting or directing others to act to mitigate the magnitude and duration of the instance of exceeding an IROL within that IROL’s Tv, the Coordinator  initiated one or more of the actions listed in R1 prior to or concurrently with the initiation of the Eastern Interconnection TLR procedure (or continuing management of this procedure if already initiated)(R1).    
M2.
Each Reliability Coordinator shall provide evidence (such as logs, voice recordings, or other information) that at the time it initiated the Eastern Interconnection TLR procedure, and at least every clock hour after initiation up to and including the hour when the TLR level was identified as TLR Level 0, the Reliability Coordinator identified both the TLR Level and a list of flow reduction actions to be implemented based on the TLR level chosen (R2).
M3. 
Each Reliability Coordinator shall provide evidence (such as logs, voice recordings, or other information) that after it identified a TLR level and a list of flow reduction actions to take, it 1.) notified all Reliability Coordinators in the Eastern Interconnection of the TLR Level, 2.) communicated the list of actions to all Reliability Coordinators in the Eastern Interconnection and those Reliability Coordinators in other Interconnections responsible for curtailing Interchange Transactions crossing Interconnection boundaries identified in the list of flow reduction actions, and 3.) requested the Reliability Coordinators identified in R3.2 to implement the flow reduction actions identified in R2.2 (R3).

M4.
Each Reliability Coordinator shall provide evidence (such as logs, voice recordings, or other information) that upon receipt of a request as described in R3, the Reliability Coordinator complied with the request by taking one or more of the following: 1.) implemented the communicated flow reduction actions requested by the issuing Reliability Coordinator, 2.) implemented a procedure pre-approved by the ERO for use by the responding Reliability Coordinator that would achieve the same or greater effect than some or all of the flow reduction actions requested, 3.) implemented alternate flow reduction actions based on analysis which showed that some or all of the flow reduction actions in 1 or 2 would have resulted in a reliability concern or would have been ineffective, the alternate flow reduction actions were agreed to by the initiating Reliability Coordinator, and analysis showed that the alternate flow reduction actions would not adversely affect reliability (R4).
M5.
Each Reliability Coordinator shall provide evidence (such as logs, voice recordings, or other information) that within ten minutes of receiving a request to implement flow reduction actions pursuant to the implementation of the Eastern Interconnection TLR procedure, it acknowledged to the initiating Reliability Coordinator the flow reduction actions it will take in response to their request. 

D. Compliance

1. Compliance Monitoring Process

1.1. Compliance Enforcement Authority

Regional Entity.

1.2. Compliance Monitoring Period and Reset Time Frame

Not applicable.
1.3. Data Retention

The Reliability Coordinator shall keep data or evidence to show compliance as identified below unless directed by its Compliance Enforcement Authority to retain specific evidence for a longer period of time as part of an investigation:
· The Reliability Coordinator shall maintain evidence to show compliance with R1, R2, R3, R4, and R5 for the past 12 months plus the current month.  

· If a Reliability Coordinator is found non-compliant, it shall keep information related to the non-compliance until found compliant. 

The Compliance Enforcement Authority shall keep the last audit records and all requested and submitted subsequent audit records.  

1.4. Compliance Monitoring and Enforcement Processes:

1. The following processes may be used:

· Compliance Audits

· Self-Certifications

· Spot Checking

· Compliance Violation Investigations

· Self-Reporting

· Complaints

1.5. Additional Compliance Information

None.
3. Violation Severity Levels 

	R #
	Lower VSL
	Moderate VSL
	High VSL
	Severe VSL

	R1
	
	
	
	

When acting or directing others to act to mitigate the magnitude and duration of the instance of exceeding an IROL within that IROL’s Tv, the Reliability Coordinator did not initiate one or more of the actions listed under R1 prior to or in conjunction with the initiation of the Eastern Interconnection TLR procedure (or continuing management of this procedure if already initiated).

	R2
	The Reliability Coordinator initiating the Eastern interconnection TLR procedure missed identifying the TLR Level and/or a list of flow reduction actions to take based on the TLR level chosen for one clock hour during the period from initiation up to the hour when the TLR level was identified as TLR Level 0, 
	The Reliability Coordinator initiating the Eastern interconnection TLR procedure missed identifying the TLR Level and/or a list of flow reduction actions to take based on the TLR level chosen for two clock hours during the period from initiation up to the hour when the TLR level was identified as TLR Level 0,
	The Reliability Coordinator initiating the Eastern interconnection TLR procedure missed identifying the TLR Level and/or a list of flow reduction actions to take based on the TLR level chosen for three clock hours during the period from initiation up to the hour when the TLR level was identified as TLR Level 0,
	The Reliability Coordinator initiating the Eastern interconnection TLR procedure missed identifying the TLR Level and/or a list of flow reduction actions to take based on the TLR level chosen for four or more clock hours during the period from initiation up to the hour when the TLR level was identified as TLR Level 0,

	R3
	The initiating Reliability Coordinator did not notify one or more Reliability Coordinators in the Eastern Interconnection of the TLR Level (R3.1)
	Not applicable

	The initiating Reliability Coordinator did not communicate the list of flow reduction actions to one or more of the Reliability Coordinators listed in R3.2.1 and R3.2.2
OR

The initiating Reliability Coordinator requested some, but not all, of the Reliability Coordinators identified in R3.3 to implement the identified flow reduction actions.


	The initiating Reliability Coordinator requested none of the Reliability Coordinators identified in R3.3 to implement the identified flow reduction actions.

	R4
	
	
	
	The responding Reliability Coordinator did not take one or more of the following actions:
1.) Implemented the requested flow reduction actions.

2.) Implemented a procedure pre-approved by the ERO for use by the responding Reliability Coordinator that would achieve the same or greater effect than the requested flow reduction actions requested.  

3.) Implemented alternate flow reduction actions based on analysis which showed that some or all of the actions in 1 or 2 would have resulted in a reliability concern or would have been ineffective, and that the alternate flow reduction actions were agreed to by the initiating Reliability Coordinator and analysis showed that the alternate flow reduction actions would not adversely affect reliability.

	R5
	The responding Reliability Coordinator communicated its intended flow reduction actions to the initiating Reliability Coordinator, but did so more than ten minutes but not more than fifteen minutes after receiving the request. 
	The responding Reliability Coordinator communicated its intended flow reduction actions to the initiating Reliability Coordinator, but did so more than fifteen minutes but not more than twenty minutes after receiving the request.
	The responding Reliability Coordinator communicated its intended flow reduction actions to the initiating Reliability Coordinator, but did so more than twenty minutes but not more than twenty five minutes after receiving the request. 
	The responding Reliability Coordinator communicated its intended flow reduction actions to the initiating Reliability Coordinator, but did so more than twenty five minutes after receiving the request.

OR

The responding Reliability Coordinator did not communicate its intended flow reduction actions to the initiating Reliability Coordinator.




E. Variances
None.
F. Associated Documents

G. Revision History

	Version 
	Date 
	Action 
	Tracking 

	1 
	
	Creation of new standard, incorporating concepts from IRO-006-4 Attachment; elimination of Regional Differences, as the standard allows the use of Market Flow
	New 


Appendix A

The following guidelines are intended to assist the Reliability Coordinator in determining what level of TLR to call.  However, the Reliability Coordinator has the discretion to choose any of these levels regardless of the guidelines listed below, provided the Reliability Coordinator has reliability reasons to take such action.  

	Level
	Guidelines for System Conditions

	TLR-1
	· At least one Transmission Facility is expected to approach or exceed its SOL or IROL within 8 hours.

	TLR-2
	· At least one Transmission Facility is approaching or is at its SOL or IROL. 

· Analysis shows that holding new and increasing non-firm Interchange Transactions and energy flows for the next hour can prevent exceeding this SOL or IROL.

	TLR-3a
	· At least one Transmission Facility is expected to exceed its SOL or IROL within the next hour.

· Analysis shows that full or partial curtailment or reallocation
 of non-firm Interchange Transactions and energy flows can prevent exceeding this SOL and IROL.

	TLR-3b
	· At least one Transmission Facility is exceeding its SOL or IROL, or

· At least one Transmission Facility is expected to exceed its SOL or IROL within the current hour.

· Analysis shows that full or partial curtailment or reallocation
 of non-firm Interchange Transactions and energy flows can prevent exceeding this SOL or IROLs.

	TLR-4
	· At least one Transmission Facility is expected to exceed its SOL or IROL.

· Analysis shows that full curtailment of non-firm Interchange Transactions and energy flows, or reconfiguration of the transmission system can prevent exceeding this SOL or IROL.

	TLR-5a
	· At least one Transmission Facility is expected to exceed its SOL or IROL within the next hour.

· Analysis shows that the following actions can prevent exceeding the SOL or IROL: 

· Full curtailment non-firm Interchange Transactions and energy flows, and
· Reconfiguration of the transmission system, if possible, and
· Full or partial curtailment or reallocation
 of firm Interchange Transactions and energy flows.

	TLR-5b
	· At least one Transmission Facility is exceeding its SOL or IROL, or

· At least one Transmission Facility is expected to exceed its SOL or IROL within the current hour.

· Analysis shows that the following actions can prevent exceeding the SOL or IROL:

· Full curtailment of non-firm Interchange Transactions and energy flows, and
· Reconfiguration of the transmission system, if possible, and

· Full or partial curtailment or reallocation
 of firm Interchange Transactions and energy flows.


	TLR-6
	· At least one Transmission Facility is exceeding its SOL or IROL, or
· At least one Transmission Facility is expected to exceed its SOL or IROL upon the removal from service of a generating unit or another transmission facility.


	TLR-0
	· No transmission facilities are expected to approach or exceed their SOL or IROL within 8 hours, and the ICM procedure may be terminated




















� “Reallocation” is a term defined within the NAESB TLR standards.  


� “Reallocation” is a term defined within the NAESB TLR standards.


� “Reallocation” is a term defined within the NAESB TLR standards.


�  “Reallocation” is a term defined within the NAESB TLR standards.





�Need to think about this


�Suggest to combine the two bullets and just call it "redispatch" since it also includes internal redispatch.


�We used these terms in our past filings; suggest we leave all 5 in order to be consistent.  


�We think new RC procedures will address – e.g., passive action by RC’s like WECC, ERCOT


�We think we can delete and handle as variance – pending discussion w/ Cook and Ben to research # of approved local procedures


�I checked with Gerry and he concurs, but I have not heard back from Cook yet.





Draft 2: February 26, 2009

Page 12 of 13 

