Facility Ratings [ FAC-008-2]
Standard Purpose: To ensure that Facility Ratings used in the reliable planning and operation of the Bulk Electric System (BES) are determined based on technically sound principles. A Facility Rating is essential for the determination of System Operating Limits.

Applicability

Transmission Owner.

Generator Owner.
Summary of Changes
1. FAC-008-1 (Facility Ratings Methodology) and FAC-009-1 (Establish and Communicate Facility Ratings) were combined into one standard: FAC-008-2 (Facility Ratings)

2. Developed requirements to document in the Facility Ratings methodology the underlying assumptions and methods used to determine normal and emergency facility ratings; 
3. Developed requirements to develop Facility Ratings consistent with industry standards approved through an open, transparent and validated process such as IEEE or CIGRE.
Reasons for Changes

1. FERC Order 693 Directives

2. FERC Staff Report Comments: Lack of consistent facility ratings methodology (Blackout item #27)

Deinition of Facility Ratings in the NERC Glossary: The maximum or minimum voltage, current, frequency, or real or reactive power flow through a facility that does not violate the applicable equipment rating of any equipment comprising the facility.

NAESB WEQ SRS Review

Comments [Sam Ciccone - FirstEnergy Corp.]: Based on a review of the changes to NERC standards FAC-008/-009, the current NAESB business practices are not affected and there does not appear to be a need to create a business practice as a result of these NERC changes. The NAESB practices do not currently discuss nor define Facility Ratings. The only indirect reference to Facility Ratings in the business practices are the definition for System Operating Limits (SOL) where a Facility Rating is an essential element in determing the SOL. Another indirect reference is in Information to be Posted on the OASIS where the limiting element and cause of the limit are factors in calculating ATC and TTC. Also, the OASIS Data Dictionary references “FACILITY_CLASS” and “FACILITY_LIMIT_TYPE”.
References

008-0.36 System Operating Limit (SOL) - The value (such as MW, MVar, Amperes, Frequency or Volts) that satisfies the most limiting of the prescribed operating criteria for a specified system configuration to ensure operation within acceptable reliability criteria. System Operating Limits are based upon certain operating criteria.

001-1.6 INFORMATION TO BE POSTED ON THE OASIS
(b) Posting transmission capability. The transmission capability that is expected to be available on the Transmission Provider’s system (ATC) and the total transmission capability (TTC) of that system shall be calculated and posted for each Posted Path as set out in this section.

(ii) On request, the Responsible Party must make all data used to calculate ATC and TTC for any constrained posted paths publicly available (including the limiting element(s) and the cause of the limit (e.g., thermal, voltage, stability) in electronic form within one week of the posting. The information is required to be provided only in the electronic format in which it was created, along with any necessary decoding instructions, at a cost limited to the cost of reproducing the material. This information is to be retained for six months after the applicable posting period.

[image: image1.png]003-0 OASIS DATA DICTIONARY, Version 1.4

Data Dictionary Alias, Field Format : Restricted Values Definition of Data Element
Element Name minimum characters
{type of ASCIl}
maximum characters
DEALS DREF O[ALPHANUMERIC12 | Unique value, Assigned by | The UNique reference assigned by a CUSIomer (0 wo or
Customer more senvice purchases 1o identfy each of tem as
related to others in the same power service deal. These
requests may be related to each other in time sequence
through 2 single Provider, or as a series of wheels
through multiole Providers, or a combination of both
time and wheels. The User uses the DEAL_REF to
uniquely identify a combination of requests relating to a
particular deal
DISCRETION DESCRIPTION | DISCDESC OALPHANUMERICIO00 | Free form text A detailed description of fhe discretion being reported

ELEMENT_NAW

ENENT

T{ALPRANUNERICI0

Valid Template element
name

Template element name as Indicated in Gata dictonary.

PNANE

TALPRANUVERICI25

Free form et

Narme of person wha 1s transferting from one position 1o
another.

ERROR_MESSAGE

ERROR

T{ALPRANUVERICI250

Free form et

Ermor message related ©© 3 RECORD_STATUS of
REQUEST_STATUS,

EVENT_D

OALPHANUMERICIZS

Free form et

The EVENTID Daia Element s any regional or
interconnection-wide recognized security event identier
for_events that are of greater scope than those
administered locally by the Provider (g, a NERC
Security Coordinator assigned identifier corresponding
t a particular implementation of the NERC TLR
procedure

FACILITY_CLASS

FACLASS

OALPHANUMERICIZS

Free form tex, for example:
TRANSFORMER,

LN
FLOWGATE

Or s defined in the LIST
Template

Type of Imifing device such as ransiormer Tne or
flowgate’

FACILITY_LIMIT_TYPE

FACLIMTYP

OALPHANUMERICIZS

Thermal, Sbilty, voliage o
defined in LIST Templat

For example: thermal, sty voags,





